Code No:420805

Set No.

1

IV-B.Tech. II Semester Examinations April, 2003

OPTIMIZATION OF CHEMICAL PROCESSES

(Chemical Engineering)

Time: 3 Hours							  Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Discuss the problem of optimization of economic insulation thickness.

b)	Assume that the bare surface of a vessel is at 700 0F with an ambient temperature of 700F. The surface heat loss is 4000 Btu/(h)(ft2).  Add one inch of calcium silicate insulation and the loss will drop to 250 Btu/(h)(ft2).  At  an  installed  cost of  $ 4/ ft2 and a cost of energy at $ 5/106 Btu, a saving of $ 164 per year (8760 hours of operation) per square foot would be realized.  Calculate the pay back period and mention the approach to determine the optimum insulation thickness.

2.	Describe two commonly used models of reactors along with their governing equations.

3.	Classify the following functions as unimodel or multimodel or neutral:

a)      f(x) = 2x +3

b)      f(x) = x2

c)      f(x) = a cos(x).

4.	Solve the following linear programming

maximize  	f = x1 + x2

x1 + 3x2  ¡Ü  12

x1 ¨C x2  ¡Ü   1

2x1 ¨C x2  ¡Ü 4

2x1 + x2  ¡Ü  8

xi ¡Ý  0             (i = 1,2)

Does the solution via simplex method exhibit cycling?

5.	Obtain an expression for the optimum temperature of the working fluid to be used in a power system for recovery of waste heat from a boiler. Draw a neat schematic diagram of the power system.

6.	For the steady state continuous counter current liquid extraction in a column it was found that the plug flow model was sufficient accurately to represent the data collected. With the help of a schematic diagram, develop the model equations necessary to describe the process and mention the constraints. If the objective is to maximize the total extraction rate, write the objective function. Assume that (i) an analytical solution exists (ii) concentrations are expressed on a solute-free mole basis (iii) the equilibrium relation is a straight line       Y* = mX + B and that the operating line is straight.

Contd…..2.

Code No.420805				:: 2 ::				Set No.1

7.	The following reaction takes place in a well mixed batch reactor:

k3	      C

k1

A                            B

k4	        D

k2

The initial concentrations are:

cAo = 50.0 mol/lit, cBo = 5.0 mol/lit,  cCo = 0.0 mol/lit and cDo = 0.0 mol/lit.

The reaction rates are:

k1 = 2.0 h-1,  k2 = 1.0 h-1,  k3 = 0.20 h-1 and  k4 = 0.60 h-1.

Calculate the optimal residence time to yield a maximum concentration of B.

e.=γ k2 c  −  D ec.=k1 s (e−c)−(k2+k3) c−D cs.= - k1s (e-c)+(k3 c+D ( so−s)8.	The dynamic model of a continuous  flow biological chemostat is given by

where e, c and s are biomass, metabolic and substrate concentrations in gmol/lit, respectively. The values of rate constants k1, k2  and k3 are 0.9 lit/mol.h, 0.7 h-1 and 0.0, respectively. The limiting feed substrate concentration so =10.0 mol/lit and the parameter  = 0.09. The dilution rate D is an independent variable. The objective is to maximize the steady state production of biomass given by  f = De. Obtain the steady solution of the system for   0   D    0.6 and plot the responses of e, c, s and f as a function of D. Find the value of D that provides maximum of   f.

@@@@@

Code No:420805

Set No.

2

IV-B.Tech. II Semester Examinations April, 2003

OPTIMIZATION OF CHEMICAL PROCESSES

(Chemical Engineering)

Time: 3 Hours							  Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1.	The production in two plants A and B, each of which can manufacture two products – No.1 and No. 2 is to be scheduled.  How should the scheduling take place to maximize the profits while meeting the market requirements based on the following data :

| Plant | Material 1 Processed, lb/day | Material 2 Processed, lb/day | Profit for Material 1, $/lb | Profit for Material 2, $/lb |
| --- | --- | --- | --- | --- |
| A | MA1 | MA2 | SA1 | SA2 |
| B | MB1 | MB2 | SA2 | SB2 |

How many days per year(365 days) should each plant operate processing each 	kind of material ?.

[Hints : Does the table contain the variables to be optimized? How do you use the above information to mathematically formulate the optimization problem? What other factors must be considered?]

2.	Describe and discuss the relative merits and limitations of the common models for electro-static precipitator.

3.	Determine the convexity or concavity of the following objective functions:

a)    f(x1,x2) = (x1-x2)2 +  x22

b)      f (x1,x2,x3)= x12 + x22 + x32

c)      f (x1,x2) = ex1 + ex2.

4.	A confectioner manufacturer two kinds of candy bars Ergies and Nergies. 	Ergies sell at a profit of 50 paise per box while Nergies have a profit of 60 	paise per box. The candy is processed in three main operations blending and 	cooking and packaging. The following table records the average time in 	minutes required by each box of candy for each of the three activities.					Blending	cooking 	packaging

Ergies		     1		     5		     3

Nergies	     2		     4		     1

During each production run, the blending equipment is available for a 	maximum of 14 machines hours, the cooking equipment for atmost 40 	machines hours and the packaging equipment for 15 machine hours. If each 	machine can be allocated to the making of either type of candy at all times that 	is available for production, determine how many boxes of each kind of candy 	the confectioner should make in order to realize the maximum profit. Use a 	graphical technique.

Contd…..2.

Code No.420805				:: 2 ::				Set No.2

5.	Develop the equation for determining the optimum number of stages and the optimal performance ratio in multiple effect evaporation.

# 6.a)	Discuss about the classification of optimization problems for steady state distillation.

b)	For optimal design and operation of conventional staged distillation columns formulate the equality constraints with the help of a schematic diagram.

7.         The following first order series reaction

k1                                        k2

A                            R                                S

with k1 = k2  = 0.1 min-1 and cAo = 0.5 mol/lit  occurs in a batch reactor. Find  	the optimal time and the maximum concentration of R at that time.

8.	The steady state dependence of chemostat variables, namely, cell mass concentration x and substrate concentration  s are expressed by the following equations:

x=Yx/sSf−DKsμmax−Ds=DKsμmax−D

where D is the dilution rate. The parameter values are: maximum specific growth rate max=1.0 h-1,  yield factor Yx/s = 0.5, substrate growth rate constant Ks=0.2 g/lit and substrate feed concentration sf = 10.0 g/lit. The objective to maximize the rate of  cell production is Dx. Obtain a steady state solution of x, s and Dx as a function of  D such that  0   D  1.0 and plot the corresponding responses.

@@@@@

Code No:420805

Set No.

3

IV-B.Tech. II Semester Examinations April, 2003

OPTIMIZATION OF CHEMICAL PROCESSES

(Chemical Engineering)

Time: 3 Hours							  Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1.	In a process, shown schematically in the following figure, suppose the flow rates entering and leaving the process are measured periodically.  Determine the best value for stream A in pounds for the process shown from three hourly measurements indicated of B and C in the figure, assuming steady state at a fixed operating point.  The process model is MA + MC  =  MB where M is the mass per unit time of throughput.

__________________                  (a) 92.4 kg/h

l                                  1------B  (b) 94.3 kg/h

l                                  l                  (c) 03.8 kg/h

A   l                                  l

l                                  l------C-  (a) 11.1 kg/h

l_________________l                   (b) 10.8 kg/h

(c) 11.4 kg/h

2.	Discuss the important steps involved in building a model for practical applications.

3.	Are the following functions convex? Strictly convex? Why?

a)      2x12 + 2x1x2 + 3x22 + 7x1+ 8x2 + 25

What are the optimum values of x1 and x2.

b)      exp(5x).

4.	Minimize 	f = 3x1 + 5x2

subject to 	x1 + 3x2 ¡Ý    10

2x1 ¨C x2   ¡Ý  4

-x1 + 4x2  ¡Ý  -2

-x1 ¨C x2   ¡Ý  -20

x1, x2  ¡Ý   0

a) Use artificial variables and phase I by phase II procedure to obtain x*.

b) Formulate the dual problem and solve it. Compare answers with (a).

c) Identify where the shadow prices appear in parts (a) and (b).

5.a)	Write the pertinent equations required for optimizing the process design of a baffled shell-and-tube single-pass counterflow heat exchanger in which the tube fluid is in turbulent flow but there is no change of phase of fluids in the shell or tubes.

b)	Give the algorithm for calculating the optimum values of the design variables of the heat exchanger mentioned in part (a).

Contd…..2.

Code No.420805				:: 2 ::				Set No.3

6.	Assuming that the plug flow model accurately represents the experimental data obtained in a steady state continuous counter current liquid extraction column, develop the model equations necessary to describe the process. Mention the constraints and write the objective function to maximize total extraction rate. Assume that (i) an analytical solution exists (ii) concentrations are expressed on a solute-free mole basis (iii) the equilibrium relation is a straight line and that the operating line is straight.

7.	A unimolecular first reaction occurs in series in a batch reactor

k1                                        k2

A                            R                                S

Find  the time at which the maximum concentration of R occurs and determine  	the expression for maximum concentration of R.

8.	Apply linear programming to maximize  a thermal cracker objective function represented by f = 2.84 x1 - 0.22 x2  - 3.33 x3 + 1.09 x4 + 9.39 x5 + 9.51 x6       where x1 = fresh ethane feed, x2 = fresh propane feed, x3 = gas oil feed,          x4 = DNG feed,  x5   = ethane recycle and x6  = propane recycle.

The objective is subjected to the following constraints:

1.1 x1 + 0.9 x2  + 0.9 x3 + 1.0 x4 + 1.1 x5 + 0.9 x6     200,000

0.5 x1 + 0.35 x2  + 0.25 x3 + 0.25 x4 + 0.5 x5 + 0.35 x6     100,000

0.01 x1 + 0.15 x2  + 0.15 x3 + 0.18 x4 + 0.01 x5 + 0.15 x6     20,000

0.4 x1 + 0.06 x2  + 0.04 x3 + 0.05 x4 - 0.6 x5 + 0.06 x6   =  0

0.1 x2  + 0.041x3 + 0.01 x4 - 0.9 x6   =  0

xi   0.

@@@@@

Code No:420805

Set No.

4

IV-B.Tech. II Semester Examinations April, 2003

OPTIMIZATION OF CHEMICAL PROCESSES

(Chemical Engineering)

Time: 3 Hours							  Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1.	A poster is to contain 300 cm2 of printed matter with margins of 6 cm at the top and bottom and 4 cm at each side.  Find the overall dimensions that minimize the total area of the poster, after formulating the objective function, indicating equality and inequality constraints(if any) and specifying the independent variables, the number of degrees of   freedom and the coefficients in the optimization problem.

2.	Explain how to determine the form of a model suitable for fitting functions to empirical data.

3.	Does the following sets of constraints form convex  region ?

g1(x) = -(x12 + x22) + 9¡Ý  0

g2(x) = -x1-x2 + 1 ¡Ý  0.

4.	Maximize 	y = x1 + 3 x2 ¨C x3

subject to	x1 + 2x2 + x3 = 4

2x1 + x2  ¡Ü 5

x1, x2, x3  ¡Ý   0

using the simplex method find the optimize optimum values of x1, x2, x3 and 	y solve the problem by 	(1) guessing to obtain an initial basis.

(2) Using a phase I ¨C phase II method.

5.	Draw the schematic diagram of a power system for the recovery of waste heat from a boiler. Ignoring mechanical friction and heat leaks, obtain an expression for the optimum value of the working fluid temperature TH, at which the sum of the operating costs and surface area costs would be minimum.

6.	With the help of a schematic diagram, develop the model equations necessary to describe the process of steady state continuous counter current liquid extraction in a column. The plug flow model was found to accurately represent the experimental data. Assume that (i) an analytical solution exists (ii) concentrations are expressed on a solute-free mole basis (iii) the equilibrium relation is a straight line Y* = mX and that the operating line is straight. Write the constraints and the objective function to maximize the total extraction rate.

Contd…..2.

Code No.420805				:: 2 ::				Set No.4

7.	The following first order reactions occur in series in a batch reactor

k1                                        k2

A                            R                                S

Find  the expressions for maximum concentration of R and the time at which it is reached. Consider the cases of    k1      k2   and   k1  = k2.

8.	The steady state monod  chemostat model for substrate mass balance and cell mass balance is described by the following equations :

Dsf−s−μmaxsxYx/ss+Ks=0

μmaxsKs+s−Dx+Dxf=0

where x and s are cell and substrate concentrations, respectively and D is the dilution rate. The parameter values are: maximum specific growth rate max=1.0 h-1,  yield factor Yx/s = 0.5, substrate growth rate constant Ks=0.2 g/lit, substrate feed concentration sf = 10.0 g/lit and initial cell concentration xf = 0. The objective is to find the optimal value of D that provide maximum cell production Dx. Obtain the steady state solution for x and s as a function of  D,   0   D  1.0 and find the value of D that provide maximum cell production Dx.

@@@@@