Code No.420202

Set No.

1

IV-B.Tech.  II  Semester Examinations April, 2003

ADVANCED CONTROL SYSTEMS

(Electrical and Electronics Engineering)

Time: 3 hours							        Max. Marks:80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Define absorbability and explain its concept using an example.

b)	Consider the system.

x(t) = 1e−t x(t)

y(t) =  	11   x(t)

Is this system observable at t = 0? If  yes, find X(0) when y(t) = et.

2.a) 	What are the methods used to construct the Liapunov function for a dynamical system? Explain at least two methods.

b) 	Check the stability of the system described by

x1=   x2

x2=− x1 -b1x2 - b2x23  ,   b1,b2> 0

3.a)	Consider the system with

A=110320

Convert the system into a single-input controllable system.

b) 	Design a state feedback control law for this system so that the closed loop system has poles at -1, -2, -3.

Contd…2.

Code No.420202			      -2-					Set No.1

4.	Consider the system shown in figure given.  The performance index to be minimized is   J=1/2 0α(y−r)2+u2dt  convert this problem to the form of the output regulator problem and find the optimum value of k.

r(t) = unit step +

X

K

1/s

U(t)

y(t)

5.a)	Derive the Euler Lagrange equation and the boundary conditions for the final time t1 free and x( t1) specified, starting form a given x(t0) extremise.

J(x)=t0t1g(x,x,t)dt

b)	Find the extremal for the following functional:

j(x)=t0t1(2x+12x2)dt

x(1) = 2; x(t1) = 2;    t1 > 1

6.a)	Explain

i)	Principle of casuality

ii)	Principle of invariant imbedding

- Principle of optimality

In dynamic programming problem

b)	What are the merits and demerits of dynamic programming method.

7.	Write the MATLAB commands for drawing root locus for the following system 	with.

G(s) H (s) =   K(s+1)(s+3)(s+4)

8.	Write short notes on the following in MATLAB

(a) String evaluation    (b)  Switch giving suitable examples.

- x -

Code No.420202

IV-B.Tech.  II  Semester Examinations April, 2003

Set No.

2

ADVANCED CONTROL SYSTEMS

(Electrical and Electronics Engineering)

Time: 3 hours							        Max. Marks:80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Name some of the controllable properties of time invariant system to be equivalent.

b)	Show that the pair { A , C } is completely observable for all values of

αi1S.

0	0	0………..0	- n

1	0	0………..0	- n -1

A =   	0	0	0………..0	- n -2

:	:	:	    :	   :

:	:	:	    :	   :

0	0	0………..0	- 2

0	0	0………..0	- 1

C = 	0	0…......0	1

2.a) 	Determine the stability of the origin of the following system:

x1= - x1+ x2 - x1(x12+ x22)

x2= -x1 -x2+x2(x12+ x22)

b) 	Consider the second-order system

X= AX,  where X = x1x2 , 	A=−1−21−4,

Find the real symmetric matrix P, which satisfies stability condition of Liapunov’s

method.

3.a)	Explain the linear system with full-order state observer with neat block diagram.

b)	Design a full-order state observer for the given state model.

GivenA=1−1

and given values are 1 = -5, 2 = -5

Contd..2.

Code No.420202				-2-				Set No. 2

4.	Referring to the block diagram given, consider that G(s) =100/s2 and R(s)=1/s.  Determine the optimal values of  parameters K1 and K2 such that

J=0α(e2(t)+0.25μ2(t))dt  is minimum.

x

G(s)

R(s)	        +	E(s)	U(s)             	         C(s)

5.a)	Find the function for which the functional:

J(x)=0t11+x2xdt has a minimum subject to the boundary conditions.

x(0) = 0 and x(t1) = t1- 5

- Distinguish between functions and functionals.

6.a)	Explain the merits and demerits of dynamically programming problem. What is the dimensionality problem

b)	Obtain the HJB equation in continuous time.

7	Write the MATLAB Programme for finding the error constants for step, rampand parabolic inputs and steady state error of the system whose transfer function is given by

G(s) H (s) =            10(s+4)

(s +1) (s+3) (s+5)

8.	Explain the following model conversion functions with examples

(a)  C2d	       (b) SS2tf	(c)  tf22p		(d) 2p 2 tf.

- x –

Code No.420202

Set No.

3

IV-B.Tech.  II  Semester Examinations April, 2003

ADVANCED CONTROL SYSTEMS

(Electrical and Electronics Engineering)

Time: 3 hours							        Max. Marks:80

Answer any FIVE questions

All questions carry equal marks

- - -

1..	Convert the following state model into the Jordan canonical form and there from comment on controllability and observability.

x(t)	= 010001  x(t)   + 1001   u(t)

y(t)  = 01−1

2..a)	 Explain the Liapunov’s stability analysis of the dynamical systems.

- Determine the stability of the origin of the following system:

x1=  x1 - x2 - x13

x2= x1+x2 - x23

3. a)	Consider a single-input/single-output system

X=01

Y = [ 1    2] X

Check for controllability and observability.

b)	Now introducing a feedback signal u = r + [2  -1] x in (a) comment on controllability and observability.

Contd…2.

Code No. 420202				-2-				Set No.3

4.	For the discrete – time system given Gp(s)=1/S.

G0(s)=1−e−STS, r(t)=unit step T=1 sec.

Find optimal transfer function T*(z) so that output C(t) follows input r(t) 	minimizing

Je=k=0αr(kT)−c(kT)2 with

Ju=k=0αu2(kT)=0.5

5.a)	Formulate the optimal control problem with constraints in terms of Hamiltonian

b)	Find U* for the system  x=−x+U  x(0) = 1   Which minimizes

J=1202(x2+u2)dt

6.	Derive the H J B equation in the continuous time case.

7.	Write a MATLAB programme for getting Bode – plot for the following system with G (s) = 10/s2+s+3 and also obtain it Bode plot.

8.	Write short notes on the following in MATLAB

(a)	Vector and subscripted control flow constraints.

(b)	Control system tool – box.

- x –

Code No.420202

Set No.

4

IV-B.Tech.  II  Semester Examinations April, 2003

ADVANCED CONTROL SYSTEMS

(Electrical and Electronics Engineering)

Time: 3 hours							        Max. Marks:80

Answer any FIVE questions

All questions carry equal marks

- - -

1.	Consider the system

x(t)	=  010001    x(t)   + 0110  u(t)

Find, if possible, a control law to transfer the state from x(1) =  10 to x(4) = 32

2.a) 	Consider the system described by

x1= x2 - x1(x12+ x22)

x2= x1 - x2(x12+ x22)

Determine its equilibrium state stability and comment on the result.

b)	Explain the following

(i) Equilibrium state, (ii) Stability, (iii) Asymptotic stability and (iv) Instability.

3. a)	Explain effect of state feedback on controllability

b)	The state model of a system is given by

X=001−2−30

Y = [1	0  0] X.

Convert the state variable model to controllable phase Variable form.

Contd…2.

Code No.420202				-2-	`			Set No. 4

4.	Referring to the block diagram given, consider that G(s)=100s2 and R(s)=1/S.  Determine the optimal value of parameter K such that

J=0αe2(t)dt is minimum

-

+

-

+

C(s)

U(s)

E(s)

R(s)

xxxxxxxx

x

G(s)

KS

5.a)	Find the Euler Lagrange equations and the boundary conditions for the extremal of the functional

J(x)=0π2(x2+2x1x2+x22)dt

x1(0) = 0    x1(π2) is true

x2(0) = 0     x1(π2) = - 1

- What is a Hamiltonian. Formulate the optional control problem in terms of Hamiltonian.

6.a)	Formulate the continuous time process as a multi state decision process

b)	Formulate dynamic programming problem.  Explain its merits and demerits.

c)	What is the principle causality?

7.	Design a phase lead compensator such that a system with G(s)= 	8

s(s+1) (s+4)

has a phase margin of 450 and a steady state error of 0.25 due to ramp input and write a  MATLAB Programme for the above problem.

8.	How do you use MATLAB techniques in computer aided analysis and design? Explain with examples.

- x -