Code No: 420204

Set No

1

IV-B.Tech. II Semester Examination April, 2003

POWER SYSTEM RELIABILITY

(Electrical and Electronics Engineering)

Time: 3 Hours							Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1.a)	Define the terms “mutually exclusive events” and “statistically independent 	events”.

b)	A factory gets electric power from a generator(G) driven by a diesel engine(E).  If 	the probability of failure of the generator is 0.1 and that of engine is 0.2, what is 	the probability of the system working satisfactorily?

2.a)	Explain the concept of random variables with examples.

b)	In a certain manufacturing process one percent of the products are known to be 	defective.  If 50 items are purchased by a customer, what is the probability of his 	getting two or less number of defectives?  Use the Binomial distribution and 	Poisson distribution and compare the results.

3.a)	Define hazard function and derive the relation between reliability R(t) and hazard 	function h(t).

b)	The reliability network of a system is as shown in the figures.  The figures marked 	indicate the reliabilities of the components.  Calculate the reliability of the system 	by network reduction.

Contd….2

Code No:420204				-2-				Set No:1

4.a)	What is a minimal cut-set?

b)	The reliability network of a system is shown in figure(b).  Estimate the reliability 	of the system if each component has a reliability of 0.9.  Choose B as the critical 	component.

5.	Give the state space diagram of a system consisting of two independent 	components with adequate repair facility.  Also calculate the steady state 	probabilities and state frequencies.

6.a)	Explain loss of load probability and expected loss of energy method for finding 	the generation inadequacy.

b)	What are the short comings of LOLP?

7.a)	What are exact margin states and cumulative margin states.

b)	What are passive and active events?  Give the “three state”  model of a 	component with switching after faults.

8.a)	What are the assumptions made in reliability evaluation of inter connected 	systems.

b)	Give the state space diagram of two systems interconnected with unlimited tie-	line capacity.

- - -

Code No: 420204

Set No

2

IV-B.Tech. II Semester Examination April, 2003

POWER SYSTEM RELIABILITY

(Electrical and Electronics Engineering)

Time: 3 Hours							Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1.a)	Define Reliability, random variable and probability density function.

b)	A switch board receives on an average 60 calls per hour.  If the operator is away 	for ½ minute, what is the probability that there is no call during the absence of the 	operator?

2.a)	Explain the terms: (i) MTTF  (ii) MTBF.  Derive the expression for MTTF in 	exponential distribution.

b)	A component has a reliability of 0.9 for a mission time of 50 hrs.  What is the 	reliability for a mission time of 100 hrs.

3.a)	Derive the expressions for reliability of (i) series systems (ii) parallel systems.

b)	A four engine air craft can operate only if atleast two engines are working.  It has 	two generators driven by the engines; each one of which can supply the minimum 	load demand.  Write down the various system configurations possible and 	compare reliabilities.

Reliability of engine RE = 0.8

Reliability of generator RG = 0.9.

4.a)	Explain conditional probability method for calculation of reliability of non-series-	parallel systems.

b)	Calculate the reliability of the system shown in figure by network reduction.  The 	figures marked indicate the reliabilities of the components.

5.a)	Explain the method of calculating steady state probabilities for a single 	component with repair by two state markov process.

b)	Explain frequency of occurrence of state.

Contd….2

Code No:420204				-2-				Set No:2

6.	Give the state space diagram of a two component system with dependent failures.  	The dependence is of the form no component can fail while the other is down.  	Also get the transition intensity matrix.

7.	A generation system consists of 2 units of 30Mw capacity each with =0.04/year 	and =0.96/year.  The daily peak loads observed are found to be as follows:

Daily peak load		% time the peak has occurred

(MW)

50					30

40					40

30					20

20					10

Estimate the loss of load probability.

8.a)	Explain SDEF load model and give the state space diagram for SDEF load model.

b)	Give the state space diagram for two systems interconnected with limited tie-line 	capacity.

- - -

Code No: 420204

Set No

3

IV-B.Tech. II Semester Examination April, 2003

POWER SYSTEM RELIABILITY

(Electrical and Electronics Engineering)

Time: 3 Hours							Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1.a)	Define the terms “Reliability, hazard function”.

b)	In a sample of 100 nails, 10 are found to be defective of the head, 15 are found to 	be defective of the tail and 5 are found to have both the defects.  What is the 	probability of picking a nail without any defect?

2.a)	Derive the expressions for probability density function, reliability and MTTF of 	irrepairable components with exponential distribution.

b)	The hazard function of a component follows the Weibull distribution with the 	scale parameter =0.05/hr and shape parameter =0.5.  Calculate the MTTF.  	What is the reliability for a mission time of 100 hours.

3.a)	With a neat sketch explain the various regions of “bath-tub” curve.

b)	An electronic equipment has the following components with failure rates shown 	against them.

S.No.		Component		Quantity		failure rate/hr

1.		Transistors		     5			0.015X10-5

2.		Diodes			     8			0.055X10-5

3.		Resistor		   25			0.007X10-5

4.		Capacitors		   12			0.025X10-5

5.		Soldering joints	   70			0.0015X10-5

Determine the reliability of the system for a mission time of 10,000 hrs.  What is 	the MTTF of the system.

4.a)	What is a minimal cut-set?

b)	A bridge network is shown in the figure.  Calculate the system reliability if each 	component has a reliability of 0.9.

Contd……2

Code No:420204				-2-				Set No:3

5.	Give the state space diagram of a system consisting of two identical components 	with independent failures, but only one repair facility.  When one component is 	down, if the other component fails, it has to wait.  Also get the transition intensity 	matrix after merging the states.

6.a)	Explain the terms exact margin states and cumulative margin states.

b)	A generating unit has failure rate =0.4/year and repair rate =9.6/year.  	Determine the probability of failure of the unit after 8 hrs if the unit was working 	at t = 0.

7.a)	What are various failure modes of a circuit breaker?

b)	Give the state space diagram of a circuit breaker with the component protected by 	it.

8.a)	What are the criteria for failure of a bulk power system.

b)	Explain the concept of Generation Redespatch.

- - -

Code No: 420204

Set No

4

IV-B.Tech. II Semester Examination April, 2003

POWER SYSTEM RELIABILITY

(Electrical and Electronics Engineering)

Time: 3 Hours							Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1.a)	Explain the concept of continuous and discrete random variables with one 	example for each.

b)	Explain Binomial and Poisson distributions.

2.a)	Explain the failure time distributions of components using bath-tub curve.

b)	Explain the method of calculation of reliability of series systems and parallel 	systems.

3.a)	Explain the concept of cut-sets and tie-sets.

b)	Calculate the reliability of the bridge network using the minimal cutsets.

4.a)	Explain the state space method of system reliability evaluation.

b)	Explain two state Morkov process for the calculation of steady state probabilities.

5.a)	Explain frequency and duration method.

b)	A generation system consists of two units of capacities 20 MW and 30 MW with 	forced outage rates 0.1 and 0.05/day respectively.  If the mean time to repair of 	either unit is 20 days, calculate the probability and frequency of failure of the 	system if it has to deliver a steady load of 25 MW.

6.a)	Explain loss of load probability and loss of energy probability.

b)	What is a margin state?  How the frequency of occurrence of a cumulative margin 	state is calculated?

Contd……2

Code No:420204				-2-				Set No:4

7.a)	What are the assumptions made in interconnected systems?

b)	Give the state space diagram of two interconnected systems with limited tie-line 	capacity.

8.a)	Give the state space diagram of two components A and B with switching after 	faults.

b)	Explain the weighted average rate model.

- - -