Code No: 420302

Set No

1

IV-B.Tech. II Semester Examination April, 2003

ENVIRONMENT AND POLLUTION CONTROL

(common to Mechanical Engineering,  Production Engineering.)

Time: 3 Hours							Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1.a)	A sample of air contains 8.583 moles /m3 of oxygen and 15.93 moles/m3 of 	nitrogen at STP.  Determine the partial pressures of oxygen and nitrogen in 1.0m3 	of the air.

b)	List the major air pollutants for which the US EPA has designated air quality 	standards, mention its importance and its effects on environment?

2.a)	Discuss the different methods which determine the health effects of air pollutants on 	human being.

b)	Explain how Acid rains occur.

3.	Explain  Air – Fuel ratio & also explain the effects of Air-fuel ratio on exhaust 	composition with the help of a neat diagram

4.a)	What are the two exchange mechanisms working  between earth & the 	atmosphere, through which pollutants are continually removed

b)	Explain the terms

  - Subsidence inversion

  - Radiational inversion

5.	Enumerate the term “Stack monitoring” and indicate the importance of stack 		monitoring in air pollution.

6.	Name and describe three control devices developed for control of automotive 	emissions.

7.a)	Explain working of settling chamber for particulate removal.

b)	Discuss the natural removal mechanism that work in the atmosphere.

c)	A settling chamber of size is 12m 2m , 2m is processing 240 m3/hr of air at temp 	of 75o c.  Determine the maximum size of particle with specific gravity of 1.8 that 	is removed with a theoretical efficiency of 100 %.

8.	An electrostatic precipitator has to be designed to process 5 m3 /s of stack gas. 	The rift velocity of dust  particles  is   w = 1.5 x 10dp   m/s. Determine  the  plate 	area  required  to remove particles with dia of 0.7  m  with  95% efficiency.

$$$

Code No: 420302

Set No

2

IV-B.Tech. II Semester Examination April, 2003

ENVIRONMENT AND POLLUTION CONTROL

(common to Mechanical Engineering,  Production Engineering.)

Time: 3 Hours							Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1.	With the help of the schematic diagram discuss in detail the reactions of nitrogen 	xides in the atmosphere?

2.	Explain the ways through which the material damage occurs due to air pollution.

3.a)	Methane is burned in air with 20% excess air & air moisture of 0.0116 mol/ mol 	dry air.  Calculate the stiochiometric Air Fuel Ratio & actual AFR.

b)	Describe the two methods which are used to control NOx emissions from 	automobiles

4.a)	Explain the effect of terrain roughness on the wind velocity profile figure.

b)	Explain how the dilution and transportation of pollutants occur through air 	turbulence.

5.a)	Discuss the Air Pollution dispersion modeling for predicting ambient air 			pollutant concentrations.

b)	Explain the Estimation of Plume rise by using various formulae?

6.	Name and describe the suitable solvents used in industry for the removal of 	various gaseous Pollutants.

7.a)	Explain the working principle of a filter bag with a  neat sketch.  Also explain the 	cleaning process of the filter bags

b)	Compare cyclone separators and filter bags

c)	Under what industrial  conditions, the use of a fabric filter will be appropriate.

8.	Outline a programme for the air quality monitoring of a industrial city.   What are 	the problems and limitations of sampling and measurement?

$$$

Code No: 420302

Set No

3

IV-B.Tech. II Semester Examination April, 2003

ENVIRONMENT AND POLLUTION CONTROL

(common to Mechanical Engineering,  Production Engineering.)

Time: 3 Hours							Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1.	Write a summary on natural air pollutants and air pollution due to anthropogenic

activities.

2.	Explain the effects on plants due to the following air pollutant.

i)	sulpher dioxide	ii)	chlorine

iii)	oxides of nitrogen       iv)	ozone

v)	PAN			vi)	Ammonia

vii)	Herbicides		viii)	Flouride compounds

3.a)	How can the thermodynamics be applied for the control of HC, in combustion

process?

b)	Explain the thermodynamics of the combustion of natural gas.

4.	Explain the plume behavior with figures under various conditions of atmospheric 	stability.

5.a)	Explain the Radiation Inversion and Subsidence Inversion

b)	A rising Plume of Stack gas has a temperature of 10000C at 200m. Assuming 		a dry adiabatic lapse rate, determine the temperature at 1000m.

6.a)	Name and discuss four methods of controlling emissions at the source

b)	Discuss the Gaseous pollutant control systems based on absorption in a liquid.

7.a)	Explain the working principle of a  cyclone  separators with a neat sketch

b)	A cyclone of standard dimensions with a diameter of  1.6m process  4.5m3  /s of 	air with a temperature of 50oC. Determine the minimum diameter of the particle 	which can be completely removed   if the specific gravity of the particle is 1.2.  	Assume necessary data suitably.

8.	Explain air quality monitoring and its importance.

$$$

Code No: 420302

Set No

4

IV-B.Tech. II Semester Examination April, 2003

ENVIRONMENT AND POLLUTION CONTROL

(common to Mechanical Engineering,  Production Engineering.)

Time: 3 Hours							Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1.		In the city of Hyderabad what are the major sources of air pollution? Describe 			them briefly.

2.		Write short notes on

- mechanism through which the air pollutant reaches and affects the

animals.

- allergic agents of human health.

3.a)	How the thermodynamics  can be applied for the control of SO2 in combustion

process

b)	How the thermodynamics can be applied  for the control of CO in combustion 	process.

4.	How the following parameter influences the Air quality

- Heat

- Pressure

- Wind

- Moisture

5.a)	What weather is usually associated with high-pressure systems? Low- pressure 			systems? Discuss the weather generated by such systems in relation 				to air Pollution Problems.

b)	Pressure is an important variable in meteorological phenomena. Explain?

6.	What are the principal gases of concern in air pollution control?   Explain the four 		primary types of treatment process available for control of gases.

7.a)	Suggest a simple device for removing  coarse  particles from dirty gas stream, 			by the method of gravitational settling

b)	Describe the advantages and suitability of  the  various materials used for making 			bag  filters

8.	What is air quality monitoring? Explain the procedure for monitoring of SO2 and 			NO.

$$$