Code No. 421051

OR

IV-B.Tech. II Semester Examinations April, 2003

DIGITAL SIGNAL PROCESSING

(common to Electronics and Instrumentation Engineering, Electronics and Control Engineering)

Time: 3 hours							        Max. Marks: 70

Answer any FIVE questions

All questions carry equal marks

- - -

1. a)	Prove that a discrete linear shift invariant system is stable if and only if its unit sample response is bounded.

b)	Prove that the convolution in time domain leads to multiplication in frequency domain for discrete time signals.

c)	Plot the magnitude and phase response as a function of frequency for  ≤ , for the system whose impulse response is given by

h(n) = { aunu(n)a<1for n≥ 0

for n ≤ 0

2. a)	State and prove the parseval’s property of DFTs.

b)	Find the DFT of the sequence (1, -2, 0, 3).  From this find the DFT of the sequence (6, 2, -4, 0).  Prove the properties you have used.

c)	Perform and obtain circular convolution of (-1, 0, 2, 1) with (1, 2, -1, 0) using DFT approach.

3. a)	Explain the symmetry properties of DFTs which provide the basis for fast algorithms.

b)	Show that to any DIT FFT algorithm there corresponds to an inverse transform algorithm that is DIF.

c)	Obtain the DFT of the following sequence using Radix-2, DIF FFT flow graph.  Show all intermediate results.

x(n) = { 0, 1, 2, 0, 0, -2, -1, 0}.

4. a)	State and prove the convolution property of z-transforms.

b)	Determine the causal sequence whose z-transform is given by

x(z)=z2(z−3)(z−4)

c)	Determine the respuse y(n), n≥0 of the system descubed by the difference equation.

y(n) – 3y(n-1) – 4y(n-2) = x(n) + 2x(n-1) to the input x(n) = 4n u(n).

Contd…2

Code No: 421051					..2..				OR

5. a)	How do you obtain the frequency response of a discrete time system?  Map s-plane on to z-plane.

b)	State the advantages of cascade realization of filter structures.

c)	Realize the following transfer function with cascaded second order section.

H(z)=(5+4z+5z−1+12z−2)(4−2z−1+z−2)

6. a)	What is frequency warping in Bilinear transformation and how do you over come it in the design of IIR digital filters?

b)	An IIR digital filter is required for the following specifications;

0.8 ≤  H(ejwj ≤ 1 for 0 ≤ w ≤ 0.2 

H(ejw) ≤ 0.2 for 0.6  ≤ w ≤ 

Obtain H(z) of the corresponding IIR digital fitter using Butter worth approximation and Bilinear transformation technique.

7. a)	Explain the necessity of windows in the design of FIR digital filters.  Enumerate the desirable properties of windows.

b)	Design an ideal linear phase Band-pass FIR digital fitter of length 7 samples using Hamming window for the following specifications:

(i)	Pass-band: 1000Hz to 2000Hz

(ii)	Sampling frequency: 1000Hz

- Pass band gain: Unity

Obtain the impulse response and H(z) of the desired fitter.

8.	Write short notes on the following:

(a)	Impulse invariance transformation technique in the design of IIR digital fitters.

(b)	Applications of FFT in spectrum analysis.

(c)	Speech processing.

###