Code No:420403

Set No.

1

IV-B.Tech. II Semester Supplementary Examinations July, 2003

INDUSTRIAL ELECTRONICS

(Common to Electronics and Communication Engineering, Electronics and Instrumentation Engineering and Mechatronics.)

Time: 3 hours						                  Max.Marks: 80

Answer any five questions

All questions carry equal marks

- - -

1.a)	What are the different drift compensation techniques in DC amplifiers?  Explain in detail about one such technique.

b)	A dc amplifier has a voltage gain of 40dB.  When the ambient temperature increases from 25 to 50oC the output changes by 0.25V.  What is the drift of the amplifier referred to the input?

2.a)	Draw a BJT series regulator circuit and its equivalent circuit.  What should be the value of Ro of the circuit?

b)	Draw the internal circuit of a servo voltage stabilizer.

3.a)	Give the two transistor version of PNPN diode and explain its working principle.

b)	Draw the VI characteristics of PNPN diode and make all regions of operation in the graph and explain them.

4.a)	Give the basic structure and circuit symbol of diac.  Draw its V I characteristics and explain the same.

b)	How is the operation of PN PN diode dependent on temperature.

5.a)	Describe the different output voltage controls in inverters.

b)	What is the scope of sequence times in automatic welding processes.  Explain the principle of sequence timer.

6.a)	Describe the piezo-electric generator for generating ultrasonic waves.

b)	Explain about a typical inverter circuits.

7.	Explain in detail the testing of materials by ultrasonic waves.

8.	Write short notes on:

a) Soldering and welding by ultrasonics.

b) Thyristor inverter using centre tapped load.

- - -

Code No:420403

Set No.

2

IV-B.Tech. II Semester Supplementary Examinations July, 2003

INDUSTRIAL ELECTRONICS

(Common to Electronics and Communication Engineering, Electronics and Instrumentation Engineering and Mechatronics.)

Time: 3 hours						                  Max.Marks: 80

Answer any five questions

All questions carry equal marks

- - -

1.a)	Explain how a difference amplifier can be used as a d.c. amplifier.

b)	Write a short note on drift compensation techniques.

2.a)	Draw a single BJT shunt regulated circuit and derive the stabilization factor Sv for the same.

b)	Explain how the above circuit improves the performance when compared to a  simple Zener diode regulator.

3.a)	Draw a two transistor equivalent circuit of SCR.  Also draw the V I characteristics of SCR and explain.

b)	What is intrinsic stand off ratio of UJT?  Derive an expression for the same.

4.a)	What is a silicon controlled switch.  How does it differ from SCR?  Explain.

b)	What is triac?  Draw the basic structure and circuit symbol of triac.  Explain its working with the help of its V I characteristics.

5.a)	Explain how the control of voltage with in the inverter is done.

b)	Draw a neat circuit of AC Resistance welding and explain the same.

6.a)	Draw a basic circuit arrangement of energy storage welding.

b)	What are the different energy storage welders available?  Explain their working.

7.a)	What is the principle of Induction heating.

b)	Write about the Electrodes used in dielectric heating.

8.	Write short notes on:

a) Applications of dielectric heating in industry.

b) Ultrasonic flow delection.

- - -

Code No:420403

Set No.

3

IV-B.Tech. II Semester Supplementary Examinations July, 2003

INDUSTRIAL ELECTRONICS

(Common to Electronics and Communication Engineering, Electronics and Instrumentation Engineering and Mechatronics.)

Time: 3 hours						                  Max.Marks: 80

Answer any five questions

All questions carry equal marks

- - -

1.a)	Draw a neat circuit diagram of the dc-amplifier with potential divides coupling circuit.  Explain the working of the above circuit.

b)	A dc amplifier has the following drift co-efficients ∂ViodT=1mv/oC;∂Vio∂Vps=0.5mv%;∂Vio∂t=5mvday.  Find the maximum input offset voltage over a temperature range of 50oC and a supply voltage regulation of 0.5% over a period of one day.

2.a)	Draw a neat current limiting and overload protection circuit in power supplies.  Derive the expression for current stabilization ratio Sc.

b)	Write a note on monolithic voltage regulators.

3.a)	Draw a three junction device characteristics.  Derive the expression for total current across the junction J2.

b)	Explain the working of UJT.

4.a)	Draw a neat V I characteristics of triac and explain the same.

b)	Draw different biasing modes of diode transistor equivalent of silicon controlled switch.

5.a)	Explain the working of a single phase inverter using Thyristors.

b)	Describe the measures adopted for over current protection.

6.a)	Explain the principle behind the working of time-varying voltage generators.

b)	Describe the resistance welding process.

7.a)	What are the electronic welding controls used in resistance welding.

b)	Write a note on theory of induction heating.

8.	Write short notes on:

a) Theory of dielectric heating.

b) Generation of Ultrasonic waves.

- - -

Code No:420403

Set No.

4

IV-B.Tech. II Semester Supplementary Examinations July, 2003

INDUSTRIAL ELECTRONICS

(Common to Electronics and Communication Engineering, Electronics and Instrumentation Engineering and Mechatronics.)

Time: 3 hours						                  Max.Marks: 80

Answer any five questions

All questions carry equal marks

- - -

1.	For a dc amplifier of multistage using Silicon BJTs calculate the Quiescent currents and voltages of each stage (say for 3 stages) with the following component values: Vcc=6V, RL1=100K, RL2=10K and  RL3=1K.  Assume all diodes are to be shorted out.  RF=200k,  RF2=100K, VBE=0.6V and =100.

2.	For a series regulator circuit, if we have Vi=182V, Vo nom=12V, IL=50 to          250 mA; Calculate the specification for the series BJT and Zener diode and the values of RD.

For the above specifications also calculate Sv and Ro for the following device parameters: hfe=44, hie = 5.5 ohms at Ic=250 mA, rz=12 ohms, Vz max=12.6V,     Vz min=11.4 V, Pz max=500 mw, hfe max=130, Vz nom = 11.7 V and Rs=0.

3.a)	Draw a neat circuit diagram of an SCR.  Explain  its characteristics.

b)	Explain the principle of UJT.  Derive the equation for the firing voltage VF.

4.a)	Draw a neat half wave rectifier circuit connections of SCR and explain its working.

b)	What are the different applications of SCR.  Explain about them.

5.a)	Draw a neat bridge type thyristor invertor circuit and output waveforms.

b)	Explain the working of the above circuit.

6.a)	Explain how controlling of voltage input to the invertor takes place.

b)	What are the different types of Resistance welding?  Explain about them in detail.

7.a)	What are the merits of Induction heating.

b)	Give the applications of Induction heating in detail.

8.	Write short notes on:

a) Applications of dielectric heating in industry.

b) Applications of ultrasonic waves.

- - -