Code No: 421302

Set No.

1

IV-B.Tech. II-Semester Supplementary Examinations July, 2003

Computer aided design of control systems

(Electronics and Control Engineering)

Time: 3 hours						               Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	What is a system? How system models are obtained?

b)	Explain how system matrices are generated.

2.a)	Define the following terms

(i) Over shoot		(ii) Transient frequency	(iii) Offset	(iv) Noise.

b)	Explain the term stability.

3.	For the system given by

pg(s)= S 0 0 −6 −2−1 S 0 −11 1 0 −1 S −3 0 0 0 −1 S+3 0

form the matrices

(B AB……An-1B) and CCA..

4.	If q(s) be strictly proper, and f(s)=f, independent of s, prove that for given any f1>0, the radius R of the semicircle in the contour D can be chosen large enough to ensure that no zero of Th(s) lies on this semi circle for any f satisfying 0ff1.

5.	Explain how the phase margin Mm can be obtained for compensated plant directly from Inverse Nyquist plot for a system having small phase margins.

6.	If  so is a triple pole of h(s), how do the root loci depart from so?

Contd…2

Code No. 421302				-2-				Set No.1

7.a)	What are the multi-variable systems?

b)	Show that any decoupling zero of pk, pL or pF is a decoupling zero of pH (with usual notation). Is the converse true?

8.	Write short notes on

a)	M-files

b)	Menu-driven options in MATLAB.

^^^

Code No: 421302

Set No.

2

IV-B.Tech. II-Semester Supplementary Examinations July, 2003

Computer aided design of control systems

(Electronics and Control Engineering)

Time: 3 hours						               Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	What is relative left prime, relative right prime?

b)	What is the test for primeness?

2.a)	Define the following terms.

(i)	Speed of response	(ii)	Conditional stability

(iii)	Offset			(iv)	Band-width.

b)	What are decoupling zeros.

3.	In inverse Nyquist plot show that if q(s) arises from a least order system and Tf(s)=1 Nh= pq-po-Zq becomes Nh= -Zq (for then pq = ps)

4.	Show that the phase margin and the value of Mm are in variant under transfer of gain from the forward path to the feedback path.

5.	If so is a triple pole of h(s), how do the root loci depart from so?

6.	Show that for the open-loop system

Q(s) =11+s 23+s	the point (10,10) is in the unstable region of the gain space.

7.	Explain multivariable circle criteria.

8.	Write short notes on

a)	For loops in MATLAB.

b)	Plotting facilities available in MATLAB.

^^^

Code No: 421302

IV-B.Tech. II-Semester Supplementary Examinations July, 2003

Set No.

3

Computer aided design of control systems

(Electronics and Control Engineering)

Time: 3 hours						               Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1. a)	What is a system?  How system models are obtained?

b)	What is the test for primeness?

2. a)	Define the following terms

i)	Over Shoot		ii) 	Conditional Stability

iii)	Band width		iv)	Off set

- What are decoupling zeros

3. 	Show that the phase margin and the value of the Mm are invariant under transfer of gain form the forward path to feedback path.

4	If so  is a triple pole of h(s), how do the root loci depart from so?

5.	For the system given by

Pg (s)= S 0 0 −6 −2−1 S 0 −11 1 0 −1 S −3 0 0 0 −1 S+3 0

form the matrices

(B AB……An-1B) and CCA..

Contd…2

Code No. 421302				-2-				Set No.3

6.	Show that for the open-loop system Q(s) =11+s 23+s the point (10,10) is in 	the unstable region of the gain space.

7.	Explain Multivariable circle criteria.

8.	Write short notes on

a)	Control system tool boxes in MATLAB.

b)	Menu driven options.

^^^

Code No: 421302

IV-B.Tech. II-Semester Supplementary Examinations July, 2003

Set No.

4

Computer aided design of control systems

(Electronics and Control Engineering)

Time: 3 hours						               Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1. a)	What is relative left prime, relative right prime?

b)	Explain how system matrices are generated.

2. a)	Define the following terms

(i) Over shoot	 (ii) Off set	(iii) Band width    (iv)  Noise.

b)	Explain the term stability.

3	In inverse Nyquist plot show that q(s) arises from a least-order system and  Tf(s)=1 Nh= pq-po-Zq becomes Nh= -Zq (for then pq = po)

4.	If so is a triple pole of h(s), how do the root loci depart from so?

5.	Show that for the open-loop system Q(s) =11+s 23+s the point (10,10) is in 	the unstable region of the gain space.

6. a)	What are the multi-variable systems?

b)	Show that any de coupling zero of pk, pL or pF is a decoupling zero of pH (with usual notation). Is the converse true?

7.	Explain multivariable circle criteria.

8.	Write short notes on

a) M-Files

b) Menu-driven options in MATLAB.

^^^