Code No:420805

Set No.

1

IV-B.Tech. II-Semester Supplementary Examinations July 2003.

OPTIMIZATION OF CHEMICAL PROCESSES

(Chemical Engineering)

Time: 3 hours							Max. Marks:80

Answer any FIVE questions

All questions carry equal marks

- - -

1.	Find the area of the largest rectangle with its lower base on the x-axis and whose corners are bounded at the top by the curve : y = 10 – x2 .  Formulate the objective function, the equality constraints(if any) and inequality constraints(if any). Specify and list the independent variables, the number of degrees of freedom, and the coefficients in the optimization problem.

2.	Describe the methodology of operating cost estimation mentioning all the normal         components and percentages used in standard practice.

3.	Explain the difference between uni model and multi model functions

4.	Describe in detail the basic concepts involved in linear programming?

5.	Write the procedure for optimizing the process design of a baffled shell- and-tube single-pass counter flow heat exchanger in which the tube fluid is in turbulent flow but there is no change of phase of fluids in the shell or tubes.

6.a)	What are the categories into which optimization problems for steady state distillation are classified in general? Discuss briefly.

b)	With the help of a schematic diagram, formulate the equality constraints for optimal design and operation of a conventional staged distillation column.

7.	The cost function C representing the annual costs of a pipe line transporting a fluid is given by

C =  C1 D1. 3 L + 0.142 (C0 /)m2.8 0.2-2.0D-4.8L

where   the cost coefficients are considered as C0 =  Rs. 0.59 and C1 = Rs. 5.7. The mass flow rate of fluid  m = 25 kg/s, density  = 1000 kg/m3 ,   = 1.08 x 10-3 N/sm2 , the pumping efficiency   = 0.60 and the pipe length  L= 10 m.

The fluid velocity is correlated by

v = 4 m / (D2)

Find the  optimal pipe diameter Dopt  and optimal fluid velocity vopt .

Contd….2

Code No:420805				-2-				Set No:1

8.	In a refinery blending operation, crude oil-1 and crude oil-2 are used to produce gasoline, kerosene, fuel oil and residual. The linear programming problem of the refinery operation is setup as follows.

The objective function to be maximized is

f(x) =   8.1 x1 + 10.8 x2

The inequality constraints are:

Gasoline:  0.80 x1 + 0.44 x2    24,000

Kerosine:  0.50 x1 + 0.10 x2    2,000

Fuel oil:    0.10 x1 + 36 x2     6,000

and x1   0 ,    x2   0

Assume the variables are in liters/day denoted by  x1 = crude-1, x2 = crude-2, x3 = gasoline, x4 = kerosene, x5 = fuel oil and x6 = residual.

Use the above information and plot the constraints on  x1 - x2  plane.  Determine the feasible region for x1 and x2  that satisfy the constraints.

- - -

Code No:420805

Set No.

2

IV-B.Tech. II-Semester Supplementary Examinations July 2003.

OPTIMIZATION OF CHEMICAL PROCESS

(Chemical Engineering)

Time: 3 hours							Max. Marks:80

Answer any FIVE questions

All questions carry equal marks

- - -

1.	Three points on x are selected a distance h apart ( x0, x0+h, x0+2h) with corresponding values of f0, f1, and f2.  Find the maximum or minimum attained by a quadratic function passing through all three points. Also formulate the objective function and indicate the equality and inequality constraints(if any). Give the number of degrees of freedom and the coefficients in the optimization problem.

2.	Mention the process variables used in cost estimation for different process equipment like : Flash drum, Distillation column/Tray absorber, Condenser/Reboiler/Heat exchanger, etc.

3.	Consider the following objective functions:

Is it convex ? use eight values in the analysis.

f(x) = 2x12 + 2x1x2 + 1.5x22+7x1+8x2+24

4.	Write in detail the simplex method of solving linear programming problems

5.	Enlist the variables that are specified a priori by the designer, the variables to be calculated via optimization and the variables used for minimizing the objective function in the optimization of a shell-and-tube heat exchanger design (single-pass counterflow heat exchanger in which the tube fluid is in turbulent flow but there is no change of phase of fluids in the shell or tubes).

6.a)	Describe the classification of optimization problems for steady state distillation.

b)	Formulate the equality constraints for optimal design and operation of a conventional staged distillation column with the help of a neat schematic diagram.

Contd….2

Code No:420805				-2-				Set No:2

7.	The following first reaction takes place in a well mixed batch reactor

C

k2

k1                               k3

A                            B

k4	        D

The initial concentrations in mol/lit  are:

cAo =1.0 and cBo = cCo = cDo = 0.0

The reaction rates in h-1are:

k1 = 0.001,  k2 = 0.002,  k3 = 0.05  and  k4 = 0.02

Calculate the optimal residence time to yield a maximum concentration of B.

8.	Various feeds and product distribution for a thermal cracker which produces olefins are listed in weight fractions in the following table.

Feed

Product                    Ethane        Propane         Gas Oil          DNG

Ethane                           0.40            0.06              0.04              0.05

Ethylene                        0.50            0.35              0.20              0.25

Propylene                      0.01            0.15              0.15              0.18

Propane                          --               0.10              0.01              0.01

Butadiene                       0.01           0.02              0.04              0.05

Gasoline                         0.01           0.07              0.25              0.30

The capacity to run gas feeds through the cracker is 200,000 kg/stream hour. Ethane uses the equivalent of 1.1 kg of capacity per kg of ethane, propane uses 0.9 kg of capacity per kg of propane, gas oil uses 0.9 kg/kg and DNG has a ratio 1.0. Down stream processing limits exists of 100,000 kg/stream hour on the ethylene and 20,000 kg/stream hour on the propylene. Methane and fuel oil produced by the cracker are recycled as fuel. All the ethane and propane produced is recycled as feed. Using the above data, setup the constraint equations on cracker capacity, ethylene processing limitation, ethane recycle and propane recycle.

- - -

Code No:420805

Set No.

3

IV-B.Tech. II-Semester Supplementary Examinations July 2003.

OPTIMIZATION OF CHEMICAL PROCESS

(Chemical Engineering)

Time: 3 hours							Max. Marks:80

Answer any FIVE questions

All questions carry equal marks

- - -

1.	Find the volume of the largest right circular cylinder that can be inscribed inside a sphere of radius R. Formulate the optimization problem, indicate the equality and inequality constraints(if any) and specify and list the independent variables, the degrees of freedom and the coefficients in the optimization problem.

2.	Discuss the methodology normally followed to account for the effects of inflation.

3.	Classify the function f(x) =2x12 - 3x1x2 + 2x22

4.	Use the simplex procedure with un restricted variables

minimize 	f(x) = x1 +  4 x2

subject to 	x1 + x2  ¡Ü  3

-x1 + x2  ¡Ü   1

x1 unrestricted, x2 ¡Ý  0

find the optimal values of x1 and x2?

5.	In the design of a shell-and-tube heat exchanger (single-pass counterflow heat exchanger in which the tube fluid is in turbulent flow but there is no change of phase of fluids in the shell or tubes) mention the total number of design variables, both given and unspecified. Give the design relationships involved. Thus determine the degrees of freedom for optimization.

6.a)	Categorize the optimization problems for steady state distillation and discuss briefly.

b)	List the equality constraints for the optimal design and operation of a conventional staged distillation column with a neat schematic diagram.

7.	The following reactions occur in series in batch reactor

k1                                        k2

A                            R                                S   ,       k1  = k2

Find  the maximum concentration of R and the time at which it is reached.

Contd….2

Code No:420805				-2-				Set No:3

8.	In a refinery blending operation, crude oil-1 and crude oil-2 are used to produce gasoline, kerosene, fuel oil and residual. The linear programming problem of the refinery operation is setup as follows.

The objective function to be maximized is

f(x) =   8.1 x1 + 10.8 x2

The inequality constraints are:

Gasoline:  0.80 x1 + 0.44 x2    24,000

Kerosine:  0.50 x1 + 0.10 x2    2,000

Fuel oil:    0.10 x1 + 36 x2     6,000

and x1   0 ,    x2   0

Assume the variables are in liters/day denoted by  x1 = crude-1, x2 = crude-2, x3 = gasoline, x4 = kerosene, x5 = fuel oil and x6 = residual.

Use graphical method and find the optimum values for x1 ,  x2  and objective function.

- - -

Code No:420805

Set No.

4

IV-B.Tech. II-Semester Supplementary Examinations July 2003.

OPTIMIZATION OF CHEMICAL PROCESS

(Chemical Engineering)

Time: 3 hours							Max. Marks:80

Answer any FIVE questions

All questions carry equal marks

- - -

1.	By formulating the objective function, specifying the equality and inequality constraints if any, list of independent variables, the number of degrees of freedom and the coefficients in the optimization problem : find the point on the curve –

f  = 2 x2 +3x +1 nearest to the origin.

2.	The installed capital cost of a pump is $ 200/hp and the operating costs are 4 cents/kWh. For 8000 h/year of operation, an efficiency of  70  percent, and a cost of capital i  = 0.10 for n = 5 years, determine the relative importance of the capital versus operating costs.

3.	Does the following set of constrains that from a closed region a convex region

-x12 + x2   ¡Ý1

x1- x2 ¡Ü -2

4.a)	What are the natural occurrence of linear constraints?

b)	Suppose three ( light naphtha, heavy naphtha, and ¡°catalytic¡± oil     made in a refinery are to be blended to produce an aviation fuel. The octane number of the fuel must be at least 95. the octane numbers for the three intermediates are as follows:

Amount blended,		Octane No

bbl/day

Light naphtha		x1			92

Heavy naphtha	x2			86

Catalytic Oil		x3			97

Write an inequality constraint on the octane No of the aviation fuel assuming a linear mixing rule.

5.	What are the variables that an engineer must consider in the selection of optimal evaporation equipment? Make a list of them under separate headings of “known” and “unknown”. Also mention the features to be determined and the utilities that may be consumed.

6.	Describe the procedure for optimization of liquid extraction column flow rates given that a plug flow model accurately represents the data. Make necessary assumptions.

Contd….2

Code No:420805				-2-				Set No:4

7.	The expression for the optimal pipe diameter is given by

Dopt = 0.900 [C0 / (C1gc)]0.164 m0.459 0.033-0.328

where the  mass  flow rate  of  fluid   m  =  35 kg/s,  density   = 1000 kg/m3 , viscosity   = 1.08 x 10-3 N/sm2 , the pumping efficiency   = 0.70 and the pipe length  L= 10 m.   The cost coefficients are considered as C0 =  Rs. 0.59 and

C1 = Rs. 5.7.

The mass flow rate is correlated by

m = (D2/4)v

Find the  values of optimal pipe diameter Dopt  and optimal fluid velocity vopt .

8.	Find the maximum of the following linear programming problem representing a thermal cracker by graphical method.

The objective function :

f = 2.84 x1 - 0.22 x2  - 3.33 x3 + 1.09 x4 + 9.39 x5 + 9.51 x6

where x1 = fresh ethane feed, x2 = fresh propane feed, x3 = gas oil feed, x4 = DNG feed,  x5 =  ethane recycle and x6  = propane recycle.

The objective is subjected to the following constraints:

1.1 x1 + 0.9 x2  + 0.9 x3 + 1.0 x4 + 1.1 x5 + 0.9 x6     200,000

0.5 x1 + 0.35 x2  + 0.25 x3 + 0.25 x4 + 0.5 x5 + 0.35 x6     100,000

0.01 x1 + 0.15 x2  + 0.15 x3 + 0.18 x4 + 0.01 x5 + 0.15 x6     20,000

0.4 x1 + 0.06 x2  + 0.04 x3 + 0.05 x4 - 0.6 x5 + 0.06 x6   =  0

0.1 x2  + 0.041x3 + 0.01 x4 - 0.9 x6   =  0

xi   0

- - -