Code No. 420454

OR

IV-B.Tech- II Semester Supplementary Examinations July 2003

SATELLITE COMMUNICATIONS

(Electronics and Communication Engineering)

Time: 3 hours							Max. Marks: 70

Answer any Five questions

All questions carry equal marks

---

1. a)	Describe the Orbital effects in communications system performance.

b)	Determine the maximum north latitude that an earth station can have and still communicate with a geosynchronous satellite.

2. a)	Draw and explain the block diagram of simplified Double Conversion transponder for 14/11GHz.

b)	Describe the frequency Reuse Antennas in the spacecraft.

3. a)	Derive the equation of Friss Transmission.

b)	A Satellite at a distance of 40,000km from a point on the earth’s surface radiates a power of 2W from an antenna with a gain of 17dB in the direction of the observer.  Find the flux density at the receiving point, and the power received by an antenna with an effective area of 10m2.

4. a)	Discuss the Design of Satellite link for specified C/N ratio.

b)	The INTELSAT V 14GHz transponder receiver has a G/T of 3.3 dBk-1 and a saturation flux density of -80.3 dBW/m2.  The 11GHz transmitter part of this transponder has a saturated EIRP of 44.4dBW.  The transponder bandwidth is 72MHz, centered at 14205MHz up and 11,155MHz down.  The satellite is used to set up a 72MHz Bandwidth link between two earth stations.  Assume that both uplink and downlink path lengths are 40,000km.  Both transmitting and receiving antennas use 4m diameter dishes, and the receiving earth station has an overall noise temperature of 120K. Both antennas have aperture efficiencies of 60%.  The link operates with 4-dB back off on satellite input and on satellite output.  Determine (i) Uplink EIRP (ii) Uplink Carrier to noise ratio (iii) downlink carrier to noise ratio in dB.

5. a)	Explain CDMA Techniques in a satellite system.

b)	Compare FDMA, TDMA and CDMA Techniques in a Satellite system.

6. a)	Explain different tracking techniques used in tracking satellites with a large antenna.

b)	Explain uplink equipment for the transmitting portion of an FDMA earth station with block diagram.

7. a)	Explain with block diagram frequency reuse feed system for 6/4GHz.

b)	Discuss the Design considerations of small earth station Antennas.

8.	Write short notes on the following:

a)	Telemetry, Tracking and Command system

b)	Single conversion Transponder for 6/4GHz.

c)	Look angle Determination.

###