Code No: 420301.

Set No.

1

IV-B.Tech II Semester Supplementary Examination - July 2003

JET PROPULSION AND ROCKET ENGINEERING

( Mechanical Engineering )

Time: 3 hours							Max.Marks:80

Answer any five questions

All questions carry equal marks

---

1.	In a gas turbine plant comprising a single stage compressor, combustion 	chamber and turbine, the compressor takes in air at 15oC and compresses it to 4 	times the initial pressure with an isentropic efficiency of 85%. The fuel air ratio is 	0.0125 and the calorific value of the fuel is 42000 kJ/kg. If the isentropic 	efficiency of the turbine is 82%. Find the overall thermal efficiency and the 	amount of air intake for a power output of 260 kW. Take the mass of the fuel into 	account. The turbine inlet temperature is 1000 K.

2.	What is meant by thrust? Derive the thrust equation for a general propulsion 	system. Explain the effective jet exit velocity, thrust power and propulsive 	power.

3.a)	What do you mean by Thrust Augmentation?

b)	What are the different methods of Thrust Augmentation? Explain each method.

4.a)	Draw the neat sketch of a supersonic diffuser for a ramjet engine with obliqe and 	normal shacks.

b)	Depict variation of the static pressure through the diffuser from its entry to exit.

5.	What are different types of rocket propulsion systems? Explain with their merits 	and demerits.

6.a) 	Explain different types of propellants used in rocket engines?

b) 	How the rockets are classified based on propellants?

7.a)	Derive an expression for the velocity of a rocket vehicle at the end of the powered 	flight.

b)	What is the effect of mass ratio, specific impulse and burnout time on the 	maximum velocity attained by the rocket-vehicle?

8.a) 	With the help of neat diagram, explain the working of magneto-plasma rocket

engine.

b) 	Can rocket work in vacuum? Explain.

~ ~ ~ ~

Code No: 420301.

Set No.

2

IV-B.Tech II Semester Supplementary Examination - July 2003

JET PROPULSION AND ROCKET ENGINEERING

( Mechanical Engineering )

Time: 3 hours							Max.Marks:80

Answer any five questions

All questions carry equal marks

---

1.	In a regenerator gas turbine cycle air enters the compressor at a temperature 	and pressure of 30oC and 1.5 bar and discharges at 220oC and 5.2 bar. After 	passing through the regenerator the air temperature is 395oC. The temperature 	of air entering 	and leaving the gas turbine is 900oC and 510oC. Assuming no 	pressure drop through the regenerator. Determine (i) the output per kg of air,

(ii) the efficiency of the cycle, (iii) the work required to drive the compressor.

2.a)	What is the principle of working of Thermal jet engines and their classification.

b)	 Derive an expression for Propulsive efficiency of Thermal jet engine.

3.a)	Describe the working of a Turboprop engine

b)	Show the various processes occurring in the engine on h-s diagram.

4.a)	How ram rocket combines the desirable characteristics of a racket and a ram jet?

b)	Discuss various performance curves of altitude with Mach number and Net thrust 	for Ram jet engine.

5.	Explain the working principle of a hybrid rocket engine with a neat diagram? List 	its applications.

6.a)	Explain the working principle of liquid propellant engines?

b) 	What are the important requirements for the rocket propellants?

7.a)	Define the following quantities, write down their formulae and units: specific 	impulse, total impulse, and specific propellant consumption.

b)	Calculate thrust, specific impulse, propulsive, thermal and overall efficiencies of a 	rocket engine from the following data:

Effective jet speed = 1250m/s.

Flight to jet speed ratio = 0.80

Oxidizer flow ratio = 3.5 kg/s

Fuel flow rate = 1.0 kg/s

Heat of reaction per kg of exhaust gases = 2500 kJ/kg.

8.a) 	What is the working principle of plasma arc rocket engine?

b) 	What are the basic components of the plasma arc rocket engine. Explain the 	function of each.

~ ~ ~ ~

Code No: 420301.

Set No.

3

IV-B.Tech II Semester Supplementary Examination - July 2003

JET PROPULSION AND ROCKET ENGINEERING

( Mechanical Engineering  )

Time: 3 hours							Max.Marks:80

Answer any five questions

All questions carry equal marks

---

1.	A simple gas turbine unit consists of single stage compressor, regenerator 	combustion chamber and single stage turbine the initial pressure and temperature 	of the cycle is 1.03 bar and 15.5oC. The pressure ratio of the cycle is 5. The 	maximum temperature of the cycle is limited to 813 K. the isentropic efficiencies 	of the compressor and turbine are 85% and 80%. Take Cp =1.005 kJ/kg K and

r = 1.4 for air and gas and find the flow rate through the system if power capacity 	of the plant is 2000 kW. Neglect the mass of fuel.

2.a)	Discuss the advantages and disadvantages of jet propulsion over other 	systems.

b)	Give the complete classification of thermal jet engines.

c)	‘Rockets are non-atmospheric thermal jet engines’.  Justify the statement.

3.	How are efficiencies of the inlet diffuser and the propelling nozzle of a turbojet 	engine defined and present them on T-s diagram, involved with the main 	thermodynamic cycle with respect to the cycle. Write down their formulae.

4.a)	Distinguish between Ram jet and scramjet engine

b)	Explain the following terms as applied to Rockets and derive equations for them 	(i) Thrust (ii) Thrust power (iii) Propulsive efficiency

5.	Derive the equation for the calculation of exit velocity, pressure and temperature 	and specific impulse.

6.a) 	What are different types of pressurization systems used in liquid propellant   	rocket engines?

b) 	Explain the working of solid propellant rocket engine with a neat diagram.

7.	A rocket nozzle has a throat area of 18 cm2 and combustion chamber pressure of 	25 bar. If the specific impulse is 127.42 sec and weight flow rate 44.145 N/s. 	Determine

i) Thrust coefficient.  			          ii) Propellant weight flow coefficient.

iii) Specific propellant consumption and     iv) Characteristic velocity.

8.a) 	Give the classification of liquid propellant rocket engines.

b) 	With the help of a neat diagram, explain the working of a liquid bi-propellant 	rocket engine.

~ ~ ~ ~

Code No: 420301.

Set No.

4

IV-B.Tech II Semester Supplementary Examination - July 2003

JET PROPULSION AND ROCKET ENGINEERING

(Mechanical Engineering  )

Time: 3 hours							Max.Marks:80

Answer any five questions

All questions carry equal marks

---

1.	Calculate the efficiency and specific workout of a simple gas turbine plant 	operating on Brayton cycle the max and min temperatures are 1000 K and 	288 K 	respectively. The pressure ratio is 6, the isentropic efficiencies of the compressor 	and turbine are 85 and 90% respectively. If the unit consumes 2 tons of oil per 	hour of c.v. 46,500kJ/kg. Determine the power generated the mechanical 	efficiency is 90% and the generator efficiency is 85%.

2.a)	Show that with jet propulsion the condition for max. thought power and 	maximum propulsive efficiency are not the same.

b)	Obtain an expression for over all efficiency of a thermal jet engine for 	atmospheric operation.

3.	What is “after burning” in turbojet engines, and why is it required. Explain 	briefly with the aid of a diagram and represented on T-s chart after burning.

4.a)	Draw the sketch of a pulse jet engine. Write down its main advantages and 	disadvantages.

b)	Describe the working of a scramjet engine. What is its advantages over the ram jet 	engine

5.	Compare the nuclear rocket engine with Magneto Plasma Rocket engine. And 	explain the working principles.

6.a) 	What are the difference between gas pressurization system and pump

pressurization system?

b) 	What are different combinations of liquid bipropellants? Explain.

7.a)	Draw a simple sketch of multi stage rocket vehicle depicting clearly the booster 	stage, propellant tanks and exhaust nozzle, instruments and navigational 	equipment.

b)	Explain briefly the meaning of the following.

i) Booster rocket stage. 	ii) Sustainer stage.    	iii)	Retro Rocket.

8.a) 	Discuss the relative merits of gas-pressurization rocket power plant and the pump-	pressurization system.

b) 	What is meant by mono propellent and bi-propellant fuels for rocket engines? 	Give important examples of each and their uses.

~ ~ ~ ~