Code No.420256

OR

IV-B.Tech. II-Semester  Supplementary Examinations July 2003

ELECTRICAL DISTRIBUTION SYSTEMS

(Electrical and Electronics Engineering)

Time:3 hours								Max.Marks:70

Answer any FIVE questions

All questions carry equal Marks

- - -

1.a)	With a block diagram explain the typical distribution system planning scheme.

b)	Explain the role of computers in distribution system planning.

2.a)	Define the following

i)	Coincidence factor	ii)	Load diversity

iii)	Load factor		iv)	Loss factor .

- Give the classification of loads and discuss briefly their characteristics.

3.a)	Draw a schematic diagram of a radial feeder and explain.

b)	Discuss the criteria for selecting location of substations.

4.a)	Describe the factors affecting the primary feeder loading.

b)	Obtain an expression for voltage drop and power loss in a radial feeder with uniformly distributed load.

5.a)	Prove that the power loss in a single phase lateral feeder is two times the power loss in a three phase lateral feeder for transmitting same power.

b)	In a three phase system the impedance of the line is (R+jx) ohms per phase. Find the load power factor for which the voltage drop is maximum.

6.a)	List the objectives of distribution system protection.

b)	Discuss clearly the co-ordination of protective devices in a distribution system.

7.a)	Compare shunt and series capacitor schemes of compensation.

b)	A 700 KVA load has a power factor of 0.6 5. It is desired to improve the power factor to 0.92. Find the RKVA rating of the capacitor for the power factor improvement.

8.	Write detailed notes on the following:

a)	Loss reduction due to capacitor compensation.

b)	Circuit reclosures and line sectionalizers.

^^^