Code No: 420257

OR

IV-B.Tech. II-Semester Supplementary Examination, July 2003

POWER SYSTEM RELIABILITY

(Electrical and Electronics Engineering)

Time: 3 hours							        Max. Marks: 70

Answer any Five questions

All questions carry equal marks

- - -

1.a)	An electrical power system has 10 generating units of 30 MW capacity each.  If 	the probability of failure any unit is 0.05, calculate the probability of failure of the 	system to supply a steady load of 200 MW.

b)	In a large system the average number of cable faults per year per 100 km of cable 	is 0.5. What is the probability of not more than one fault occurring in a year on a 	cable 10 km long.

2.a)	A truck has four tyres each with a failure rate of 5x10-5/km.  Calculate reliability 	of the truck against tyre failure for a mission of 1000 km.

b)	What will be the improvement in reliability if one spare tyre is taken on the same 	mission?

3.a)	A system consists of a single component with failure rate  and repair rate .  	From the state space analysis derive an expression for its steady state availability.

b)	A system consists of two components A and B with independent failure and repair 	rates.  From the state space analysis derive expressions for the steady state 	probability of occurrence of each state of the system.  Calculate their values given

A=0.2, A=9.8  ;		B=0.4, B=9.6

4.	A system has four generating units A,B,C and D.  A, B and C have a capacity of 	40 MW each while D has a capacity of 80 MW.  The failure and repair rates are 	0.4/year and 9.6/year.  Obtain the state space model with equal capacity states 	merged.

5.a)	Discuss briefly the salient features of ‘Loss of Load’ and ‘Loss of Energy’ 	methods of calculating the probability of capacity deficiency in a system.

b)	In a general system, for 10% of the days during a given period of observation, the 	peakload is between 30 MW and 40 MW, for 15%, between 40 MW and 50 MW, 	for 25% between 50 MW and 60 MW for 30% between 60 MW And 70 MW and 	for rest of the period, between 70 MW and 80MW.  Calculate LOLP if the 	installed capacity is 100 MW with 5 numbers of 20 MW units with  = 0.4/year 	and =9.6/year.

Contd….2

Code No.:420257				-2-				OR

6.	The data regarding generation capacity states of a system and the connector load 	system are tabulated below.  Assuming the system fails when the load is greater 	than the generating capacity, calculate probability and frequency of failure.

Generation System

State no.	State capacity MW	Availability	Departure	rates/year

i			Ci		      A(Ci)	  +(Ci)	  -(Ci)

1			0		0.0016		    19.2		      0

2			20		0.0384		      9.6		      0.4

3			30		0.0384		      9.6		      0.4

4			50		0.9276		      0		      0.8

Load System

State no.	State capacity MW	Availability	Departure	rates/year

j			Lj		      C(Lj)	  +(Lj)	  -(Lj)

1			40		0.2		      0		      2.5

2			30		0.12		      0		      2.5

3			20		0.08		      0		      2.5

4 low load		0		0.6		      1.67		      0

7.a)	Explain how the effect of changing weather conditions are modeled.

b)	Two identical transmission lines operate in a two weather environment with a 	mean normal weather of 10 days and a mean severe weather duration of 0.15 	days.  The line failure rate is 0.00025/day in normal weather and 0.05/day in 	severe weather.  The repair rate is 1.00/day.  Calculate the probability of double 	failure using weighted average method.

8.	Write short notes on the following:

a) Random variables and their distribution.

b) The state model of a component subject to ‘switching’.

c) Uses of Reliability network.

- - -