Code No: 420352

OR

IV-B.Tech. II-Semester Supplementary Examinations July, 2003

Hydraulic Machinery and Systems

(Common to Mechanical Engineering, Production Engineering, and Mechatronics)

Time: 3 hours						               Max. Marks: 70

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Derive an expression for the torque exerted by a fluid jet on a wheel with radial curved vanes.

b)	The resistance to motion of a vessel is 25kN at a velocity of 4.5m/s.  The jet efficiency is to be 80% and the mechanical efficiency of the pumps is 75%.  Hydraulic losses in the ducts are 5% of the relative kinetic energy at exit.  Determine (i) the velocity of the jet (ii) orifice area at exit (iii) power required to drive the pumps for the given speed of the vehicle.

Assume that the water is drawn in through the intakes facing the direction of motion of the ship.

2.a)	With a sketch explain the mechanism for governing of turbines.

b)	A Kaplan turbine produces 44mw under a head of 25m with an overall efficiency of 90%.  Assuming speed ratio = 1.6, Flow ratio = 0.5, ratio of hub diameter to outer diameter = 0.35. Find the diameter and speed of the turbine.

3.a)	Explain how the ‘best performance curve’ can be obtained with the help of characteristic curves of a reaction turbine.

b)	Sketch and explain different parts of a centrifugal pump.  What are their functions?

4.a)	Why can the suction lift of a pump not exceed a certain limit.

b)	A radial type centrifugal pump delivers 5 k litres/min. against a total head of 38m.  when running at a speed of 1450 rpm.  If the outer diameter of the impeller is 0.3m. and its width at outer periphery is 0.013 m. find the vane angle at exit.  Assume manometric efficiency as 80%.

5.a)	Why is a reciprocating pump called positive displacement pump?

b)	A single acting reciprocating pump has a plunger diameter of 20cm. and a stroke of 30 cm.  It is to be driven with SHM at 60rpm and draws water from a sump 3.6m below the pump centre line.  Find the least diameter of the suction pipe, which is 4.5m long in order to prevent separation at this speed.  Take atmospheric pressure as 10.3m of water and the separation occurs at 2.6m of water absolute.

Contd……2.

Code No. 420352				:: 2 ::					OR

6.a)	Distinguish between a hydraulic coupling and hydraulic torque converter.

b)	An accumulator has a ram of 0.3m diameter and 9m lift.  It is loaded with 1 MN total weight.  If packing friction amounts for 2% of the total force on the ram, determine the power delivered to the machine if the ram falls through its full range in 3 min., and if at the same time the pumps are delivering 1360 lit/min.

7.a)	Give the classification of positive displacement pumps and explain working of a gear pump.

b)	What are the desirable properties of hydraulic oils?  Name different hydraulic oils.

8.	Write short notes on the following:

a) Head losses of turbines.

b) Air lift pump

c) Pump duty.

d) Cavitation.

@@@@@