Code No.420203

Set No.

1

- IV-B.Tech. II-Semester Supplementary Examinations, July 2003

ENERGY MANAGEMENT

(Electrical and Electronics Engineering)

Time: 3 Hours							   Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks.

---

1.a)	Explain about the Organization of Energy Management using chart.

b)	What are the Hierarchical levels of supply side Energy Management? And Explain with chart.

2.a)	Discuss about the level of Responsibility in a company to conserve energy.

b)	Consider a company using three energy forms Oil, Gas and Electricity.  The annual energy consumption is shown in table in various energy units. Find percentage energy balance.  Assume

1 Joule = 947x10-6btu = 238.8x10-6calories = 9.478x10-9therm = 2778x10-6wh

1 Joule = 5.6497 x 10-9gal.

Energy Type				Consumption

Oil					10x103 gal

Gas					5x103 therm

Electricity				995x103 kwh

3a) 	Briefly explain the importance of energy conservation.

b) 	List out the steps required for an energy conservation programme.

4a)	 Explain the disadvantages of low pf in an electrical system from the supplier’s point of view.

b)	 A supply system supplies the following loads : lighting load of 500 Kw, 400 Kw load at 0.707 lag, 800 Kw load at 0.8 lead, 500 Kw load at 0.6 lag. A synchronous motor-dc generator set supplying a dc load of 360 Kw with an efficiency of 90 %. Calculate the pf of the motor – generator set to make the pf of the system unity.

5.a)	Write short notes on (i) control of energy (ii) Language of energy manager.

b)	 A foundry unit in an industrial area is to be energy audited. As an energy manager how would you conduct the Audit and present a report (assume any essential data)

6.	Explain the terms (a) annuity (b) present worth factor. An Industrial plant with initial value of Rs 2,00000/- has salvage value of Rs 20,000/- at the end of 20 years.  But is sold for Rs 1,45,000/- at the end of 10 years. what is the profit or loss if sinking fund depreciation method at 8% compounded annually, is adopted.

Contd..2

Code No: 420203				:2:				Set No.1

7.a)	Define inflation?  Write its importance.

b)	Explain various types of risks.

8.a)	Explain with example the measures to be taken for controlling energy  usage in an industry.

b)	Discuss the functions of Energy manger in promoting Energy Conservation schemes.

&&&&&

Code No.420203

Set No.

2

- IV-B.Tech. II-Semester Supplementary Examinations, July 2003

ENERGY MANAGEMENT

(Electrical and Electronics Engineering)

Time: 3 Hours							   Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks.

---

1.a)	Explain about energy policy of a demand side Organization of Energy Management.

b)	Explain about Energy Management and control systems for demand side.

2.	What are the various techniques used for representation of energy consumption and Discuss in detail with examples.

3.a)	 Explain the medium term energy conservation scheme

b)	 Explain the various steps to be undertaken in order to achieve significant energy conservation in the domestic sector.

4.a)	 Explain how do you achieve efficient utilization of electrical energy for lighting systems.

b)	 A factory is equipped with 3-phase induction motor taking 1000 Kw at 0.85 pf lag. A synchronous motor taking 160 Kw is installed in parallel with the induction motor. At what pf should the synchronous motor work so that the new overall pf is 0.9

5.a)	Discuss the duties of energy Manager.

b)	Explain the role of Energy Manager in a processing industry with example suggest and justify the place of Energy Manager in the same organizational chart.

6.	Explain the terms (a) annuity (b) present worth factor

An Industrial plant with initial value of Rs 2,00000/- has salvage value of Rs 20,000/- at the end of 20 years.  But is sold for Rs 1,45,000/- at the end of 10 years. What is the profit or loss if sinking fund depreciation method at 8% compounded annually, is adopted.

7.a)	Discuss the concept of Inflation.

b)	Explain what do you understand by risk analysis.

8.a)	Explain the role of Energy manager in Monitoring and Reporting an Energy management program.

b)	Explain the importance of Energy manager in organizing and Leading Energy Management Schemes.

&&&&&

Set No.

3

- Code No.420203

- IV-B.Tech. II-Semester Supplementary Examinations, July 2003

ENERGY MANAGEMENT

(Electrical and Electronics Engineering)

Time: 3 Hours							   Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks.

---

1.a)	Discuss about essential data required for supply side Energy Management.

b)	Discuss about essential imperatives and steps in user side Energy Planning.

2.a)	How energy conservation measures are classified on an economic basis.

b)	Write short notes on types of plant energy studies.

3.	Briefly discuss the factors affecting energy saving potential in an energy intensive industry. Explain the same with an example.

4.a)	Explain the effect of low pf on transformers and generators.

b)	A load taking 500 Kw is working at 0.8 pf lag. The pf is improved to 0.9 lag by installing a phase advancing plant working at 0.1 pf lead. Calculate the Kva rating of the advancer.

5.a)	Discuss the role of energy manager in energy management bringing out clearly his functions, duties.

6.	What do you understand by time value of money.  Discuss its significance in Energy Management.	What is the future worth of Rs. 40,000 placed in an investment for 10 years  that earns a yearly rate of return of 8 percent for yearly compounding? What is the worth for daily compounding? Compute the worth for

continuous compounding.

7.a)	Explain the concept of group replacement.

b)	Explain how the theory of replacement is used in the following problems:

(i) Replacement of items whose maintenance cost varies with time.

(ii) Replacement of items that fail completely.

8.a)	Explain the role of Energy manager in Monitoring and Reporting an Energy management program.

b)	Explain the importance of Energy manager in organizing and Leading Energy Management Schemes

&&&&&

Code No.420203

Set No.

4

- IV-B.Tech. II-Semester Supplementary Examinations, July 2003

ENERGY MANAGEMENT

(Electrical and Electronics Engineering)

Time: 3 Hours							   Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks.

---

1.a)	Explain about different sectors of supply side Energy Management.

b)	Discuss about Objectives of supply side Energy Management and Objectives of end-user side Energy Management.

2.	Discuss about Detailed energy Audits.

b)	The consumption and cost of energy used by a company over a monthly period is shown in table,

Monthly Energy Consumption and Cost

Type of Energy			Quantity		Price per unit

Solid fuel (tones)			40			Rs. 29/-

Liquid fuel (gallons)			10000			Rs. 0.33/-

Gaseous fuel (therms)			400			Rs. 0.18/-

Electricity (103 Wh)			90000			Rs. 0.25/-

Find out energy index, the cost index and plot the specific fuel consumption against product output.  If the monthly output of the company were 150 tonnes.  Assume

1 Joule = 947x10-6btu = 238.8x10-6calories = 9.478x10-9therm = 2778x10-6wh

1 Joule = 5.6497 x 10-9gal.

3.a)	 Explain the long term energy conservation scheme

b)	 Explain the various steps to be undertaken in order to achieve significant energy conservation in the commercial sector.

4.a) 	What are the disadvantages of low pf in an electrical system and what is the economical limit to which pf can be improved.

b) 	A 3-phase synchronous motor is connected in parallel with a load of 800 Kw at 0.8 pf lag. The load on the synchronous motor is 150 Kw. Its excitation is adjusted to make the overall pf 0.93 lag. Calculate the Kva input and pf of the synchronous motor. Neglect motor losses.

5.a)	Write short notes on (a) control of energy, language of energy manager.

b)	A foundry unit in an industrial area is to be energy audited. As a energy

manager how would you conduct the Audit and present a report

(assume any essential data)

Contd..2

Code No: 420203				:2:				Set No.4

6.	What are the causes for decrease in worth of productive asset.

An asset has a first cost of Rs 50,000/- depreciated to a salvage value of             Rs 9,600/- in 8 years.  Assuming an interest rate of 8% calculate the total present worth by straight line depreciation and by sinking fund method (any other data missing may be assumed).

7.	Explain the concept of group replacement.

Explain how the theory of replacement is used in the following problems:

- Replacement of items whose maintenance cost varies with time.

- Replacement of items that fail completely.

8.a)	Explain with example the measures to be taken for controlling energy  usage in an industry.

b)	Discuss the functions of Energy manger in promoting Energy Conservation schemes.

&&&&&