Code No. 420255

OR

IV B.Tech. II.Semester Supplementary Examinations July-2003

HVDC TRANSMISSION

(Electrical and Electronics Engineering)

Time:  3hours							Max.Marks:70

Answer any Five questions

All questions carry equal marks.

---

1.	Discuss the relative merits and demerits of AC and DC transmission from the economical, technical and reliability aspects.

2.a)	Give neat sketches of different types of HVDC lines that are generally used and compare their merits and demerits.

b)	Draw a schematic diagram of typical HVDC converter station and describe the various components of the station.

3.	Develop an equivalent circuit of a converter working as rectifier with delay angle ‘α’ and overlap angle ‘µ’ and hence show that the equivalent resistance of the converter is 3ωLΠ where ‘ω ‘ is the angular frequency and ‘L’ is the series inductance of the converter transformer.

4.	Discuss the need for HVDC system control and explain the necessity of constant current and constant voltage control at converter station.

5.	Explain in detail about equidistant pulse control and individual phase control schemes for firing angle control employed in a converter.

6.a)	Explain the necessity of smoothing reactor in a dc line.

b)	Why commutation failure occurs in HVDC converter and explain the phenomenon.

7.	Why filters are necessary at HVDC converter station and discuss the design of some of the filters.

8.	Write short notes on any two of the following:

(a)    HVDC circuit breaker.

(b)    Characteristic harmonics.

(c)    Reactive power requirement at HVDC converter station.

&&&