Code No: 420304.

Set No.

1

IV-B.Tech II Semester Supplementary Examination - July 2003

INTERNAL COMBUSTON ENGINES

(Mechanical Engineering)

Time: 3 hours							Max.Marks:80

Answer any five questions

All questions carry equal marks

---

1.	Explain uni-flow scavenging with a neat sketch and its merits over other types.

2.a)	Would the fuels for spark ignition present the same anti-knock behaviour in 	different engines. If not, give reasoning.

b)	Name the anti-knock additives and their other effects on environment.

3.a)	Explain the working of a simple carburetor.

b)	What are the mixture requirements for (i) Starting and warm up.  (ii)Acceleration

4.a)	Explain the stages of combustion in a CI engine.

b)	Explain air injection system.

5.	A four stroke diesel engine is designed to operate with the following conditions   	at a mean pressure of 1 bar and temperature 150C.

Volumetric Efficiency	 =      80%

Brake horse power 		=       250kW

Specific fuel consumption 	=      0.24 kg/ kWh

Air fuel ratio 			=     18:1

Speed				=     1500 rpm.

Compute the required engine swept volume and the mean effective pressure.

The engine is fitted with a supercharger  so that it may be operated at an 	altitude of 3 km where    the atmospheric pressure is 0.7 bar the power taken by 	the supercharger is 10% of othe total power produced by the engine. And the 	temperature of the air leaving the super- charger is 300C.  The air fuel ratio and 	the thermal efficiency remains the same for the super charged engine as when 	running unsuper charged at sea level, as does the volumetric efficiency. Calculate  	the increase of air pressure required at the supercharger to maintain same power 	output of 250kW

6.a)	What is generalized  performance  map of IC engines?

b) 	Sketch a typical generalized performance map for an engine

7.a) 	Briefly explain how smoke formation takes place during diesel combustion?

b)	What causes blue smoke in CI engine?

8.	How is Cetane number of a fuel determined and how it can be improved in a 	fuel?

~ ~ ~ ~

Code No: 420304.

Set No.

2

IV-B.Tech II Semester Supplementary Examination - July 2003

INTERNAL COMBUSTON ENGINES

(Mechanical Engineering)

Time: 3 hours							Max.Marks:80

Answer any five questions

All questions carry equal marks

---

1.a)	Why do you need ignition in I.C. engines? Discuss the types of ignition employed.

b)	Explain any method of fuel introduction with a neat schematic.

2.a)	What do you mean by primary reference fuels for knock rating of S.I. engine fuels 	and their knock ratings.

b)	What is preignition and how to identify it.

3.a) 	Explain the following.(i)Rich mixture  (ii) Stoichiometric   (iii)Lean mixture

b)	Briefly explain choke and the need for it

4.a) 	How CI engine combustion chambers are classified.

b) 	Draw a schematic diagram of fuel pump and explain its working.

5. 	A turbocharger fitted to   a diesel engine sucks in air at a pressure of 0.9 bar and at 	a temperature of 200C. The delivery pressure is 2 bar. The air fuel ratio is 18:1. 	The temperature of the gases leaving the engine is 6000C. and the pressure is 1.8 	bar. The gases come out of   the turbine at 1 bar. The compressor and turbine 	efficiencies are 0.75 and 0.85 respectively. Calculate the mechanical power lost in 	the turbocharger developed by the turbine. Cp of air and gas are 1.005 and	1.15kJ/kgK respectively and the ratio specific heats is 1.4

6.a) 	What are the advantages of using lean mixtures in SI engines?

b) 	Describe briefly the   methods of charge stratification

7.a) 	What are the different methods used for smoke measurement?

b)	Explain briefly the operation of the Hartridge smoke meter?

8.	What is warm up period of an engine? Why and how inlet manifold heating is 	used in some SI engines?

~ ~ ~ ~

Code No: 420304.

Set No.

3

IV-B.Tech II Semester Supplementary Examination - July 2003

INTERNAL COMBUSTON ENGINES

(Mechanical Engineering)

Time: 3 hours							Max.Marks:80

Answer any five questions

All questions carry equal marks

---

1.a)	Explain the working of a crank case scavenged two-stroke I.C. engine with the aid 	of a sketch.

b)	What do you understand by symmetric scavenging?

2.	What criteria is to be fulfilled by combustion chamber design in S.I. engines to 	avoid detonation.  Explain in detail.

3.	What are the main devices of main metering system ? Explain any	three of them.

4. 	Compare the open combustion chamber with divided combustion chamber giving  	merits and demerits of each.

5. 	A six cylinder diesel engine produces 150 kW when running at 200 rpm. The mechanical efficiency is 80 % the engine is supercharged by a turbocharger having a pressure ratio of 2. The adiabatic efficiency of the turbocharger is 75%.  The indicated mean effective pressure is proportional to the inlet density of the   charge and the mechanical efficiency of the engine does not change due to supercharging. An after cooler of effectiveness 0.7 is used to cool the air before it enters the engine. Calculate the power of the supercharged engine with and without cooler.  Take inlet conditions as 1 bar and 310 K

6.a) 	Explain the principle  involved in the measurement  of  brake  horse power

b) 	Discuss briefly the various   methods for measuring the brake   horse power

7.	Indicate the methods tried to reduce diesel pollution?

8.	Indicate briefly the suitability of alcohols in SI engines?

~ ~ ~ ~

Code No: 420304.

Set No.

4

IV-B.Tech II Semester Supplementary Examination - July 2003

INTERNAL COMBUSTON ENGINES

(Mechanical Engineering)

Time: 3 hours							Max.Marks:80

Answer any five questions

All questions carry equal marks

---

1.a)	Explain the four stroke and two stroke cycle engine concepts.

b)	Discuss the relative merits and demerits of these engines.

2	Discuss the phenomenon of detonation and the role of time and density factors on 	it.

3.a)	Sketch a typical induction system of a petrol engine.

b)	Explain idling system

4. 	Explain with figures various types of combustion chambers used in CI engines.

5.  	An un supercharged SI engine develops 75 kW with fuel air ratio   equal to 0.70. 	The fuel consumption per brake horse power is 0.35 kg/kWh and the mechanical 	efficiency is 0.87. The inlet pressure and temperature are 0.95 bar and 320 K. The 	engine is supercharged to a pressure ratio of 1.8 by a blower with an adiabatic 	efficiency of 0.75. Assuming that the inlet conditions remain the same and the 	indicated power is proportional to the inlet density, calculate the brake horse 	power. The specific fuel consumption and the mechanical efficiency   of the 	supercharged engine

6.a) 	Briefly  discuss the various efficiency terms associated with  an engine.

b) 	Define air- fuel ratio and discuss   its effect on power output, fuel consumption 	and combustion pressure

7.	What causes oxides of nitrogen during combustion in a diesel engine?

8.	By what methods alcohol can be used as sole fuel in a diesel engine?

~ ~ ~ ~