Code No: 420305

Set No.

1

IV-B.Tech. II-Semester Examinations July 2003

NON-CONVENTIONAL SOURCES OF ENERGY

(Mechanical Engineering)

Time: 3 hours						                    Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a) 	Calculate daylength at location ( latitude 280 35’N, longitude 770 12’ E) on December 1.

b) 	Assuming the earth’ solar constant to be 4871 KJ/Cm2 hr, calculate the equivalent surface temperature of the sun, if the sun is assumed to be black body radiator.

2.	Write short notes on:

- Selecting absorber coatings.

- Materials for flat plate collectors.

- Evacuated solar collectors.

3.a) 	List out the various solar applications.

b) 	Discuss in detail about any three of the solar applications.

4.a) 	How do you measure the speed and the direction of a wind? Explain in detail.

b) 	What are the various characteristics of the wind?  Discuss them in detail.

5.a)  	Explain the classification of biomass resources.

b)  	How the biomass conversion takes place?

What is the difference between biogas and biomass?

6.a)	What is geothermal energy?  Explain.

b)	Give the classification of different geothermal sources in detail.

7.a)	Explain the working of an OTEC system with a line diagram.

b)	Show by sketches the method of harnessing the energy potential associated with ocean tides.

8.a)	What is thermoelectric generator?

b)	On what effects does thermoelectric generator function depend?

c)	Explain the working of thermoelectric generator with a sketch.

***     ***     ***

Code No: 420305

IV-B.Tech. II-Semester Examinations July 2003

Set No.

2

NON-CONVENTIONAL SOURCES OF ENERGY

(Mechanical Engineering)

Time: 3 hours						                    Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a) 	Calculate the day length on May 1, for a south facing surface located at New Delhi (280 38’N, 770 17’E) and tilted at an angle of 400

- Determine the atmospheric tranmittance for scattering only for  = 0.5m and pressure 750 mm Hg., d= 800 Cm2 and  = 20 mm, when the sun is at zenith.

2. 	Describe with neat sketches, the procedure for thermal performance testing of a cylindrical parabolic collector.

3. 	Write short note on the following:

(a) Sensible heat storage	(b) Latent heat storage

(c) Electrical storage		(d) Photo voltaic cell.

4. 	How do you get power from the wind mill? Derive an expression for the power.

5.	What are different biomass conversion technologies? Write about them in detail.

6.a)	What are liquid dominated hydrothermal convective systems?  Write about them.

b)	With the help of a neat diagram, explain the working of a liquid dominated double flash steam system.

7.a)	With a schematic diagram, explain briefly the working of open cycle OTEC plant.

b)	With reference to typical examples, explain the nature and magnitude of energy possessed by ocean tides.

8.a) 	Discuss the direct and indirect energy conversion systems emphasizing on the advantages and limitations of each.

b)	How is the operation of thermoelectric generator different from that of conventional generators?

***     ***     ***

Code No: 420305

IV-B.Tech. II-Semester Examinations July 2003

Set No.

3

NON-CONVENTIONAL SOURCES OF ENERGY

(Mechanical Engineering)

Time: 3 hours						                    Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Determine the solar time and declination at Ahamedabad ( longitude 720 40’ E, latitude 230 0’N) corresponding to 14:30 IST on December 15.

b)	Determine the sunset hour angle and daylength for Allahabad (longitude 810 58’E, latitude 240 25’N) for the following dates: January 1, March 22, July 15.

2. 	Write short notes on:

- Solar radiation measurements.

- Temperature measurements.

c)	Liquid flow measurements.

3.a)	List out the advantages of photo-voltaic solar energy conversion.

b) 	How can solar energy can be stored in the form of thermal energy? Explain and discuss in brief.

4.a) 	List out the difficulties encountered in general in operating large wind power generators.

b) 	Discuss in detail with a neat sketch about the working of a wind mill.

5.a)  	What is biogas? How is it produced?

b) 	Explain in detail about anaerobic digestion and the different phases and the processes involved in it?

6.a)	What are the advantages and disadvantages of geo thermal energy over other energy forms?

b)	Discuss the applications of geo thermal energy.

7.a)	Explain with neat sketches the various methods of tidal power generation.  What are the limitations of each method?

b)	What are the advantages, limitations and operational difficulties encountered in OTEC plants?

8.a)	List the various direct energy conversion devices.

b)	With a neat sketch, explain the working of thermo-electric generator.

***     ***     ***

Code No: 420305

Set No.

4

IV-B.Tech. II-Semester Examinations July 2003

NON-CONVENTIONAL SOURCES OF ENERGY

(Mechanical Engineering)

Time: 3 hours						                    Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1. 	Calculate the angle made by beam radiation with the normal to a flat plat collector, pointing due south location in New Delhi (28o  38’N, 77o  17’E) at 9:00 hr, solar time on December 1. The collector is tilted at an angle of 36o  with the horizontal. Also calculate the day-length.

2.	Explain the design procedure for a solar based forced convection type drier.

3.a) 	Discuss in detail about the mechanism of salt-gradient solar pond, with the aid of neat sketches.

b) 	Discuss the following:

(i)  Packed bed storage system.

(ii) Photo-voltaic cell.

4.	Write short note on the following:

- Performance of a wind mill.

- Savonius rotor wind mill.

- Cup anemometer.

- Darrius rotor.

5.  	Explain in detail about the factors which affect the bio-digestion.

6.a)	Write about the concept of interconnecting geo thermal-fossil systems.

b)	With the help of neat diagram, explain the working of geo thermal-preheat hybrid system.

7.a)	Explain with a neat sketch the energy extraction techniques from tidal waves

b)	The efficiency of power plant working on OTEC system is very less.  However, the secondary advantages make it commercially attractive.  Discuss.

8.a)	What is the basic principle of Direct Energy Conversion systems?

b)	Describe briefly the working of a thermoelectric generator.

c)	Explain Seebeck, Peltier, Joule and Thomson effects.

***     ***     ***