Code No: 420803

Set No.

1

IV-B.Tech. II Semester Supplementary Examinations July, 2003

COMPUTER APPLICATIONS IN CHEMICAL ENGINEERING

(Chemical Engineering)

Time: 3 hours						                  Max.Marks: 80

Answer any five questions

All questions carry equal marks

- - -

- Given that dy/dx = x + y2 and y = 1 at x = 0. Find an approximate value of y at         x = 0.5 by modified Euler’s method.

- Solve by Cramer’s rule, the equations:

3x1+x2-x3 = 2,	x1+2x2+x3 = 3,    -x1+x2+4x3 = 9.

3.	Solve by Gauss elimination method:

x + y + z  = 6.6;    x - y + z  = 2.2;      x + 2y + 3z  = 15.2

4.a)	 Find the roots of the following polynomial equation using the Newton-Raphson 	method.

X4 – 8X3 + 14.91X2 + 9.54X – 25.92 = 0

b) 	What are the bounds within which the roots lie?

5. 	Calculate the molar volume of methanol vapor at 400o K and 8 bar by using Redlich-Kwong equation of state V=[RT/P  + b – a(V – b)/{T0.5.PV(V + b)}]

Where a=0.4278R2 Tc2.5/Pc;  b= 0.0867RTc/Pc ; Tc=512.6 K; Pc =81 bar. Use the regular falsi method.

6.	 The steady state substrate and biomass concentration for continuous CSTR operated at various dilution rates are given below. The fresh feed concentration is 675mg/l, calculate the value of the monod constants m and ks by fitting the data using least square regression technique in the equation of the form: X/D(S0 – S) = ks.Y/( m.S) +Y/m

Data: Yield factor(Y)=0.5

| Dilution rate(D) h-1 | 0.29 | 0.24 | 0.21 | 0.115 | 0.075 |
| --- | --- | --- | --- | --- | --- |
| Substrate conc.(S) mg/l | 44 | 41.5 | 15.8 | 8.2 | 3.92 |
| Biomass conc.(X) mg/l | 324 | 332 | 339 | 343 | 345 |

7.	Find the optimum and locate the nature of the stationary point of the following function where a is a constant.

f(x)=axe−1/x2

8.  (a)  How many experiments are needed for preplanned regular intervals method  to reduce the final interval in which the optimum will lie with in 1 % and 0.1 % of the original region ?

- Find the effectiveness of preplanned regular interval method and sequential two point regular interval method when the number of experiments is 8.

-x-

Code No:420803

IV-B.Tech. II Semester Supplementary Examinations July, 2003

Set No.

2

COMPUTER APPLICATIONS IN CHEMICAL ENGINEERING

(Chemical Engineering)

Time: 3 hours						                  Max.Marks: 80

Answer any five questions

All questions carry equal marks

- - -

- Given that dy/dx = 2+ √(xy) and y = 1, when x =1. Find approximate value of y at x = 2 in steps of 0.2 using Euler’s modified method.

- Solve by Cramer’s rule, the equations:

x1+2x2+3x3 = 14,   3x1+x2+2x3 = 11,   2x1+3x2+x3 = 11.

3.	 Solve the following equation by matrix inversion method

x + y + 2z  = 4

2x - y + 3z  = 9

3x - y - z  = 2

4.a) 	Find the roots of the equation

X3 - 4X + 1 = 0

Correct to 3 significant digits using iterative method.

b) 	When does one stop the iterative cycle?

5. 	Calculate the specific volume of superheated steam at 10Mpa a and 623 K(350C) using the Vander waals equation: (p + a/v2)(v – b)=RT. Use the Iterative method.

Data: a=0.5531  (m3)2.Mpa/(kmol)2 ; b=0.0304 m3/kmol

6. 	 A zero order liquid phase reaction AR is conducted in a constant volume batch reactor and the following data were reported. Fit the data in the zero order rate equation using least square regression technique and find the rate constant(k).

Data: Initial reactant concentration CA0=2gmol/lit, -rA=-dCA/dt=k

| Time(t),min | 0 | 0.25 | 0.5 | 0.75 | 1.0 | 1.25 | 1.50 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Conversion(x) | 0 | 0.11 | 0.19 | 0.31 | 0.39 | 0.51 | 0.60 |

7.           Find the minimum and range of optimum for the function

y = (2x-9)2

using direct search with appropriate fixed step size.

8.  (a)  How many experiments are needed for Fibonacci method  to reduce the final region in which the optimum will lie with in  0.1 % and  0.01 % of the original region.

(b) Find the effectiveness of  Fibinacci method and modified Fibonacci method when the number of experiments is 10.

-x-

Code No:420803

IV-B.Tech. II Semester Supplementary Examinations July, 2003

Set No.

3

COMPUTER APPLICATIONS IN CHEMICAL ENGINEERING

(Chemical Engineering)

Time: 3 hours						                  Max.Marks: 80

Answer any five questions

All questions carry equal marks

- - -

- Given dy/dx = (y-x)/(y+x) with initial condition y = 1 at x = 0; find y for x =  0.1 by Euler’s method.

- Solve by Cramer’s rule, the equations:

2x 1 – 3x2 + x3 = 9,	x1+x2+x3 = 6,	  x1 – x2+x3 = 2.

3.	Solve the following equation by matrix inversion method

x + 3y + 6z  = 2

3x - y + 4z  = 9

x - 4y + 2z  = 7

4.a)	 Find a real root of the equation

X3 – 2X – 5 = 0

by  the Regula-falsi method correct to three decimal places.

b) 	How does one choose the initial value of the root .

5. 	The flux equation of an iron core electric circuit is given by f()= 10 – 2.1 -0.013. The steady state value of flux is obtained by solving the equation f()=0. Use iterative method to estimate the steady state .

6.	Find the overall order of the irreversible reaction 2H2 +2NO N2+2H2O from the following constant volume data using equimolar amounts of hydrogen and nitric acid. Use the least square regression technique.

Data:

| Total pressure (0),mm Hg | 200 | 240 | 280 | 320 | 360 |
| --- | --- | --- | --- | --- | --- |
| Half-life( t1/2),sec | 265 | 186 | 115 | 104 | 67 |

7.         Find the minimum  of  the function  y = (2x-9)2 using direct search with fixed step size of one unit. What is the range of optimum ?

8.a)  	Determine the number of experiments required to locate the final region with in    1 % of the original search region by Fibonacci and preplanned regular intervals search methods.

- Find the effectiveness of  Fibinacci and preplanned regular intervals methods when the number of experiments is 6.

-x-

Code No:420803

IV-B.Tech. II Semester Supplementary Examinations July, 2003

Set No.

4

COMPUTER APPLICATIONS IN CHEMICAL ENGINEERING

(Chemical Engineering)

Time: 3 hours						                  Max.Marks: 80

Answer any five questions

All questions carry equal marks

- - -

- Given dy/dx = (y-x)/(y+x) with boundary conditions y = 1 when x = 0, find approximately y for x = 0.1, by Euler’s modified method.(5Steps).

- Solve by Cramer’s rule, the equations:

x1–x2+x3 –x4+1= 0,	3x1–2x2+3x4+2 = 0,    5x1–4x2+x4+3 = 0

3.	 Solve the following equation by matrix inversion method

2x + 3y + 2  = 9 ,   x + 2y + 3z  = 6  , x + y + 2z  = 8

4.a)	Find the real root of the equation

x – x3/3 + x5 /10 – x7/42 + x9/216 – x11/1320 + ……. = 0.443

correct to three decimal places using iteration method.

b)  	Write an algorithm for the same problem.

5.	The supply and demand functions of a petrochemical product are Qs= P3 –500,

Qd =P2 –60P+1500 determine the market equilibrium which occurs when Qs=Qd. Use Newton Raphson method.

6.	 A zero order gas phase reaction 2AR is carried out in a batch reactor. The following data were reported .Fit the data using least square regression technique and find the rate constant (k).

Data: Initial reactant concentration CA0=1.5gmol/lit

| For zero order reaction with variable volume –rA= -(1/V).dNA/dt =k, Assume volume changes linearly with conversion. Time(t), min | 0 | 0.25 | 0.5 | 0.75 | 1.0 | 1.25 |
| --- | --- | --- | --- | --- | --- | --- |
| Conversion(x) | 0 | 0.14 | 0.3 | 0.49 | 0.7 | 0.95 |

7.          Find the minimum of the function

y = (2x-9)2

adopting direct search with accelerating step size.

8.a) 	Find the effectiveness of  sequential Dichotomous search and preplanned dichotomous search methods when the number of experiments is 20.

b) 	Compare the number of experiments required to reduce the final region in which the optimum will lie with in  0.01 % of the original search region when sequential Dichotomous search and preplanned dichotomous methods are employed.

-x-