Code No: 420201

Set No.

1

IV-B.Tech II Semester Supplementary Examination - July 2003

INSTRUMENTATION

(Electrical and Electronics Engineering)

Time: 3 hours							Max.Marks:80

Answer any five questions

All questions carry equal marks

---

1.a)	Explain the advantages and disadvantages of electrical transducers over 	mechanical transducers

b)	What are digital transducers? Give examples

2.a)	What is meant by tuned filter?

b)	Draw the circuit diagram of second order active low pass filter and derive the 	expression for the gain of the circuit

3. 	Describe with the help of suitable circuit diagrams, how the following types of 	measurements are carried out using a digital frequency meter

(i)	Single and multiple period measurements

(ii)	Time interval measurements

- Multiple ratio measurements

4.a)	Describe the engineering applications of wave analysers

  - Explain the different types distortions caused by amplifiers.

5.a)	What is the gauge factor and derive the formula.

b)	 Explain how strain gauge is used for torque measurements

6.	 Draw the equivalent circuit of piezo electric transducer. Explain? the effect of

loading on piezo electric transducer?

7.a)	 What is a full bridge. Derive an expression for gauge sensitivity of a full bridge.

b)	A bridge circuit has two fixed resistances and two strain gauges all of which have 	a value of 1200. The gauge factor is 2.04 and the strain applied to twin strain 	gauges one in tension and the other in compression is 0.000165. if the battery 	current in the initial balanced condition of the bridge is 50mA determine.

- voltage o/p of the bridge

- The sensitivity in volt per unit strain

8.a)	How drag cup ac tachometer generators are used for the measurement of angular

velocity.

b)	Give the limitations of ac tachometer generator.

~ ~ ~ ~

Code No: 420201.

Set No.

2

IV-B.Tech II Semester Supplementary Examination - July 2003

INSTRUMENTATION

(Electrical and Electronics Engineering)

Time: 3 hours							Max.Marks:80

Answer any five questions

All questions carry equal marks

---

1.a)	Distinguish between deterministic signals and random signals. Give suitable 	examples.

b)	 Describe the procedure used to determine whether the sum of two periodic 	signals is periodic or not

2.a)	Draw the circuit diagram of second order active high pass filter and derive the 	expression for the gain of the circuit

b)	Describe the Band pass passive RC filters

3.a)	Explain the following features of an analog type storage oscilloscope

- bistable persistence storage. (ii)  bistable storage   (iii)  fast storage

b)	Discuss the advantages and disadvantages of analog and digital type of 	oscilloscopes.

4.	Describe the basic circuit of a spectrum analyzer. Explain how the spectra of the 	following are displayed

(i)	continuous wave signals.	(ii)	amplitude modulated signals.

(iii)	Frequency modulated signal.

5.a)	What are the various types of inductive transducers?

b) 	Explain in detail the working of inductive transducers operating on the          	principle of eddy currents?

6.	What is the frequency response of piezo electric transducer?

7.a) 	Discuss the principle of operation of strain gauge. What is gauge factor? Compare 	some of the important characteristics of metallic and semiconductor type strain 	gauges.

b) 	A resistive strain gauge with a gauge factor of 2 is fastened to a member which is 	subjected to a strain of 1x10-6. If the original value of gauges is 130 ohms, 	calculate change in resistance.

8.a)	Compare the advantages and disadvantages of dc tachometer generation and ac 	tachometer generator.

b)	A variable reluctance type tachometer has 60 rotor teeth. The counter records 	3600 counts per second. Determine the speed in rpm.

~ ~ ~ ~

Code No: 420201.

Set No.

3

IV-B.Tech II Semester Supplementary Examination - July 2003

INSTRUMENTATION

(Electrical and Electronics Engineering)

Time: 3 hours							Max.Marks:80

Answer any five questions

All questions carry equal marks

---

1.a)	Briefly explain the following processes as applied to Pulse Code modulation

- Quantization process

ii)	Encoding process

b)	Describe the following modulation processes:

i)	Pulse amplitude modulation

- Pulse frequency modulation

2.a)	A second order system has a natural frequency of 10Hz and damping ratio of 0.3  	Calculate the value of resonant frequency and bandwidth

b)	What is meant by Zero order system? Give examples

3.	Describe in detail the construction and working of an analog type storage 	oscilloscope. Explain the principle of secondary emission.

4. 	Describe the circuit and working of a Q– meter. Describe its applications.

5.a)	 Discuss in detail about the principle of operation of a capacitive transducer?

b)	What is the relation between sensitivity and area of plates?

6.	Explain in detail the applications of photo devices?

7.a) 	Discuss with a suitable processing circuit how efficient load cells are 	constructed using metallic strain gauge.

b) 	A strain gauge of nominal resistance 200 is fixed on one flat surface of a short 	column of 2cm x 2cm cross sectional area. The column is subjected to an axial 	force of 100N. the strain gauge forms one arm of a bridge with other arms all 	equal to 200. Find open circuit voltage of the bridge excited by 10v. Given 	young’s modulus of elasticity is 2.1x 1011N/m2.

8.a)	Give the advantage of digital methods in measuring angular velocity over 	electromechanical methods.

b)	Explain the measurement of angular velocity using photo electric tachometer. 	Also give advantage and disadvantage of the method.

~ ~ ~ ~

Code No: 420201.

Set No.

4

IV-B.Tech II Semester Supplementary Examination - July 2003

INSTRUMENTATION

(Electrical and Electronics Engineering)

Time: 3 hours							Max.Marks:80

Answer any five questions

All questions carry equal marks

---

1.	Explain the following with suitable examples:

i)	Transducer

- Inverse transducer

- Output transducer

2.a)	Derive the expression for the time response of second order system subjected to 	ramp input

b)	Derive the frequency and phase responses of harmonically excited first order 	systems

3.	Describe the principle of working and circuit diagram of a digital oscilloscope.

4.	Describe the circuits and working of wave analysers used for audio frequency and 	megahertz ranges.

5.a)	A single strain gauge is mounted to measure the axial strain in a simple tensile

member. If the recorded strain is 380 micro strain what is the axial stress

i)if the member is steel?

ii)if the member is aluminium?

b) 	What are the advantages of electrical transducers?

6.	 Explain in detail about piezo electric transducer

7.a) 	What is a load cell. Explain the working of a load cell strain gauge bridge.

b) 	A load cell consists of a solid cylinder of steel 40mm in diameter with four 	strain gauges boned to it and connected in to the four arms of a voltage 	sensitive bridge. The gauges are mounted to have Poisson’s arrangement. 	The 	gauge factor is 2.1 and each gauge has 1000 ohms resistance. The bridge 	excitation voltage is 6v. Determine the sensitivity of the cell in V/KN 	modulus of elasticity for steel is 200GN/m2 and Poisson ratio is 0.29.

8.a)	Explain the principle of hot wire anemometer for the flow measurement.

b)	Explain the flow-direction measurement using hot wire anemometer. Give a neat 	sketch.

~ ~ ~ ~