Code No.420302

Set No.

1

- IV-B.Tech. II-Semester Supplementary Examinations, July 2003

Environment and pollution control

(Common to Mechanical Engineering and Production Engineering)

Time: 3 Hours							   Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks.

---

1.a)	Mention the difference between thermal NO, prompt NO, and fuel NO.  Explain with the help of chart of Temperature vs NO the effect of temperature on the three types.

- Explain the effect of soot and particle emissions during the combustion of liquid fuels?

2.a)	Name the global effects of air pollution and state pollutant responsible for those effects.

- b) 	Write short notes on the effects of particulate maters on plants.

3.a)	Explain  thermodynamics of combustion

- b)	Explain Air-Fuel ratio.  Find the air-fuel ratio if (i) 90% & (ii) 120% mixture strengths are used for the combustion of Ethyl alcohol, C2H6O in a motor vehicle.  Also find exhaust Gas analysis.

4.a)	Write about dispersion of air pollutants.

- b)	Explain about atmospheric stability.

- 5.	Discuss the various types of atmospheric stabilities with reference to adiabatic 	lapse rate. Also discuss which type of stability is more favourable for dilution of 	pollutants.

- 6.	What are the principal gases of concern in air Pollution control? What are the four 	primary types of treatment processes available for control of gases.

7.a)	 Describe with a neat sketch the principle and working  of a centrifugal separator.

b)	 Write a short notes on i) Packed towers ii) Condensation of gaseous pollutants.

8.a)	Explain the criteria for selection of  particulate  control equipments.

b)	Describe with a neat sketch the working of electrostatic precipitator.

^^^

- Code No.420302

IV-B.Tech. II-Semester Supplementary Examinations, July 2003

Set No.

2

Environment and pollution control

(Common to Mechanical Engineering and Production Engineering)

Time: 3 Hours							   Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks.

---

1.a)	Explain the reason for the changes in trace components present in the atmosphere?

b)	Differentiate between primary and secondary air pollutants with examples.

2.a)	Explain the effects of air pollution  on  human being.

b)      Write short notes on effect of radioactive pollution on human health.

3.	Describe the method which is used to control hydrocarbons (HC) vapors from fuel tank & carburetor with the help of a diagram.

4. 	Discuss briefly the following:

a)	Mixing height & their importance in air pollutant dispersion.

b) 	How moisture & relative humidity influence the air quality.

5.a)	Name and define the two types of thermal Inversions. Which type prompts 	the formation of fog?

b)	 Give the Effect of Inversion on Plume behaviour. Discuss the different Plume 	behaviour in a given environment.

6.a)	Define adsorption and differentiate between physical and chemical adsorption.
   b)	Name and describe three types of adsorbers.

7.a)	What specific air pollution control devices are available for control of particulate emission  at their source? Indicate the  range of the particulate in the each type of device is capable of removing efficiently.

8.	Discuss the following:

a)	Ambient air quality

b)	Emission standards

c)	Pollution due to automobiles.

^^^

Code No.420302

IV-B.Tech. II-Semester Supplementary Examinations, July 2003

Set No.

3

Environment and pollution control

(Common to Mechanical Engineering and Production Engineering)

Time: 3 Hours							      Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks.

---

1.a)	Mention certain examples of effects of forest damage because of secondary air pollutants?

b) 	Explain the mechanism of the effect of soil acidification in the case of acid rain?

2.	Explain the symptoms and effects of the following pollutants on the health of animals

- Fluoride		(b)    arsenic		(c)    lead.

3.	The products of combustion of a fuel of unknown composition have the following composition on a dry air basis:

8% CO2, 0.9 % CO, 8.8 % O2 and 82.3% N2.  Calculate the air fuel ratio, fuel composition on mass basis and the percent theoretical air employed.

4.a)	Define & briefly discuss about the wind rose with the help of a diagram &        significance of wind rose in air pollution studies.

b)	What are the reasons for wind velocity and turbulence in the earth’s atmosphere?

5.a)	Discuss the advantages and limitations of Gaussion model.
   b)	It is estimated that a burning dump emits 5 g/s of NOx. What is the 	concentration 	of NOx directly down wind from this source at distance of 0.5 km 		on an overcast night (sy=30m and sx =15m) with a wind speed of 8 m/s? 	Assume the dump to be a ground level point source with no plume rise. 	Assume any other data required.

6.a)	Explain the reduction of sulphur in combustion process with schematic diagram.

7.a)	Explain working of settling chamber for particulate removal.

b)	Discuss the natural removal mechanism that work in the atmosphere.

c)	A settling chamber of size is 12m, 2m , 2m is processing 240 m3/hr of air at temp of 75o C.  Determine the maximum size of particle with specific gravity of 1.8 that is removed with a theoretical efficiency of 100 %.

8.a)	An electrostatic precipitator has to be designed to process 5 m3 /s of stack gas. The drift velocity of dust  particles  is   w = 1.5 x 10dp   m/s. Determine  the  plate area  required  to remove particles with dia of 0.7  m  with  95% efficiency.

^^^

- Code No.420302

IV-B.Tech. II-Semester Supplementary Examinations, July 2003

Set No.

4

Environment and pollution control

(Common to Mechanical Engineering and Production Engineering)

Time: 3 Hours							   Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks.

---

1.a)	The maximum one-hour carbon monoxide levels in Calcutta reach 35ppm.  Calculate the equivalent concentration in terms of mass fraction (wp), and in milligrams per cubic meter at 250C and 1 atm .

- b)	Explain the effect of photochemical smog on materials?

2.a)	Name the air pollutants, which have impact on vegetation.

- b)	List the possible damages of air pollution on leaf  and explain them.

3.a)	Explain the thermodynamics involved in the combustion of oil.

- b)	How the thermodynamics can be applied  for the control of NO2 in combustion process.

4.a)	Explain  various meteorological factors that effect the dispersion of air pollutants in atmosphere.

- b)	What do you understand by the term inversion.  Explain the cause & effects of inversion.

- 5.	Explain the Maximum Mixing Depth (MMD). What is the concept of MMD 	for stability conditions in Environment? Define mean Maximum Mixing Depth 	(MMD). How is MMD obtained?

- 6.	Explain the following equipment with neat sketches
   a)    	Multiple fixed - bed adsorber
   b)   	Single – bed adsorber
   c)   	Fluidized – bed adsorber.

7.a)  	Explain the working principle of a filter bag with a  neat sketch.  Also   explain the cleaning process of the filter bags.

b) 	Compare cyclone separators and filter bags.

c)	Under what industrial  conditions, the use of a fabric filter will be appropriate.

- 8.	Outline a programme for the air quality monitoring of a industrial city.   What are the problems and limitations of sampling and measurement?

- ^^^