Code No: 421202

Set No.

1

IV-B.Tech. II-Semester Supplementary Examinations July, 2003

TELEMATICS

(Computer Science and Information Technology)

Time: 3 hours						               Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Describe the basic functions performed by a switching office.

b)	Compare TSI and space switching.

c)	Describe switching matrix.

2.a)	Describe the various video compression techniques and compare them.

b)	Explain the data compression technique.

c)	A compact disc holds 650 MB data.  Is compression used for audio CDs? Give reasoning.

3.a)	With a neat schematics explain the connectionless internet.  What are the problems associated with it?

b)	What is a network?  How the networks differ from each other?  List item wise and some possibilities.

4.a)	Develop a state diagram (finite state machine) representation of pseudoternary coding.

b)	Discuss multiframe structure for 1.544 Mbps interface.

5.a)	Distinguish between Circuit-mode Bearer services and packet-mode Bearer services. Describe the various circuit mode Bearer Services.

b)	Compare the T A standards such as

i) 1.463/v.110 and 1.461/x.30		ii) 1.465/v.120	iii) 1.462/x.31

6.a)	Show that the blocking probability of a TST network is PB=T−1!2N!2T−2−N!PN2−P2T−2−NWhere P is the circuit occupancy in Erlangs.  If T stages contained 100 time slots and S stages contained 100 slots calculate the blocking probability if P=0.5 Erlangs.

b)	Describe the basic concept of echo suppressor and explain the functioning of a balanced echo suppressor.

7.a)	Discuss the features of Internet Message Access Protocol (IMAP) and compare it with Post Office Protocol version 3 (POP3).

b)	Describe internetworking using concatenated virtual circuits.

8.	Write notes on:

a) Primary rate user-network interface.

b) ISDN transmission structure.

c) Link coding techniques.

@@@@@

Code No: 421202

Set No.

2

IV-B.Tech. II-Semester Supplementary Examinations July, 2003

TELEMATICS

(Computer Science and Information Technology)

Time: 3 hours						               Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Explain the hierarchy of switching centers showing possible inter connections.

b)	Explain why it is necessary that, in the absence of the voice generated signal current, the diaphragm of a telephone receiver must be displaced from its unstressed position either by a quiescent current or by the presence of a permanent magnet.

2.a)	Discuss the different audio compression techniques.

b)	Explain the Simple Mail Transfer Protocol (SMPT) with illustrations.

3.a)	Discuss the fire walling property of data networks.

b)	Tunneling through a concatenated virtual-circuit subnet is straight forward:  The multi protocol router at one end just sets up a virtual circuit to the other end and passes packets through it.  Can tunneling also be used in datagram subnets?  If so, how?

4.a)	Discuss the various objectives of ISDN and its benefits.

b)	What an ISDN physical layer?  Describe the various functions of physical layer.

5.a)	Draw the frame format of 1.465/V.120 frame structure for circuit-mode bearer services and explain.

b)	X.25 and most other layer 3 protocols provide techniques for flow control and error control.  Why such features are not provided in 2.931?

c)	List the 2.931 messages used for user to user signaling stating their significance/directions and functions.

6.a)	Draw a schematic of a central exchange system and explain its operation.

b)	Discuss the operating modes of a strowger switch.

7.a)	Explain continuous multimedia and streaming multimedia.  Describe the processes of sending e-mail, and reading e-mail.

b)	Describe a way to reassemble IP fragments at destination.

c)	Differentiate between transparent fragmentation and non-transparent fragmentation.

8.	Write notes on:

a) ISDN physical layer

b) ISDN protocols

c) Tele services.

@@@@@

Code No: 421202

Set No.

3

IV-B.Tech. II-Semester Supplementary Examinations July, 2003

TELEMATICS

(Computer Science and Information Technology)

Time: 3 hours						               Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	With a neat diagram illustrate the principle of cross-bar switch.

b)	Draw a 3-stage switching structure and explain its operation.

Assume that there are N=8 inputs and 8 outputs in the 3-stage switching system.  Assume that there are 4 first stage and 4 third stage switches so that n=2.

i)  Verify by means of sketch, that if K3 there is no blocking.

ii) Verify by means of sketch, that if K=2 there is a blocking.

2.a)	Discuss the basic functions of e-mail systems.

b)	Describe the various types of e-mail services.

c)	What is the bit rate for transmitting uncompressed 800x600 pixel colour frames with 8 bit/pixel at 40 frames/sec?

3.a)	What do you understand by the term fragmentation? What are its advantages and disadvantages?

b)	Describe the various functions of internetworking developed by IEEE 802.1.

4.a)	Explain the principles of ISDN and its evolution.

b)	With a schematics describe the possible configurations for ISDN user network interfaces.

5.a)	What is the function of a Terminal Adaptor?  Describe the various types of terminal adaptors.

b)	Distinguish between circuit-mode bearer services and teleservices.  What are the different types of ISDN teleservices?  Describe them briefly.

6.a)	If there are 100 possible users of a telephone system and no more than 10 will be using the system at any one time.  Calculate the probability of blocking.  Assume that each call lasts T-sec, each initiated call is independent of the others and the average number of calls initiated per second is .

b)	Discuss the two stage matrice system to provide multipath.  What are its advantages and disadvantages.

7.a)	Distinguish between scanning and interlaced scanning, sketch the scanning pattern used for NTSC video and Television and explain.

b)	Describe the tunneling technique and inter network routing with necessary.  Illustrations.

8.	Write notes on:

a) U-interface		b) ISDN Connections		c) LAPD.

@@@@@

Code No: 421202

Set No.

4

IV-B.Tech. II-Semester Supplementary Examinations July, 2003

TELEMATICS

(Computer Science and Information Technology)

Time: 3 hours						               Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	With a neat diagram explain the stronger automatic dialing system.

b)	With a neat diagram illustrate the principle of Time-Slot- Inter charging (TSI).

2.a)	Describe multicast backbone-M Bone.

b)	In any standards such as RFC 822, a precise grammar of what is allowed is needed so that implementations can inter work even simple items have to be defined carefully.  The SMTP header allow white space between the tokens.  Give two plausible alternative definitions of white space between tokens.

3.a)	Describe networking using concatenated virtual circuits.

b)	How the networks can be connected?

c)	Give 3 examples of protocol parameters that might be negotiated when a connection is setup.

4.a)	Draw a structure of the ISDN address and explain the various address elements.

b)	Discuss the ISDN standards and their importance.

c)	List the CCITT  ISDN standards.

5.a)	What is rate adoption?  Draw the schematics of 1.460 rate adoption to 64-kbps B-channel and discuss.

b)	Describe the methods by which supplementary services may be controlled as identified by 2.932.

6.a)	Discuss the basic elements of a phone system with neat illustrations wherever required.

b)	Prove that dNx/dn = 0 in Nx = 2N(2n-1) + (2n-1) (N/n)2 yields n ~ N2for large N.

7.a)	An audio streaming server has a one way distance of 50 msec with a media player.  It outputs at 1Mbps.  If the media player has a 1-MB buffer.  What can you say about the position of the low-water mark and high water mark?

b)	Discuss connectionless internetworking.

8.	Write notes on:

a) ISDN protocol architecture.

b) Link control using V.120.

c) Conceptual view of ISDN.

@@@@@