Code No.420251

OR

- IV-B.Tech. II-Semester Supplementary Examinations, July 2003

UTILIZATION OF ELECTRICAL ENERGY

(Electrical and Electronics Engineering)

Time: 3 Hours							   Max. Marks: 70

Answer any FIVE questions

All questions carry equal marks.

---

1.a)	Where is the use of Individual drive recommended and why?

b)	 The speed of a 15 h.p. (Metric) 400 V d.c. Shunt motor is to be reduced by 25% by the use of a controller. The field current is 2.5 amps and the armature resistance is 0.5 Ohm. Calculate the resistance of the controller, if the torque remains constant and the efficiency is 82%.

2.a)	Derive an expression for the time-dependent temperature as the electrical

Apparatus cools, in terms of the cooling time constant.

b)	A 60 h.p. Motor has a final temperature rise of 45 degrees C on continuous full 	load. Its heating and cooling time constants are 100 and 150 minutes, 	respectively. The load cycle is as follows:

20 minutes at a certain load and 40 minutes on no-load.

Find the rating of the motor.

3.a)	What is skin effect? How this effect is made use in heating?

b) 	Write the expression for skin depth. What are the application of about heating.

4.a)	Explain how electric regeneration braking is obtained with a D.C. locomotive.  How is the braking torque varied?

b)	What are the requirements to be satisfied by an ideal traction system?

5.a)	Explain how you will measure the candlepower of a source of light.

b)	Explain the Rosseau's construction for calculating M.S.C.P. of a lamp

6.a)	Discuss about filament design incandescent lamps.

b)	 Discuss the various factors involved in the selection of lamps and the illumination required for street lighting.

7.a)	Discuss the merits and demerits of the D.C and 1- A.C systems for the main and suburban line electrification of the railways.

b)	Which system you consider to be the best for the suburban railways in the             	vicinity of large cities? Give reasons for your answer.

Contd….2

Code No:420251				       -2-					OR

8.a)	Deduce from the first principles the relationship between acceleration, retardation, maximum speed, running time and the distance between stops for a simplified speed-time curve run of an electric train.

b)	 A locomotive accelerates a 350 tonne train up a gradient of 1 in 100 at 	0.8km/hr/sec. Assuming the coefficient of adhesion to be 0.25, determine the 	minimum adhesive weight of the locomotive. Assume train resistance 40 	newtons/tonne and allow 10% for the effect of rotational inertia.