Code No: 421351

OR

IV-B.Tech. II-Semester Supplementary Examinations July, 2003

Digital control systems

(Electronics and Control Engineering)

Time: 3 hours						               Max. Marks: 70

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Find the Z-transform for the following:

i) te−at	ii) s+b(s+b)2+a2

b)	Find inverse Z-transform for the following:

i) z−4(z−1)(z−2)2	ii) 3z2+2z+1z2−3z+2

2.	For the sampled data system shown in fig.1, find the response to unit step input if 	 G(s)=1s+1

Fig.1

3.	A discrete – time system is described by the difference equation.

Y(k+2) + 5y(k+1) + 6y(k) = u(k)

y(0) = y(1) = 0; T = 1 sec.

- Determine a state model in canonical form.

- Find the state transition matrix.

- For input u(k) = 1, for k  0, find y(k).

4.	Check the stability of the system whose characteristic equations are given below using Jury’s stability test.

a)	z3−0.2z2−0.25z+0.05=0

b)	z4−1.7z3+1.04z2−0.268z+0.024=0

Contd…..2.

Code No: 421351				:: 2 ::					OR

5.	Design a lead compensator in the r-domain for the following system given in fig.2.0 with the following specifications.

Fig. 2.0

Specifications: Kv 4 sec-1

Phase margin 40o

Bandwidth   1Hz (i.e. 6.28 rad/sec.)

6.	Find the control sequence so that the following performance index is minimized;

J=x2(N)+k=13[x2(k)+u2(k)]

for the system

x(k+1) = 0.368 x(k) + 0.632 u(k)

and also find the control sequence when N.

7.a)	Write the procedure for design of a digital control system by minimum principle.

b)	Obtain a solution of discrete matrix Riccati equation.

8.	Write short notes on the following:

a) Controllability and observability of digital systems.

b) Kalman filter for design of digital state observers.

@@@@@