Code No:410306

Set No

1

IV-B.Tech. I-Semester Supplementary Examinations, May 2003.

FINITE ELEMENT METHODS

(common to Mechanical Engineering and Production Engineering.)

Time: 3 hours							         Max. Marks:80

Answer any FIVE questions

All questions carry equal marks

- - -

1.	Using the finite element principles, calculate the area of a circle of diameter ‘d’ 	with lower bound approximation.

2.	With the help of a suitable example, derive the finite element equations of an 	elastic bar using direct stiffness method.

3.	Derive the element stiffness matrix and stress in a 2 – noded plane truss element 	from the first principles?

4.	Determine the stiffness matrix, stresses and reactions in the truss structure shown 	below:-

E=200GPa

A=1000mm2                                                              50kN

500mm

750mm

5.a)	What is a constant strain triangular element? State its properties and applications.

b)	The nodal coordinates of the triangular element are shown in Fig.1. At the interior Point P the x coordinate is 3.3 and the shape function at node 1 is N1 is 0.3. Determine the 	shape functions at nodes 2 and 3 and also the y coordinate of the point P.

Y

3 (4, 6)

. P         2 (5, 3)

1 (1,2)

X

Contd……2

Code No:410306					-2-			Set No.1

6.		Estimate the temperature distribution in 1-D fin as shown in figure by making into 	two elements?

5 X 10-4 W/m2K, 200C

0.01 m radius

1400C

k=70X10-2W/mK

0.05m

7.	Discuss the finite element methodology to solve the torsion problems from the 	first principles?

8.	Discuss about the following mesh generation methods:						(i) 	Mesh topology first.

- Nodes first.

- Adaptive mesh template.

- Nodes and elements simultaneously.

- - -

Code No:410306

Set No

2

IV-B.Tech. I-Semester Supplementary Examinations, May 2003.

FINITE ELEMENT METHODS

(common to Mechanical Engineering and Production Engineering.)

Time: 3 hours							         Max. Marks:80

Answer any FIVE questions

All questions carry equal marks

- - -

1.	With a suitable example, explain the physical interpretation of finite element 	method for one dimensional analysis.

2.	Derive the various equations for static equilibrium of an elastic body.

3.	Estimate the stiffness matrix and the deflection at the center of the simply 	supported beam of length 3 m.  A 50 kN of load is acting at the center of the 	beam.  Take EI = 800 X 103 N-m2.

4.	Explain the methodology to formulate finite element analysis for solid truss 	element and write the elemental stiffness matrix.

5.	The nodal temperature values for the simplex element shown in the figure given 	below are  Ti = 130 0C, Tj = 100 0C and Tk = 120 0C. Determine where the 125 0C 	isotherm intersects the boundaries of the element.

Y													3 (0.5, 1)

2 (1, 0.25)

X

1 (0, 0)

6.	Derive the elemental mass matrix for 1-D bar element and 1-D plane truss element?

Contd……2

Code No:410306					-2-			Set No.2

7.	Consider axial vibrations of the steel stepped bar as shown in figure: (a) develop global stiffness matrix and mass matrix, (b) natural frequencies and (c) mode shapes.

600 mm2

10 kN

350 mm2

0.275 m  		0.275 m

8.a)	Distinguish between linear static analysis and non-linear static analysis.

b) 	Define structural analysis and explain the types of structural analysis.

- - -

Code No:410306

Set No

3

IV-B.Tech. I-Semester Supplementary Examinations, May 2003.

FINITE ELEMENT METHODS

(common to Mechanical Engineering and Production Engineering.)

Time: 3 hours							         Max. Marks:80

Answer any FIVE questions

All questions carry equal marks

- - -

1.	Explain the mathematical interpretation of finite element method for one 	dimensional field problems.

2.	Derive the finite element equations from the one dimensional second order 	equation by variational approach.

3.	Estimate the displacement vector, stresses and reactions for the truss structure as 	shown below:

30o                500N

300mm

2000 N

400mm

4.	Define and derive the Hermite shape functions for a two nodded beam element?

5.a)	Show that the shape function at node i (Ni ), for the simplex triangle is zero when 	evaluated at nodes j and k.

b)	The nodal displacements for the simplex two-dimensional element	shown in the 	figure given below are u1 = 2 mm, u2 = 6 mm, u3 = -1 mm, v1 = 4 mm , v2 = 5 mm 	and v3 = 8 mm. Determine the displacement components at an interior point        	B (10,10). The nodal coordinates (in mm) are given in parenthesis.

Y

3 (8, 20)

X

3 (0, 0)					 2 (20, -5)

Contd……2

Code No:410306					-2-			Set No.3

6.	Discuss the methodology to solve the Eigen value problem for the estimation of 	natural frequencies of a stepped bar?

7.	Determine the natural frequencies and mode shapes of a stepped bar as shown in 	figure using the characteristic polynomial technique. Assume E = 25 Gpa and 	density is 7850 kg/m3.

400 mm2

300 mm2

10 kN

0.3 m  			0.3 m

8.a)	Explain about adaptive meshing technique.

b)	Define static analysis and what are the different loads that are applied in static 	analysis.

- - -

Code No:410306

Set No

4

IV-B.Tech. I-Semester Supplementary Examinations, May 2003.

FINITE ELEMENT METHODS

(common to Mechanical Engineering and Production Engineering.)

Time: 3 hours							         Max. Marks:80

Answer any FIVE questions

All questions carry equal marks

- - -

1.	With a help of a neat block diagram, explain the model based simulation process 	of finite element method.

2.	With a suitable example explain the formulation of finite element equations by direct approach. Assume suitable data for your example.

3.	Determine the displacements at nodes and the stresses in elements shown in 	figure:

12kN

300mm

E=70GPa

A=200mm2                            500mm                           400mm

4.	Derive the methodology to develop a stiffness matrix and load vector for a 2-	noded beam element with 2 degree of freedom?

5.a)	Prove that a three nodded triangle element has a constant strain or stress state.

b)	For the triangular element shown in the figure given below, find the consistent 	nodal load.

p / unit length

3 ( 3,4)

1 (0, 0)			2 (3, 0)

Contd…..2

Code No:410306					-2-			Set No:4

6.		Explain the methodology for the treatment of all three boundary conditions in a

1-D heat transfer element?

7.	Consider the axial vibrations of a steel bar shown in the figure: (a) Develop global 	stiffness and mass matrices, (b) determine the natural frequencies?

1200 mm2

900 mm2

E = 2e11 N/mm2

Density

7200 kg/m3

0.3 m  		      0.4 m

8.a)	Explain the automatic mesh generation technique.

b) 	Explain coupled field analysis and give some examples.

- - -