Code No:410851

OR

IV-B.Tech. I-Semester Supplementary Examinations May 2003.

PROCESS DYNAMICS AND CONTROL

(Chemical Engineering)

Time: 3 hours							Max. Marks:70

Answer any FIVE questions

All questions carry equal marks

- - -

1.	Define any three forcing functions and obtain expressions for the response of a 	first-order system to those forcing functions.

2.	N storage tanks of volume V are arranged in series so that when water is fed into 	the first tank, an equal volume of liquid overflows from the first tank into the 	second and so on.  Each tank initially contains component A at some 	concentration Co and is equipped with a perfect stirrer.  At time zero, a stream of 	zero concentration is fed into the first tank at a  VFR of q.  Find the resulting 	concentration in each tank as a function of time.

3.	Obtain the transfer function relating the applied pressure and the reading indicated 	by a u-tube manometer.  You may assume that the flow in the manometer is 	laminar and that the steady state friction law for dragforce in laminar flow is valid 	at each instant.

4.	Sketch the signal-flow diagram of a simple control loop indicating its basic 	components.  Explain negative feed back, servo and regulator mechanisms.

5.	Using the Ziegler-Nichols rules, determine the settings of a proportional integral 	controller for a control system with the following components:

Process ……..	e−1.02S(S+1)2

Final control element ……  1

Measuring element ……..    1

6.	Sketch the root locus diagram for the open loop transfer function,

4kc(S+3)S(0.5S+1)(0.5S2+S+1)

7.	What is transportation lag?  Explain, in detail, Smith predictor for dead time 	compensation.

8.	Write short notes on any THREE of the following:

(a) Linearization.  (b) Ratio control.  (c) Stability.  (d) Selection of a controller.

- - -