Code No:410404

Set No

1

IV-B.Tech. I-Semester Supplementary Examinations, May 2003.

OPTICAL COMMUNICATIONS

(common to Electronics and Communication Engineering and Electronics and Telematics.)

Time: 3 hours							         Max. Marks:80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Explain a digital optical fiber link using a semiconducter laser source and 	avalanche photo diode (APD) detector.

b)	What are the applications of optical fiber communication?

2.a)	Explain in detail about the Stimulated Brillouin Scattering and Stimulated Raman 	Scattering when the optical power is passed through a fiber.

b)	When long single mode fiber is operating at a wavelength of 1300 nm, the 	attenuation is 0.5db/Km .The core diameter of the fiber is 6m and the laser 	source bandwidth is 600 MHz.Calculate the threshold optical powers for the 	stimulated Brillouin and Raman Scattering.

3.	Derive the expression for the waveguide dispersion also find the relationship 	between V and β.

4.	Describe the techniques used to give both electrical and optical confinement in 	multimode injection lasers. Contrast these techniques when used in gain-guided 	and index guided lasers.

5.	Draw a simple model of a photo detector receiver and its equivalent circuit. 	Explain the different noise sources available in photo detector receiver.

6.	What are pre amplifiers? Describe the different types of pre amplifier with 	suitable circuit.

7.	What are the underlying principles of the WDM technique? What are its various 	advantages? How is it different from FDM technique?

8.a)	Discuss with the aid of suitable diagram the measurement of dispersion in optical 	fibers using time domain measurement technique?

b)	The pulse dispersion measurements are taken on a multimode graded index fiber 	in the time domain.  The 3 dB width of the optical output pulses from a 950m 	fiber length is 827ps.  When the fiber is cut back to a 2m length the 3dB width of 	the optical output pulses becomes 234 ps.  Determine the optical bandwidth for a 	kilometer length of the fiber assuming Gaussian pulse shapes.

- - -

Code No:410404

Set No

2

IV-B.Tech. I-Semester Supplementary Examinations, May 2003.

OPTICAL COMMUNICATIONS

(common to Electronics and Communication Engineering and Electronics and Telematics.)

Time: 3 hours							         Max. Marks:80

Answer any FIVE questions

All questions carry equal marks

- - -

1.	Explain in detail elliptical and circular propagation of light.

2.a)	Discuss briefly about radiative losses in the optical fiber.

b)	Explain the core and cladding losses in the optical fiber and also derive the 	expression for those losses.

3.	Derive the expression for the relationship between material dispersion and 	wavelength of the optical fiber with suitable examples.

4.	What is internal quantum efficiency and derive the expression for the lifetime 	reduction caused by interfacial recombination.

5.a)	Why power bandwidth is an important parameter in an optical communication 	system? 
   b) 	Explain in detail the various factors which affect the performance of optical 	source.

6.	What are the requirements of an optical receiver? Using a flow chart explain the 	receiver design .

7.	Compare the advantages and disadvantages of using WDM in an optical fiber  	communication system.

8.a)	Discuss measurement of  fiber scattering loss by describing the use of two 	common scattering cells.

b) 	A Nd:YAG laser operating at a wavelength of 1.064m is used with an 	integrating sphere to measure the scattering loss in an optical fiber sample. The 	optical power propagating within the fiber at the sphere is 98.45W and 5.31W 	of optical power is scattered within the sphere. The length of fiber in the sphere is 	5.99cm. Determine the optical loss due to scattering for the fiber at a wavelength 	of 1.064m in decibels per kilometer.

- - -

Code No:410404

Set No

3

IV-B.Tech. I-Semester Supplementary Examinations, May 2003.

OPTICAL COMMUNICATIONS

(Common to Electronics and Communication Engineering and Electronics and Telematics.)

Time: 3 hours							         Max. Marks:80

Answer any FIVE questions

All questions carry equal marks

- - -

1.	Describe the quantum nature of light. Explain basic optical laws in detail with 	relevant mathematical expressions.

2.	What are the three important mechanisms that are responsible for absorption 	losses in signal through an optical fiber? Explain in brief the curve for 	wavelength versus attenuation for different ranges of the signal.

3.a)	Explain the pulse dispersion with suitable diagrams in detail.

b) 	Tabulate the various dispersion and optical parameters of LED, Multimode laser 	diode and single mode Laser diode.

4.	Explain the modulation capability and transient response of the LED.

5.	Discuss about the basics of PN junction and explain the mechanism by which 	optical radiation is generated.

6.	Describe the fiber optic link receiver by drawing a circuit diagram.

7.a)	How is WDM  accomplished?

b)  	What are all the possibilities for WDM systems?

c) 	What is DWDM?

8.a)	Describe with the aid of a suitable diagram, for measurement of fiber attenuation 	using cutback technique?

b) 	A spot measurement of fiber attenuation is performed on a 1.5km length of

optical fiber at a wavelength of 1.1m. The measured optical out put power from 	the 1.5km length of fiber is 50.1W.  When the fiber is cut back to a 2m length, 	the measured optical output power is 385.4W.  Determine the attenuation per 	kilometer for the fiber at a wavelength of 1.1m.

- - -

Code No:410404

Set No

4

IV-B.Tech. I-Semester Supplementary Examinations, May 2003.

OPTICAL COMMUNICATIONS

(common to Electronics and Communication Engineering and Electronics and Telematics.)

Time: 3 hours							         Max. Marks:80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Explain step index fiber structure.

b)	Write a brief essay on the presentation of array optics.

2.a)	Compare the optical parameters of free space with dispersive and non dispersive 	mediums.

b) 	List the differences between intramodal & intermodal dispersions.

3.	Derive the expression for the pulse dispersion in a parabolic index medium and in 	a planar step index waveguide.

4.	Explain the modulation capability of the laser diode and its temperature effects. 	How to compensate for variations in temperature?

5.	With schematic representation explain the working principle of pin photo detector 	and obtain the expression for cutoff wavelength.

6.a)	Describe with diagram and explain the operation of a unidirectional WDM system.

b)      Discuss about a bidirectional WDM system .

7.		Compare digital and analog optical receivers.

8.a)	Explain about LED coupling to single-mode fibers?

b) 	A GaAs optical source with a refractive index of 3.6 is coupled to a silica fiber 	that has a refractive index of 1.48.  If the fiber end and the source are in close 	physical contact, then find the Fresnel reflection at the interface and power loss in 	dB?

- - -