Code No: 210502

Set No.

1

II B.Tech I Semester Examination November/December 2002

LINER AND DIGITAL I. C. APPLICATIONS

(Computer Science & Engineering, Computer Science and Information Technology, Computer Science and Systems Engineering)

Time: 3 Hours							      Max. Marks: 80

# Answer any Five questions

All questions carry equal marks

----

1.a)	Explain the operation of Zero crossing detector.

b)	Briefly mention the disadvantages of using Zero crossing detector and how it is 	overcome in Schmitt Trigger?

2.a)	With a circuit diagram and waveform explain the operation of an inverting and 	on inverting comparators?

b)	What is the limitation of opamp as comparators?

3.a)	Explain the functions of each of pins in 555 timer.

b)	List the important features of 555 timer.

4.a)	Explain the concepts of ‘Modulation’ and ‘De-modulation’.

b)	Explain the use MC1496 as AM modulator with necessary circuit diagram.

5.a)	State the transfer functions for second order low pass filter,high pass filter, band 	pass filter and notch filter.

b)	Among Bessel, Butterworth & chebyshev filters which one is preferable and why?

c)	What is universal filter? Explain ?

6.a)	With neat sketch explain the operation of difference amplifier.

b)	Explain the constriction & working of an ECL gate.

7.	What is a sample and hold circuit? Why is it needed? Explain the operation of one 	sample and hold circuit with neat waveforms and circuit diagrams.

8.	Breifly explain the following.

a)	ADC specifications

b)	Weighted Resistor DAC

c)	Analog multiplexers.

$$$

Code No: 210502

Set No.

2

II B.Tech I Semester Examination November/December 2002

LINER AND DIGITAL I. C. APPLICATIONS

(Computer Science & Engineering, Computer Science and Information Technology, Computer Science and Systems Engineering)

Time: 3 Hours							      Max. Marks: 80

# Answer any Five questions

All questions carry equal marks

----

1.a)	Explain how an opamp can be used as summing amplifier? Draw the diagram of a 		four input summer.

b)	Define the common-mode rejection ratio(CMMR) and explain the significance of 			a relatively large value of CMMR

2.a)	Explain current fold back characteristics.

b)	What is the principle of switch mode power supply? Discuss its advantages and 			disadvantages.

3.a)	Discuss the operation of FSK generator using 555 timer.

b)	Explain the how Monostable multivibrator using 555 timer is used as divide-by-3 			network

4.a)	Draw the functional circuit diagram of IC1496 and explain its action.

b)	Discuss any one application along with necessary circuit diagram using IC1496.

5.a)	Differentiate narrow band pass filter & wide band pass filter

b)	Draw the single op-amp band pass filter response.

c)	Derive the equation for band width for narrow band pass filter.

6.a)	With neat circuit explain the operation of TTL inverter.

b)	Briefly explain the different types of TTL gates.

7.		Explain the operation of an Analog to Digital converter which can be used to 			converter video signal in to Digital format with neat diagram.

8.a)	Draw the circuit diagram of a 6 bit inverter R-2R ladder DAC

b)	What is the maximum output voltage if the logic 1 at the input representing 5V.

c)	What is the minimum voltage that can be resolved.

$$$

Code No: 210502

Set No.

3

II B.Tech I Semester Examination November/December 2002

LINER AND DIGITAL I. C. APPLICATIONS

(Computer Science & Engineering, Computer Science and Information Technology, Computer Science and Systems Engineering)

Time: 3 Hours							      Max. Marks: 80

# Answer any Five questions

All questions carry equal marks

----

1 a)	Define the following opamp parameters.

i)	Slew rate

ii)	Power supply rejection ratio

- Common mode rejection ratio

- Input offset voltage

b)	Give circuit of a scale changer and summer using opamp and explain their 				operation.

2 a)	How will you generate square wave with the help of clipper?

b)	Write notes on programmable trans conductance amplifier.

3 a)	Explain the significance of each of comparators and operation of 555 timer.

b)	Explain the application of 555 timer as Linear ramp generator.

4 a)	Explain the terms Lock rang, Capture range and Pull-in time a PLL.  How are 			Lock rang and Capture range determine?

b)	List the applications of IC 1496.  Explain the application of IC 1496 as a Mixer 			circuit.

5 a)	How filter response changes with the order of the filter?  Explain with suitable 			Examples and sketches.

b)	Differentiate magnitude, frequency and phase responses of filters

6 a)	Explain the fabrication of schottky transistor

b)	Discuss the transfer characteristics of TTL gates i.e resistor pull-down V/S active 			pull-down.

7 a)	Explain the working and circuit operation of the sampler and hold circuit with one 		of the most popular S/H circuit.  Also discuss the applications of the above circuit.

8 a)	Explain in detail about the important specifications of ADC and DAC.

b)	Explain how dual slop ADC provides noise rejection.

###

Code No: 210502

II B.Tech I Semester Examination November/December 2002

Set No.

4

LINER AND DIGITAL I. C. APPLICATIONS

(Computer Science & Engineering, Computer Science and Information Technology, Computer Science and Systems Engineering)

Time: 3 Hours							      Max. Marks: 80

# Answer any Five questions

All questions carry equal marks

----

1 a)	Briefly explain the characteristics of an ideal opamp.

b)	What do you mean by input offset voltage and input offset current of an opamp? 		Explain?

2 a)	Name the circuit that is used to detect the peak value of the nonsinusoidal 	waveforms.  Explain the operation.

b)	What is the difference between clippers and clampers?  Give one application of 	each.

3 a)	Derive the expression for Lock-in-range and Capture range in PLL.

b)	Explain the application of PLL for FM demodulator.

4 a)	Explain an application in which the 555 timer can be used as Astable 	multivibrator.

b)	What are the advantages and disadvantages of Monolithic PLLs over discrete 	PLLs?

5 a)	What is filter sensitivity?  Explain.

b)	Sketch the low pass, high pass and symmetric notch response

c)	What is all pass filter?  And where it is use?

6 a)	List & Explain the specification of digital IC families

b)	With relevant figure explain the operation & advantages of wired AND 	connection.

7 	With neat diagram, explain the operation of Flash type Analog to Digital

Converter.

8 a)	Describe the various types of electronic switches used in D/A converters.

b)	Why is an inverted R-2R ladder network DAC better than R-2R ladder network 	DAC?

c)	Give the conversion time for

i)	Counter type ADC.

ii)	Successive Approximation type DAC.

- Dual slop DAC.

###