Code No: 211051

# OR

II B.Tech I- Semester Examination November/December 2002.

## ELETRICAL AND ELECTRONIC MEASUREMENTS

(Common to EIE, E.Con.E)

Time: 3 hours								          Max. Marks: 70

Answer any Five questions

All questions carry equal marks

----

1 a)	With a neat diagram explain the principle of operation of a suspension galvanometer. Also explain how the torque and deflection are obtained in the galvanometer.

- A moving coil instrument has the following data

Number of turns		=	100

Width of the coil		=	20 mm

Depth of the coil		=	30 mm

Fluse density in the gap	=	0.1 wb/m2

Calculate the deflecting torque when carrying current of 10 MA.  Also calculate the deflection, if the control spring constant is 2 x 10-6 Nm/degree.

2 a)	With a neat diagram explain how a D’ Arsonval movement can be used for the measurement of DC currents.  Derive the expression for shunt resistance Rsh.

b)	Design a multi-range DC ammeter for measuring the current with ranges 1A, 5A, 10A.  A D’ Arsonval movement with an internal resistance of 50 ohms and full-scale deflection current of 1m A is used.

3 a)	With a neat circuit diagram explain the operation of a FET input voltmeter.  What are the advantages and disadvantages if this voltmeter.

b)	With the help of a neat block diagram explain the principle of operation of a chopper type DC amplifier voltmeter (Micro voltmeter).

4 a)	With a neat block diagram explain the operation of a CRD.  With examples, explain how amplitude and frequency of a wave form are measured.

- Explain the use of Lirsajous figures for the phase measurement in a CRD.

5 a)	With a net block diagram explain the principle of operation of digital storage oscilloscope.  Explain in detail about each block.

b)	With a neat construction diagram explain the principle of operation of a bistable storage tube.

6 a)	With a neat block diagram explain the principle of operation of a spectrum analyzer.  Explain indetail about each block.

b)	Explain the advantages and disadvantages and list out a few applications of spectrum analyzer.

… Contd .2

Code No: 211051				-2-				OR

7 a)	With a neat block diagram explain the function of each block of an electronic counter.

- Discuss the following typical modes of operation of an electronic counter.

  - Totalizing mode

  - Frequency mode

  - Ratio mode

  - Period mode

  - Time interval mode

8 a)	Write short notes on any two of the following:

- Q – meter

- Walt – hour meter

- Power factor meter

###