Code No: 210602

Set No.

1

II B.Tech I Semester Examination November/December 2002

## FURNACE TECHNOLOGY & PYROMETRY

(Metallurgy & Material Technology)

# Time: 3 hours							       Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1 	Describe characteristic features of Arc furnaces and of tube resistance furnaces. Identify some uses for which these furnaces are better suited than other types of

Furnaces.

2	Define the efficiency of operation of a furnace.  Indicate the sources of heat loss in a furnace and also indicate steps that can be taken to minimize these losses.

3 a)	What is the 'Seebeck effect' and how is it used to help measure temperature?

b)	What is a thermopile?  What is the advantage of such an arrangement?

4 a)	What are resistance thermometes?

- What are the disadvantages in the use of typical resistance thermometers?

- Why are compensating bridge circuits required while using resistance thermometers?

5 a)	What behavior must a body display to be classified as a black body?

b)	From the point of view of emission, what type of body are most solids and liquids to a reasonable approximation?

6	Describe the principle and construction of a pyro-optical-pyrometer. Identify some situations where it will be the convenient way to measure temperature.

7	Derive an expression for the steady state conduction of heat perpendicular to the walls of a hollow sphere.

8	Water at the rate of 75 Kg/min is heated from 40C to 80C by an oil having a specific heat of 1.9 KJ/Kg C.  The fluids are used in a counter flow heat exchanger and the oil enters the exchanger at 120 C and leaves at 80 C.  The overall heat transfer coefficient is 320 W/m2  C.  Calculate the heat exchanger area.

###

Code No: 210602

Set No.

2

II B.Tech I Semester Examination November/December 2002

## FURNACE TECHNOLOGY & PYROMETRY

(Metallurgy & Material Technology)

# Time: 3 hours							       Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1	Describe characteristic features of continuous furnaces and of vertical shaft   furnaces.  Identify some uses for which these furnaces are better suited than other types of furnaces.

2	Derive an expression that describes heat balance during the operation of a tube resistance furnace.  Clearly indicate all the operating conditions that you are assuming.

3 a)	Draw a simple thermocouple circuit and indicate the factors that contribute to the emf developed in the circuit.

b)	What does thermoelectric power mean as it applies to circuit in part  a) of this    question?

c)	Hoe are the thermoelectric powers of three pairs of metals, A&B, B&C, C&A,

related to each other for the same junction temperatures T1 and T2 ?

4 a)	Describe the principle of operation of a resistance thermometer.

- What are the advantages of a resistance thermometer over a thermocouple.

5 a)	What factors affect the energy radiated by a body?

b)	What approaches can be used to measure the temperature of a body using radiation techniques?

6	What are the sources of error in the use of optical pyrometers and how do you compensate for them?

7	Derive an expression for the steady state conduction of heat through the walls of a hollow cylinder to the exterior of the cylinder.

8	Two very large parallel plates, one at temperature 1200 C and emissivity 0.3 and the other at 500 C with emissivity 0.8, exchange heat.  Calculate the heat flow per unit area between these two plates.

###

Code No: 210602

Set No.

3

II B.Tech I Semester Examination November/December 2002

## FURNACE TECHNOLOGY & PYROMETRY

(Metallurgy & Material Technology)

# Time: 3 hours							       Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1	Describe characteristic features of continuous furnaces and of induction type furnaces.  Identify some uses for which these furnaces are better suited than other types of furnaces.

2	Describe methods you could use to measure heat loss from a furnace.

3 a)	Describe the principle and construction of a resistance thermometer?

b)	What materials are commonly used for the construction of resistance thermometers?

- List some advantages of resistance thermometers over other types of temperature measuring devices.

4 a)	What characteristics must materials possess to make them ideal for use in a thermocouple?

b)	What are base metal thermocouples and what are some of their advantages and disadvantages?

5 a)	What is Plank's law and how is it made use of to measure temperature of a body?

- What factors determine the energy radiated by a body?

- Write an expression for the heat energy radiated from a surface area 'A' in unit time. Identify the constant in the expression.

6	Describe the principle and method of measuring temperature using a disappearing filament optical pyrometer?

7	Indicate what happens when the surface temperature of a thick wall is suddenly decreased.

8 a)	Draw the temperature profiles expected in a counter flow double pipe heat exchanger for the hot fluid and the cold fluid.

b)	Water at the rate of 68 Kg/min is heated form 35 C to 75 C by an oil having a specific heat of 1.9 KJ/Kg C.  The fluids are used in a counter flow heat exchanger and the oil enters the exchanger at 110 C and leaves at 75 C.  The overall heat transfer coefficient is 320W/m2 C.  Calculate the heat exchanger area.

###

Code No: 210602

Set No.

4

II B.Tech I Semester Examination November/December 2002

## FURNACE TECHNOLOGY & PYROMETRY

(Metallurgy & Material Technology)

# Time: 3 hours							       Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1	Describe characteristic features of reverberatory furnaces and of tube resistance furnaces.  Identify some uses for which these furnaces are better suited than other types of furnaces.

2	Derive an expression that describes heat balance during the operation of a muffle type resistance furnace.  Clearly indicate all the operating conditions that you are assuming.

3 a)	Describe the principle of functioning of a thermocouple and name one commonly used pair of thermocouple wire materials.

b)	What are some advantages of thermocouples compared to other temperature measuring devices?

c)	What is 'Peltier effect'? What use can it be put to?

4 a)	What are some sources of error in the use of  error in the use of resistance thermometers?

- What can be done to compensate or correct for these errors?

5 a)	What is 'Wien's displacement law'?  Explain using a schematic plot of spectral energy distribution.

- Describe on object that closely approximates to a black body.

6	What is effect of the following on optical pyrometer based temperature measurement:

- Distance between pyrometer and source

- Presence of absorbing media

7	Describe, briefly, the three mechanisms by which heat transfer can occur.

8	Derive an expression for the log mean temperature difference for the counter flow condition in a double pipe heat exchanger?

###