Code No.:210105

Set No.

1

II/IV B.Tech. I-Semester Examination November/December, 2002.

MECHANICAL AND ELECTRICAL SCIENCE

(Civil Engineering)

Time: 3 hours								          Max. Marks: 80

Note:-1)Answer any THREE from Part-A and

any THREE from Part-B

2)All questions carry equal marks

- - -

## MECHANICAL SCIENCE

## P A R T - A

1.	Give a neat classification of welding process and explain the working principle of oxy-acetylene welding with neat sketch.

2.	In what way steam engine works. With the help of a neat sketch explain the functioning of components.

3.	How the I.C. engines are classified.  Explain the working of four stroke compression Ignition engine.

4.	Explain the working principle of single stage reciprocating air compressor.  Why the malto stage reciprocating air compressor are preferred than single stage compressor.

5.	With block diagram explain the working principle of vapour compression refrigeration system.  Write names of a few refrigerants.

### ELECTRICAL SCIENCE

### P A R T - B

6.a)	Derive the torque equation of D.C motors.

- Develop from first principles an expression for the emf of a d.c generator.  Calculate the e.m.f developed in the armature of a two-pole dc generator, whose armature has 280 conductors and is revolving at 1000 rpm.  The flux per pole is 0.03 wb.

7.a)	Define efficiency and derive the condition for Max. efficiency.

- The primary and secondary windings of a 500 KVA transformer have resistance of 0.42  and 0.0019 respectively.  The primary and secondary voltages are 11 KV and 0.415 KV respectively and the core loss is 2.9 kw, assuming the p.f of the load to be 0.8.  Calculate the efficiency on (i) full load  (ii) half full load  and also find the output at which the efficiency of the transformer is a max and calculate its value.

Contd….2

#### Code No.:210105				-2-				Set No.1

8.a)	Explain the voltage regulation of an alternator by the synchronous impedance method with phasor diagram.

- Sketch and describe the construction of a moving coil ammeter and give the principle of operation.

9.a)	Explain why an induction motor cannot develop torque when running as synchronous speed.  Define the slip speed of an induction motor and deduce how the frequency of rotor currents and the magnitude of the rotor emf are related to slip.

- If the star connected rotor winding of a three phase induction motor has a resistance of 0.01/ph and a standstill leakage reactance of 0.08 /ph, what must be the value of the resistance per phase of a starter to give the max. torque?  What is the percentage of slip when the starting resistance has been reduced to 0.02/ph.  If the motor is still exerting its max. torque.

10.	Write short notes on the following: (a) Three point starter  (b) Slip Torque characteristics of Induction motor.

- - -

Code No.:210105

Set No.

2

II/IV B.Tech. I-Semester Examination November/December, 2002.

MECHANICAL AND ELECTRICAL SCIENCE

(Civil Engineering)

Time: 3 hours								          Max. Marks: 80

Note:-1)Answer any THREE from Part-A and

any THREE from Part-B

2)All questions carry equal marks

- - -

## MECHANICAL SCIENCE

## P A R T - A

1.	What are the basic working principle of air conditioning and explain the working of summer and winter air conditioning with a neat sketch.

2.	Give a list of various belt drive.  Derive an expression for the length of crossed belt drive.

3.	Describe the working principle of C.I engine and differentiate two stroke and four stroke engine.

4.	Explain the working of single stage reciprocating air compressor.  Why the multistage reciprocating air compressors are prepared than single stage air compressor.

5.	Explain the any acetylene welding process and give a list of welding classifications.

### ELECTRICAL SCIENCE

### P A R T - B

6.a)	Explain the torque speed characteristics of a D.C series motor and give at least two applications.

- Explain the three point starter with protective coils.

7.a)	Derive the e.m.f equation for single phase transformer.

- A 50 KVA single phase transformer of 2300V/230V rating has the primary and secondary winding resistance of 2 and 0.02 respectively.  The iron loss equal to 412 w.  Calculate the efficiency (i) at half full load and (ii) at full load, when the p.f of the load is 0.8.

###### Contd…..2

#### Code No.:210105				-2-				Set No.2

8.a)	Briefly explain the constructional details of alternators.

- A 3 phase star connected alternator is rated at 1600 KVA and 13.5 KV.  The armature effective resistance and synchronous reactance are 1.5 and 30 per phase respectively.  Calculate the percentage regulation for a load of 1280 kw at p.f of 0.8 leading and lagging.

9.a)	Explain from the concepts of magnetic field, how a 3 phase induction motor works.

- A 3-phase induction motor at stand still has a rotor voltage of 100V between the slip rings.  The rotor winding is star connected and has a leakage reactance of 1/phase at stand still and a resistance of 0.2/phase.  Calculate (i) the rotor current when the slip is 4% and the slip rings are short circuited.  (ii) the slip and the rotor current when the rotor develops max. torque.  Assume that the flux remaining constant.

10.a)	Write short notes on the following: (a) EMF equation of a D.C. Generator.

(b) Permanent magnet moving coil instrument.

- - -

Code No.:210105

Set No.

3

II/IV B.Tech. I-Semester Examination November/December, 2002.

MECHANICAL AND ELECTRICAL SCIENCE

(Civil Engineering)

Time: 3 hours								          Max. Marks: 80

Note:-1)Answer any THREE from Part-A and

any THREE from Part-B

2)All questions carry equal marks

- - -

## MECHANICAL SCIENCE

## P A R T - A

1.a)	What are the differences between the petrol and diesel engines?  Discuss.

- Explain the working of a four stroke petrol engine with the help of a line diagram.

2.a)	Define the term ton of refrigeration?  What are the essential properties off an ideal refrigerant?  Explain

- Explain the principle of summer air-conditioning with the help of a line diagram.

3.a)	What is gas welding?  How would you carryout oxy-acetylene welding?  Explain the procedure.

- Compare the DC and AC arc welding.

4.a)	Explain any two methods of taper turning as a lathe.

- Explain different operations that can be performed on a drilling machine.

5.a)	On what factors does the selection of type and size of a power shovel depend.

b)	Explain the operations of a power shovel.

### ELECTRICAL SCIENCE

### P A R T - B

6.a)	What are the important parts of a DC machine.  Explain clearly their functions and specify giving reasons the materials with which they are made.

- A shunt generator delivers 50kw at 250v and 400rpm.  The armature and field resistances are 0.02 and 50 respectively.  Calculate the speed of the machine running as a shunt motor and taking 50 kw input at 250v allow 1v per brush for contact drop.

7.a)	Draw and explain the phasor diagram of a single phase transformer when it is supplying a capacitive load.

- A 400 km transformer has a core loss of 2kw and the maximum efficiency at 0.8 p.f occurs when the load is 240kw.  Calculate (i) the max. efficiency at units p.f.  (ii) the efficiency on full load at 0.71 p.f.

Code No.:210105				  -2-				      Set No.3

8.a)	Bring out, on various aspects, a comparison between 30 squirrel cage and slip ring induction motor.  Consider constructional, operational economical starting and utility aspects.

- A 4 pole, 50 Hz induction motor has an e.m.f in the rotor, the frequency of which is 2 Hz.  Determine the synchronous speed, slip and speed of the motor.

9.a)	Explain moving coil permanent magnetic type of instruments and derive the expression for the torque.

- A 1500 KVA, 6.6 KV, 3-phase, star connected alternator with a resistance of 0.4 /phase and a reactance of 6 /phase delivers the full load current at a p.f of 0.8 lagging at the normal rated voltage.  Estimate the terminal voltage at the same excitation and load current at a p.f of 0.8 leading.

10.	Write short notes on the following: (a) Voltage regulation of a Transformer.

(b) Constructional details of alternators.

- - -

Code No.:210105

Set No.

4

II/IV B.Tech. I-Semester Examination November/December, 2002.

MECHANICAL AND ELECTRICAL SCIENCE

(Civil Engineering)

Time: 3 hours								          Max. Marks: 80

Note:-1)Answer any THREE from Part-A and

any THREE from Part-B

2)All questions carry equal marks

- - -

## MECHANICAL SCIENCE

## P A R T - A

1.a)	What are the differences between two stroke and four stroke engine?  Discuss.

- Describe with neat sketches the working of 2 stroke cycle petrol engine.

2.a)	Explain the requirements of comfort air-conditioning.

- Explain the working principle of vapour compression refrigeration system with the help of a block diagram.

3.a)	State the principle of electric arc welding operation, and explain the factors that affect the depth of penetration in arc welding.

- Explain the process of flame cutting.  Discuss the principle of its operation.

4.a)	Explain the terms speed, feed and depth of cut for a lathe machine.

- Draw a simple sketch of a milling machine.  Explain various milling operations.

5.a)	How is the output of a bulldozer estimated?

b)	Name different types of concrete mixers.  Explain any one of them with the help of a sketch.

### ELECTRICAL SCIENCE

##### P A R T - B

6.a)	Explain the principle of operation of a D.C machine as a generator and motor.

- A 240V, 4 pole DC shunt motor running at 1000 rpm gives 15 H.P. with an armature current of 50A and a field current of 1A.  The armature winding is wave connected and has 540 conductors.  Its resistance is 0.1 and the voltage drop at each brush is 1v.  Find (i) the useful torque  (ii) the total torque  (iii) the useful flux per pole and (iv) the efficiency.

Contd….2

#### Code No.:210105				-2-				   Set No.4

7.a)	Define regulation and derive the condition for maximum efficiency.

- The max. efficiency of a 500 KVA, 300 V 1500V, 50 Hz, single phase transformer is 97% a 3/4th full load and unity p.f.  If the impedance is 10%, calculate the regulation at full load and 0.8 lagging p.f.

8.a)	Explain the torque-slip characteristics.

- An 8 pole, 3-phase induction motor operates from a supply whose frequency is 50 Hz.  Calculate (i) the speed at which the magnetic field of the stator is rotating (ii) the speed of the rotor when slip is 0.03 (iii) the frequency of the rotor current when the slip is 0.04 and  (iv) the frequency of the rotor current when the rotor is at stand-still.

9.a)	Explain the basic principles of operation of alternators.

- A 550V, 55 KVA, single phase alternator has an effective resistance of 0.2.  A field current of  10A produces an armature current of 200A on short circuit and an emf of 450V on open circuit.  Calculate (i) the synchronous impedance and reactance  (ii) the full load regulation when the p.f is 0.8 lagging.

10.	Write short notes on the following: (a) Moving Iron Type of instruments.  (b) Three Point starter.

- - -