Code No. 210453

OR

II/IV B.Tech. I Semester Examination November/December 2002

SIGNALS AND SYSTEMS

(Common to Electronics & Communication Engineering, Electronics & Instrumentation Engineering)

### Time: 3 Hours							Max. Marks: 70

# Answer any Five questions

All questions carry equal marks

---

1.  a)	State and prove Parseval’s power Theorem.

- State and prove the convolution property in relation to Fourier transform

2.  a)	State and prove the sampling theorem.  Illustrate the effect of over sampling and under sampling.

- Determine the Fourier transform of a two sided exponential pulse x(t) = e-|t|

3.	Explain the characteristics of an ideal LPF.  Explain why it cannot be realized?

4.	Consider a gaussian filter H() = exp(-k2 + jt0)

(i)	Determine and sketch the unit impulse response of this filter.

(ii)	State, with reasons, whether this filter is physically realizable.

(iii)	Can this filter be made approximately realizable by providing a sufficient

amount of delay t0?

5.	Show that the process of modulation causes a shift of the ESD of baseband signals and the energy of the modulated signal is half the energy of the baseband signal.

6.  a)	Let g(t) = x(t) + x(-t),

Where x(t) = e-t u(t) and laplace transform of g(t) is

s

G(s) = --------		-1 < Re{s} < 1

s2-1

- Brief Explain all the properties of Laplace transform

2s + 3

7.	For a system with transfer function H(s) = --------------

s2 + 2s + 5

Find the zero state response for (i) f(t) =10u(t) (ii) u(t-5)

8.	Given X(z) = 2z(z2-1)/[z2+1], find x(n)

@@@@@