Code No: 211003

# Set No.

1

II B.Tech. I Semester Examination November/December, 2002

ELECTROMAGNETIC THEORY

(Electronics Instrumentation Engineering, Electronics & Control Engineering)

Time: 3 hours.								       Max.Marks: 80

Answer any FIVE questions.

All questions carry equal marks.

- - -

1 a)	State and explain electric field at a point in space.

b)	A line charge of uniform density ‘q’ coulombs per meter extends from the point (0, -a) to the point (0,a) in the x-y plane.  Determine the electric field intensity E at point (a, 0).

2 a)	Find the potential at a point P (r = 10,  =60,  = 0) due to dipole with Q=1 `	mirocoulomb at (0.1, 0,0) and –1 microcoulomb at (-0.1, 0, 0).

b)	Derive an expression for the potential due to the quadrapole at a large distance R in comparison with the spacings.

3 a)	State and explain Biot-Savart law and relate it to Amperes law.  Show that the divergence of magnetic induction is always zero.

b)	If H = x2yax+0.1xazA/m find curlH.

4 a)	Define magnetic moment.

b)	Determine force per meter length between two parallel wires A & B separated by 4 cm in air and carrying currents of 30 amps.

- in the same direction

- in the opposite direction

c)	Determine the vector magnetic potential near a long conductor carrying steady current of I

5 a)	State and prove the boundary conditions on the tangential component of time varying E-field and the normal component of time varying B-field at the surface

of discontinuity.

b)	An a.c. voltage source v=V0 sub t connected across a parallel plate capacitor C. Verify that the displacement current in the capacitor is the   same as the conduction current in the connecting wires.

6 a)	Starting from the Maxwell’s curl equations, derive the wave equation in Magnetic field for free space.

- A plane wave with frequency f = 16 GHz travels in a distilled water with r = 1,r  = 50,  = 20 mhos/m.  Calculate the propagation constant, attenuation, phase factor and intrinsic impedance.

Contd…

## Code No: 211003				-2-				        Set No.1

7 a)	Starting from the Wave equation in E field for perfect conducting medium, obtain the solution for the same.

b)	A large copper conductor r = 1, r = 1,  = 5.87 x 106 mhos/m.  Supports a 10 GHz plane wave.  Compute ,,, and phase velocity of propagation.

8 a)	Establish the poynting theorem and explain the significance of poynting vector.

- Obtain an expression for the ‘surface impedance’ of a conductor.

###

Code No: 211003

# Set No.

2

II B.Tech. I Semester Examination November/December, 2002

ELECTROMAGNETIC THEORY

(Electronics Instrumentation Engineering, Electronics & Control Engineering)

Time: 3 hours.								          Max.Marks: 80

Answer any FIVE questions.

All questions carry equal marks.

- - -

1 a)	Calculate the electric field intensity that will be just sufficient to balance the gravitational force of an electron.

b)	Determine the charge that produces an electric field strength of 40 volts per cm, at a distance of 30cm in vacuum.

2 a)	A hollow sphere is charged to 12 micro coulombs of electricity.  Find the potential

(i) at its surface (ii) Inside the sphere (iii) At a distance 0.3, from the surface.  The radius of the sphere is 0.1m.

b)	Using Gauss’s theorem, show that a symmetrical spherical charge distribution is equivalent to a concentrated point charge at the center of the sphere as far as the external fields are concerned.

3 a)	Define Loerntz force.

- Derive an expression of force per unit length of two current carrying conductors.

4 a)	State Faraday’s law of electro magnetic induction.

- Define Motional EMF.

- A Straight conductor of length 40cm moves perpendicularly to its axis at a velocity  of 50m/sec in a uniform magnetic field of flux density 1.2 Tesla.  Evaluate the EMF induced in the conductor if the direction of motion.

  - Normal to the magnetic field

  - Parallel to the field.

  - At an angle 60 degree to the orientation of field.

5 a)	Write Maxwell’s equations on electromagnetic fields inpoint form as well as in integral form.  Then give the word statements of the these field equations.

b)	A copper wire carries a conduction current of 1 amp at 60 Hz.  What is the displacement current in the wire?  For copper assume  =0,  = 0, = 508 X 107.

6 a)	Starting from the Maxwell’s curl equations, derive the wave equation in Electric field for free space.

b)	Given a non-magnetic material having r = 2.35,  = 10-4 mhos/m.  Find the loss tangent, attenuation, phase factor and intrinsic impedance at 2.5 MHz.

## Code No: 211003				    -2-				        Set No.2

7 a)	Show that an elliptically polarized wave can be obtained as the superposition of two circularly polarized waves rotating in opposite directions but at the same angular rate.

b)	Determine the loss per kilometer for a plane wave propagating in wet earth at a frequency 0.6 Mhz, if r = 1, r = 10,  = 10-5 mhos/m for the wet earth.

8 a)	Derive expressions for reflection and transmission coefficients for plane electromagnetic waves normally incident on a plane boundary separating two dielectric media.

b)	Plane wave is incident form water on toan air interface with an angle of 45.  Calculate the magnitude of Electric field

i) in air, ii) Quarter wavelength above the surface if the incident E field is 1v/m for water r = 80, r = 1 and  = 0 8m.

###

Code No: 211003

# Set No.

3

II B.Tech. I Semester Examination November/December, 2002

ELECTROMAGNETIC THEORY

(Electronics Instrumentation Engineering, Electronics & Control Engineering)

Time: 3 hours.								          Max.Marks: 80

Answer any FIVE questions.

All questions carry equal marks.

- - -

1 a)	State and explain Gauss’s law.  Derive the equation for potential at a point inside a solid sphere having uniform volume charge density.

b)	 A spherical volume r = 1 cm has a uniform charge density of 1 coulomn/m3.  Calculate the potential at r = 50 cm.

2	Derive an expression for the capacitance of a pair of co-axial cylinders of radii r1 and r2 and length 1.  The dielectric is air and the outside cylinder is earthed.

A cable has core diameter of 2.5cm and impregnated paper insulation of thickness of 2.13 cm and relative permitivity 3.5. Determine the capacitance of the cable per km.

3 a)	Derive the boundary condition for the magnetic field at the interface between two magnetic mediums with and without surface current at the interface.

b)	Obtain the Amperes law in differential form.

4 a)	A copper disc of 0.5 m diameter is rotated at a constant speed of 1000 r.p.m on horizontal axis perpendicular to the disc through its center.  Two brushes make contact with the disc.  One at the edge and the other at the center. If the horizontal component of the earth’s magnetic field is 0.2 Tesla.  Calculate the EMF induced between the brushes.

b)	Distinguish between motional EMF and Transformer EMF.

5 a)	Derive the equation of continuity for Time-varying fields.

b)	Express the Maxwell’s equations in point form and in integral form for sinusoidal time variations.  Use phasor representation.

c)	Within a certain region  = 10-11 F/m, and  = 10-5 H/m.  If Bx = 2x10-4 cos 105 t som10-3 y (Tesla)  (i) find E, (ii) find the total magnetic flux passing through the surface x=0, 0<y<40m, 0<z<2m, at t=1 s.

6 a)	Derive a general expression for the relation between Electric and Magnetic fields.

b)	A wave propagating in a loss less dielectric has the components E = 500 cos (107t - z) ax  V/m and H = 1.1 cos (107t - z) ay A/m.  if the wave is traveling at v= 0.5C, where C is the velocity of light, find r, r, ,  and .

## Code No: 211003				-2-				        Set No.3

7 a)	Show that a linearly polarized wave can be obtained as the superposition of two circularly polarized waves rotating in opposite directions but at the same angular rate.

b)	Sea water at a frequency of 5 x 108 HZ has r = 1,r  = 81,  = 4.5 mhos/m.  Find the ratio of conduction current density to displacement current density at this frequency.

8 a)	What is ‘Brewster Angle’ and ‘Critical Angle’, what is significance of these angles?

b)	Derive an expression for perpendicular polarization, and obtain the ‘Brewster Angle’.

###

Code No: 211003

# Set No.

4

II B.Tech. I Semester Examination November/December, 2002

ELECTROMAGNETIC THEORY

(Electronics Instrumentation Engineering, Electronics & Control Engineering)

Time: 3 hours.								          Max.Marks: 80

Answer any FIVE questions.

All questions carry equal marks.

- - -

1 a)	Determine the electric field intensity E at a point ‘2a’ along the axis perpendicular to the plane of a circular wire charged uniformly at ‘X’ coulombs per meter, which has a radius ‘a’.

b)	Two point charges –q and +q/2 are situated at the origin and at the point (a, 0,0) respectively.  At what point does the electric field vanish?

2 a)	If J is a current density vector expressible as the curl of a vector function, what is divergence of J.

b)	Obtain by means of Laplace’s equation, the potential distribution between two concentric spherical conductors separated by a single dielectric.

3 a)	Define magnetic field intensity

b)	The magnetic field vector H has only Z component give by Hz = 3xcos+6ysin.  If it is invariant with time.  What is the expression for the current density J?

(J=curlH).

4 a)	Two long straight parallel wires, standing in air 2m apart carry currents I1 & I2 in same direction.  The magnetic field intensity at a point midway between the wires is 7.25 AT/m2.  If the force on each wires per unit length is 2.4 x 10-4evaluate current I1 & I2.

b)	Calculate approximately the energy in joule  stored in the magnetic field of a solenoid 30 cm long and 3 cm diameter wound with 1000 turns of wire when carrying a current 10 A.

5 a)	Write the integral form of Maxwell’s equations for sinusoidal time variations.  Use phasor representation.  Give word statements for the Maxwell’s equations.

b)	Let = 3 x 10-5 H/m,  = 1.2 x 10-10 F/m, and  = 0 every where.

If H = 2 cose (1010 t -  x) az (A/m), use Maxwell’s equations to obtain expressions for B, D, E, and .

Contd…

## Code No: 211003				    -2-				        Set No.4

6 a)	Define the term skin depth or depth of penetration and hence derive an expression for the same in terms of medium constants.

b)	A non-magnetic good conductor supports a plane wave traveling at a velocity of 2.5 x 104 m/sec with a wave length of 0.25 mm.  Find the frequency and conductivity.

7 a)	Prove that angle  = /4, where  is the phase angle of the intrinsic impedance of the perfect conducting medium.

b)	A plane wave has wave length of 2 cm in free space and 1.6 cm in a perfect dielectric medium.  Determine the relative permitivity of the medium.

c)	Most microwave ovens operate at 2.45 GHz.  Assume r = 600,  = 1.1 x 106 mhos/m for the strainless steel interior.  Find the depth of penetration.

8 a)	What are pointing theorem and pointing vector.  Derive the expression for average power P=E x H.

b)	A plane wave is traveling in a medium for which  = 0, r = 3, r = 1 if Epeak = 5 v/m find

- Peak pointing vector

- Average pointing vector

- Peak value of H

- Impedance of medium

###