Code No: 210302

# Set No.

1

II B.Tech I- Semester Examination November/December 2002.

## MACHANICS OF SOLIDS

( common to Mechanical Engineering, Production Engineering, Mechatronics, Metallurgy & Material Technology and Aeronautical Engineering)

Time: 3 hours								          Max. Marks: 80

Answer any Five questions

All questions carry equal marks

----

1 a)	Define and explain the terms: i) Modulus of Elasticity  ii) Modulus of Rigidity  iii) Poisson’s  ratio  iv) Bulk Modulus.

- Two vertical rods one of steel and the other of copper are each  rigidly fixed at the top and 600 mm apart.  The diameter and length of each rod are 30 mm and 375 mm respectively.  A cross bar fixed to the rods at the lower ends carries a load of 5 KN such that the cross bar remains horizontal even after loading.  Find the stress in each rod and the position of the load on the bar.  Es = 200 Gpa and Ec = 100 Gpa.

2.	A copper sleeve, 21 mm internal and 27 mm external diameter, surrounds a 20 mm steel bolt, are end of the sleeve being in contact with the shoulder of the bolt.  After putting rigid washer on the other end of the tube, a nut is screwed on the bolt until the compressive stress in the sleeve is 80 MN/m2.  Find the range of the external axial load that can be applied to this assembly if the stress in the bolt is never to be compressive and in the sleeve never to be zero.  Take Es = 200 GN/m2 and Ec = 90 GN/m2.

3.a)	How do you classify loads?  Give examples.

- A simply supported beam of length 5m carries a uniformly increasing load of 800 N/m run at one end to  1600 N/m run at the other end.  Draw the S.F. and B.M. diagrams for the beam.

4.a)	Derive the relation ship between slope, deflection and radius of Curvature of a simply supported beam.

- A 300 mm long cantilever of rectangular section 48 mm wide and 36 mm deep carries a uniformly distributed load.  Calculate the value of load w if the maximum deflection in the cantilever is not to exceed 1.5 mm.  Take E = 70X109 GN/m2.

5.a)	Derive an expression for the stresses on an oblique section of a rectangular body, when it is subjected to a direct stress in one plane only.

- The principal stresses at a point in a member subjected to two dimensional stress are 120 N/mm2 tensile.  Find the plane on which the resultant stress has maximum obliquity and magnitude of this obliquity.

Contd….2

Code No: 210302				-2-				Set No.1

6.	A solid circular shaft of diameter d has the same weight as a hollow circular shaft of mean diameter d.  Assuming the same maximum shear stress in both the cases, determine the ration of torque’s transmitted by the two shafts.  Also compare the angles of twist per unit length in these two shafts.

7.a)	Derive a relation for the hoop stress at the junction of a compound thick cylindrical shell.

- A steel cylinder is 1.5 m inside diameter and is to be designed for an internal pressure if 8 MN/m2.  Calculate the thickness if the GN/m2 and Poisson’s ration = 1/3.  Neglect any constraint due to ends.

8.	Derive the bending equation from first principles.  State the assumptions involved.

- - -

Code No: 210302

# Set No.

2

II B.Tech I- Semester Examination November/December 2002.

## MACHANICS OF SOLIDS

( common to Mechanical Engineering, Production Engineering, Mechatronics, Metallurgy & Material Technology and Aeronautical Engineering)

Time: 3 hours								          Max. Marks: 80

Answer any Five questions

All questions carry equal marks

----

1.a)	A 38 mm diameter and 450 mm long bar extends by 20 mm when subjected to an axial tension of 100 KN.  The same bar, independently, when subjected to a twisting moment of 1.27 KN-m twists through 1.9220.  Calculate the values of the four elastic constants.

- Show that if E is assumed correct, an error of 1% in the determination of N, involves an error of about 5% in calculating Poisson’s ration when its correct value is 0.25.

2.a)	Derive the relation between Bulk modulus and Modulus of rigidity.

- A rigid bar AB 9 m long is suspended by two vertical rods at its ends A and B and hangs in horizontal position by its own weight.  The rod at A is brass, 3m long, 1000mm2 area and E is 105 N/mm2.  The rod at B is steel, 5m long, 450 mm2 area and E is 2X105 N/mm2.  At what distance ‘d’ from A may a vertical load P = 5 KN be applied if the bar is to remain horizontal even after the load is applied.

3.a)	Define statically determinate and statically indeterminate beams.  Give examples.

- A cantilever beam of length 2m carries a uniformly distributed load of 2 KN/m over the whole length and a point load of 3 kN at the free end.  Draw the SF and BM diagrams.

4.a)	What is moment area method?  Explain the two Mohr’s theorems, as applicable to the slope and deflection of a beam.

- A cantilever of uniform cross-section of length/carries two point loads, W  at the free end and 2W  at a distance a from the free end.  Find the maximum deflection due to this loading.

5.a)	Obtain an expression for the difference of radii for shrinkage of a compound thick cylindrical shell.

- To measure the longitudinal and circumferential strains, strain gauges were fixed on the outer surface of a closed thick cylinder of diameter ratio 2.5.  At an internal pressure of 240 MN/m2 these strains were recorded as 9.18 x 10-5 and 36.9 x 10-5 respectively.  Determine the values of: (i) Poisson’s ratio;  (ii) Young’s modulus;  (iii) Modulus of rigidity.

Contd….2

Code No: 210302				-2-			                   Set No.2

6.a)	Derive an expression for the stresses on an oblique section of a rectangular body, when it is subjected to  direct stresses in two mutually perpendicular directions.

- A piece of material is subjected to a tensile stress of 60 N/mm2 and 30 N/mm2 at right angles to each other.  Find fully the stresses on a plane the normal of which makes an angle of 400 with the 60 N/mm2 stress.

7.	At a point in a loaded specimen the principal stresses acting on two mutually per pendicular planes are 60 N/mm2 and 40 N/mm2, both being compressive. Determine the result stress acting on a plane inclined at 60o measured clockwise to the  plane on which the larger of the normal stress is acting.

8.	A cast  iron beam is of I section having top flange of 80mm wide and 20mm thick and web of height 210mm and thickness of web is 20mm.  The bottom flange is 180mm wide and 40mm thick.  The beam is simply supported on a span of 5m.  If the tensile stress is not to exceed 20MPa, find the safe udl which the beam can carry.  Also find the max compressive stress.  Show the bending stress diagram.

- - -

Code No: 210302

# Set No.

3

II B.Tech I- Semester Examination November/December 2002.

## MACHANICS OF SOLIDS

( common to Mechanical Engineering, Production Engineering, Mechatronics, Metallurgy & Material Technology and Aeronautical Engineering)

Time: 3 hours								          Max. Marks: 80

Answer any Five questions

All questions carry equal marks

----

1.a)	State Hooke’s law.  Draw stress-strain diagram for mild steel specimen tested under uni-axial tension till fracture and mark all the salient points.

b)	A metallic rod of 1 cm diameter, when tested under an axial pull of 10 KN was found to reduce its diameter by 0.0003 cm.  The modulus of rigidity for the rod is 51 KN/mm2.  Find the Poisson’s ratio, modulus of elasticity and Bulk Modulus.

2.a)	Briefly explain thermal stresses.

b)	A bar of brass 25 mm diameter is enclosed in a steel tube 50 mm initially external diameter and 25 mm internal diameter.  The bar and tube are both initially 1.5 m long and rigidly fastened at both ends using 20 mm diameter pins.  Find the stresses in both material and the increase in length when the temperature rises from 330C to 1000C.  E for steel;  for brass = 18.7 X 10-6/0C.  Also determine the shear stress induced in the pins.

3.a)	Define shear force and bending moment.

b)	A horizontal beam AB of length 4m hinged at A and supported on rollers at B.  the beam carries inclined loads of 100N, 200N and 300N incised towards the roller support at 600, 450 and 300 respectively to the horizontal, at 1m, 2m and 3m respectively from A.  Draw the SF and BM diagrams.

4.a)	What is Macaulay’s method for finding out the slope and deflection of a beam?  Discuss the cases, where it is of a particular use.

b)	A 3 meters long cantilever is loaded with a point load of 450 N at the free end.  If the section is rectangular 80 mm (wide) x 160 mm (deep), and E = 10 GN/m2, calculate slope and deflection. (i) at the free end of the cantilever, (ii) at a distance of 0.55 m from the free end.

5.a)	Write the assumptions for solving the problems on thick cylindrical shells?

- Find the ratio of thickness to internal diameter for a tube subjected to an internal pressure when the pressure is 5/8 of the value of the maximum permissible circumferential stress.  Find the increase in internal diameter of such a tub 100mm internal diameter when the internal pressure is 100 MN/m2.  Also find the charge in the wall thickness of the tube.  Take: E = 200 GN/m2, Poisson’s ratio = 0.29.

Contd…2

Code No: 210302				-2-				Set No.3

6.	At a point in material under stress, the intensity of resultant stress on a certain plane is 60 N/mm2 (tensile) inclined 300 to normal of that plane.  The stress on a plane at right angles to this has a normal tensile component of intensity 40 N/mm2.  Find fully

(i) The resultant stress on the second plane.  (ii) The principal planes and stresses.  (iii) The plane of maximum shear and its intensity.

7.	At a point in a elastic material, a direct tensile stress of 60 N/mm2 and a direct compressive stress of 40 N/mm2 are applied on planes at right angles to each other.  If the maximum principal stress in the material is to be limited to 65 N/mm2.  Find out the shear stress that may be allowed on the planes.  Also determine the magnitude and the direction of the minimum principal stress and the maximum shear stress.

8.	A rolled steel foist of I section top and bottom flanges 160mm x 25mm are web of size 300mm x 20mm.  It is used as a simply supported beam over a span of 5m to carry an udl of 75 kN/m over its entire span.  Draw the bending and shearing stresses across a section at  (1/4)th  of span.

- - -

Code No: 210302

# Set No.

4

II B.Tech I- Semester Examination November/December 2002.

## MACHANICS OF SOLIDS

( common Mechanical Engineering, Production Engineering, Mechatronics, Metallurgy & Material Technology and Aeronautical Engineering)

Time: 3 hours								          Max. Marks: 80

Answer any Five questions

All questions carry equal marks

----

1.a)	Derive the relationship between the three moduli of elasticity.

- Show that in a prismatic bar, the maximum stress intensity due to a suddenly applied load is twice the stress intensity produced by the same load applied gradually.

2.	A compound bar consists of a circular rod of steel of diameter 20 mm rigidly fitted into a copper tube of internal diameter 20 mm and thickness 5 mm.  If the bar is subjected to a load of 200 KN, find the stresses developed in the two materials.  Take Es = 2  105 N/mm2, Ec = 1.2 X 105 N/mm2.

3.	A horizontal beam of 10m long is carrying a uniformly distributed load of 1 kN/m over the entire length.  The beam is simply supported on two supports 6m apart.  Find the position of the supports, so that the BM on the beam is as small as possible.  Also draw the SF and BM diagrams.

4.a)	Explain the procedure for finding the deflection of a beam of composite section.

- A cantilever 3 m long is of rectangular section 120 mm wide 240mm deep.  It carries a uniformly distributed load of 2.5 KN per meter length for a length of 1.5 meters from the fixed end and a point load of 1 KN at the free end.  Find the deflection at the free end.  Take E = 10 GN/m2.

5.a)	Write the relations for the hoop stress in a thick spherical shell.

- Strain gauges are fixed on the outer surface of a thick cylinder with diameter ration of 2.5.  The cylinder is subjected to an internal pressure of 160 MN/m2. The recorded strains are: longitudinal strain = 59.87 x 10-6 and circumferential strain = 240.65 x 10-6.  Determine the Young’s modulus.

6.	At a point in a piece of elastic material, there are three mutually perpendicular planes on which the stresses are as follows:  Tensile stress 50 N/mm2 and shear stress 40 N/mm2 on one plane, compressive stress 35 N/mm2 and complementary shear stress 40 N/mm2 on the second plane, no stress on the third plane.  Find

(i) The principal stresses and the positions of the planes on which they act.  (ii) The positions of the planes on which there is no normal stress.

Contd….2

Code No: 210302				     -2-				         Set No.4

7.	A mild steel column of hollow circular section is 4.5 m long with it’s ends built in.  It has to carry an axial load of 300 kN.  Determine the cross section using a factor of safety of 7.  The internal diameter is to be 0.8 times the external diameter.  Rankine’s constants for mild steel may be assumed as fc = 315 N/mm2 and a =1/15000.

8.	An unsymmetrical I-section is having top flange 80mm wide and 20mm deep.  The height of web is 250mm and its thickness is 20mm.  The bottom flange is 160mm wide and 40mm thick.  The I-section is subjected to a shear force of 50 KN.  Draw the shear stress variation diagram across the depth.

- - -