Code No. 210403

## Set No

1

II/IV B.Tech. I Semester Examination November/December 2002

SIGNALS AND SYSTEMS

(Common to Electronics & Communication Engineering Electronics & Telematics Engineering, Electronics & Control Engineering, Electronics & Instrumentation Engineering)

Time: 3 Hours							Max. Marks: 80

# Answer any Five questions

All questions carry equal marks

---

1.  a)	Obtain the Fourier components of the periodic rectangular waveform shown below:

f(t)	    A

t

-T/2		-T/4	        0		T/4		T/2

- Write a short note on Dirchlets’ condition

2.	Sketch the convolution of two signals defined as

x(t) = e-t  for 0

0  for t < 0

y(t) = - e- t for t  0

0 for t < 0

f(t) = d(t) * y(t)

3.	What is an LTI system?  Explain its properties.  What do you understand by the transfer function of an LTI system.

4.	Find the impulse response of the system shown.  Find the transfer function.  What would be its frequency response? Sketch the response.

Contd….2.

Code No. 210403				::2::				Set No.1

5.	What do you understand by the term Autocorrelation function of a signal?  What are its applications?   In what way PSD and ACF are related.

6.  a)	Use geometric evaluation from the pole-zero plot to determine the magnitude of the Fourier transform of the signal whose Laplace transform is specified as

s2 – s + 1

X(s) =  --------------		R e {s} > - (1/2)

s2 + s + 1

- Determine the Laplace transform and associated region of convergence

And pole-zero plot for the following function of time

x(t) = e-2t  u(t) + e-3t  u(t)

7.  a)	Find the inverse Laplace transform of

s + 4

----------------		Using partial fraction method

s(s-1)(s2 + 4)

- Find the Inverse Laplace transform of

s + 1

----------------

s2 + 4s +13

8.	For the sequence x(n) = 2n, n < 0

= (1/2)n, n= 0,2,4,…..

= (1/3)n, n= 1,3,5,…..

Prove that the absolute convergence region is given by (1/2) < |z| < 2

@@@@@

Code No. 210403

## Set No

2

II/IV B.Tech. I Semester Examination November/December 2002

SIGNALS AND SYSTEMS

(Common to Electronics & Communication Engineering Electronics & Telematics Engineering, Electronics & Control Engineering, Electronics & Instrumentation Engineering)

Time: 3 Hours							Max. Marks: 80

# Answer any Five questions

All questions carry equal marks

---

1.  a)	State and prove Parseval’s power Theorem.

- State and prove the convolution property in relation to Fourier transform

2.  a)	State and prove the sampling theorem.  Illustrate the effect of over sampling and under sampling.

- Determine the Fourier transform of a two sided exponential pulse x(t) = e-|t|

3.	Explain the characteristics of an ideal LPF.  Explain why it cannot be realized?

4.	Consider a gaussian filter H() = exp(-k2 + jt0)

(i)	Determine and sketch the unit impulse response of this filter.

(ii)	State, with reasons, whether this filter is physically realizable.

(iii)	Can this filter be made approximately realizable by providing a sufficient

amount of delay t0?

5.	Show that the process of modulation causes a shift of the ESD of baseband signals and the energy of the modulated signal is half the energy of the baseband signal.

6.  a)	Let g(t) = x(t) + x(-t),

Where x(t) = e-t u(t) and laplace transform of g(t) is

s

G(s) = --------		-1 < Re{s} < 1

s2-1

- Brief Explain all the properties of Laplace transform

2s + 3

7.	For a system with transfer function H(s) = --------------

s2 + 2s + 5

Find the zero state response for (i) f(t) =10u(t) (ii) u(t-5)

8.	Given X(z) = 2z(z2-1)/[z2+1], find x(n)

@@@@@

Code No. 210403

## Set No

3

II/IV B.Tech. I Semester Examination November/December 2002

SIGNALS AND SYSTEMS

(Common to Electronics & Communication Engineering Electronics & Telematics Engineering, Electronics & Control Engineering, Electronics & Instrumentation Engineering)

Time: 3 Hours							Max. Marks: 80

# Answer any Five questions

All questions carry equal marks

---

1.  a)	State and prove Rayleigh Energy theorem.

- Explain in detail with suitable examples any two property of Fourier transform.

2.  a)	Find and sketch the convolution of two signals:

x(t) = 2 ((t-5)/2) and h(t) =  ((t-2)/4).

- Find the Fourier transform of the following function: 5 Sin2 3t

3.	What do you mean by causality?  What is the relationship between bandwidth and

rise time?  What is the difference between signal bandwidth and system bandwidth?

4.	A modulated signal g (t) cos 0 t, where g (t) = a cos 1 t with 1 = 100 and  0= 106, is transmitted through the RC filter as shown in the fig.

(i)	Is this a distortionless transmission? Explain the reasons?

(ii)	What is the amplitude loss suffered by the signal?

(iii)	What is the envelop delay?

(iv)	What is the phase delay?

(v)	What is the output signal?

5.	For the following two signals find the power, and rms value, and sketch their PSD

(i)	A sin 100 t + B sin 80 t

(ii)	(A+ sin 100t) cos 200t

Contd….2.

Code No. 210403				::2::				  Set No.3

6.  a)	Obtain the Laplace transform of the square wave of unit amplitude and periodic time 2T.

- Obtain the Laplace transform of the periodic rectified half sine wave.

7.  a)	Explain Time differentiation and Time Integration properties of Laplace Transform.

b)	Find the Laplace transform of Cos(wt) and Sin(wt) using frequency shifting property.

8.	Determine the energy and average power of the following signals

(a)	(n)	          			(b)	u(n)

(c) 	ramp function r(n) for n 0	(d)	A exp(j0n)

@@@@@

Code No. 210403

## Set No

4

II/IV B.Tech. I Semester Examination November/December 2002

SIGNALS AND SYSTEMS

(Common to Electronics & Communication Engineering Electronics & Telematics Engineering, Electronics & Control Engineering, Electronics & Instrumentation Engineering)

### Time: 3 Hours							Max. Marks: 80

# Answer any Five questions

All questions carry equal marks

---

1.  a)	Given the set of functions shown below

- If x(t) = Cos2t 0 t  1, Find the Fourier coefficients and hence find y(t) such that integral square error is minimized.

- Find y(t) for x(t) = Sin 2t, 0 t  1

2.  a)	State and Prove Modulation theorem.

- Using the modulation theorem find out the Fourier transform of RF pulse given as

y(t) = A rect (t / ) cos 2 fct.

3.	Show that the mean square value r the power of the signal

A cos(w0t +1) + A cos(w0t + 2) is A2 + A2 cos (1-2).  What do you mean by Physical reliability of a network.

4.	Explain the difference between a time invariant system and time variant system?  Write some practical cases where you can find the systems.  What do you understand by the filter characteristics of a linear system?  Explain the condition of causality?

5.	The transfer function of a system is given by H(w)=k.  Where k is a constant.  Sketch the magnitude and phase function of this transfer function.  Evaluate the impulse response of this filter.  Sketch this response and state whether the filter is physically realizable.

6.  a)	Explain the relationship between Laplace transform and Fourier transform with An example

- Given f(t) = e-10t  u(t) Check for the existence of Laplace transform only or

Both Laplace and Fourier Transform.

7.	Use method of partial fraction to find the time signal corresponding to following

s						   2s - 1

(i)	X(s) = ----------------			(ii)	X(s) = ----------------

(s2 + 5s + 6)					 s2 + 2s + 1

8.	Given X(z) = z / [z-1] 3, find x (n) using contour integration method.

@@@@@