Code No: 210653

OR

II B.Tech I Semester Examination November/December 2002

## FURNACE TECHNOLOGY & PYROMETRY

(Metallurgy & Material Technology)

# Time: 3 hours							       Max. Marks: 70

Answer any Five questions

All questions carry equal marks

---

1 	Describe characteristic features of Arc furnaces and of tube resistance furnaces. Identify some uses for which these furnaces are better suited than other types of

Furnaces.

2	Define the efficiency of operation of a furnace.  Indicate the sources of heat loss in a furnace and also indicate steps that can be taken to minimize these losses.

3 a)	What is the 'Seebeck effect' and how is it used to help measure temperature?

b)	What is a thermopile?  What is the advantage of such an arrangement?

4 a)	What are resistance thermometes?

- What are the disadvantages in the use of typical resistance thermometers?

- Why are compensating bridge circuits required while using resistance thermometers?

5 a)	What behavior must a body display to be classified as a black body?

b)	From the point of view of emission, what type of body are most solids and liquids to a reasonable approximation?

6	Describe the principle and construction of a pyro-optical-pyrometer. Identify some situations where it will be the convenient way to measure temperature.

7	Derive an expression for the steady state conduction of heat perpendicular to the walls of a hollow sphere.

8	Water at the rate of 75 Kg/min is heated from 40°C to 80°C by an oil having a specific heat of 1.9 KJ/Kg °C.  The fluids are used in a counter flow heat exchanger and the oil enters the exchanger at 120° C and leaves at 80° C.  The overall heat transfer coefficient is 320 W/m2 ° C.  Calculate the heat exchanger area.

###