Code No:210204

Set No

1

II B.Tech. I Semester Examination November/December,2002

ELECTROMAGNETIC FIELDS

(Electrical and Electronics Engineering)

Time: 3 hours.									Max.Marks:80

Answer any FIVE questions.

All questions carry equal marks.

- - -

1.a)	State and explain Coulomb’s law.

b)	Four like charges of 30 μc each are located at the four corners of a square the diagonal of which measures 8m.  Find the force on a 150 μc charge located at 3m above the center of the  square.

2.a)	Show that E= - ∆V

b)	Derive the expression for electric field intensity due to a dipole.

3.a)	Derive ohms law in point form from first principles.

b)	In a cylindrical conductor of radius 2mm, the current density varies with the distance from the axis according to

J = 103 e-400r A/m2

Find the total current.

4.	State Biot Savart’s law.  Use the same to find an expression for the magnetic field intensity due to a long current carrying conductor.

5.	Find out the most general expression for the field intensity at the centre of a square current loop in XY plane.

6.	A solenoid of 500 turns has a length of 50cm and radius of 10 cm.  A steel rod of circular cross section is fitted in the solenoid co-axially and tightly.  Relative perameability of steel is 3000.  A d.c. current of 10 Amps is passed through the solenoid.  Compute inductance of the system, energy stored in the system and mean flux density inside the solenoid.

7.a)	Write down Maxwell’s equations in their general integral form, derive the corresponding equations for fields varying harmonically with time.

b)	Show that the ratio of the amplitude of the conduction current and displacement current density is σ for the applied field E = Emax Cos wt V/m.

ωE

8.a)	Show that the electric field E induced by a time varying magnetic field B is given by the expression.

XE = -∂B/∂t

b)	A plane electromagnetic wave traveling in free space has amplitude of Eo equal to 50 μV/m and the electric field vector at any point varies sinusoidally with time.  What are the peak and average values of pointing vector?

******

Code No:210204

Set No

2

II B.Tech. I Semester Examination November/December,2002

ELECTROMAGNETIC FIELDS

(Electrical and Electronics Engineering)

Time: 3 hours.									Max.Marks:80

Answer any FIVE questions.

All questions carry equal marks.

1.a)	State and prove Gauss’s  law.

b)	Derive and plot the electric field intensity of a spherical volume distribution of charge using Gauss’s law.

2.	A uniform charge density of  v c/m2 exists throughout the volume of a sphere of radius  b meters.  Using Poisson,s equation, find the value of electric field intensity and potential at any point inside the sphere for which 0< = r < = b.

3.a)	Derive the expression for energy stored in concentric sphere.

b)	The co-axial cable is required too transmit electrical power.  The P.d. between the inner and outer conductors is to be filled mainly with nitrogen gas under pressure whose dielectric strength is 25* 10-6 volt/meter.  The radius of outer conductor is double that of inner conductor.

(i)	Determine the capacitance of the Cable, and

(ii)	Determine the energy stored in the electric field of this cable when P.d. is 2*105 volts

4.	Derive an expression for the magnetic field intensity at a distance R from a long straight conductor carrying current I.  State the basic law used.

5.	Derive an expression for the torque on a current loop placed in a magnetic field.

6.	Current in a coil is increased from zero to 10 Amps at a uniform rate in 5 seconds.  It is found that this coil develops self-induced. e.m.f. of 100 volts where as an e.m.f.of 20 valts is produced in a nighbouring coil.Compute self inductance of the first coil and mutual induatance between the two coils.

7.a)	State and Prove the Poynting theorem for conducting medium.

b)	A plane-traveling wave in free space has an average pointing vector of 1.5 watts/m2.  Calculate the average density.

8.a)	Explain the Pysical interpretation of Maxwell’ equations.

b)	A square coil rotates at a constant sped of 500 rpm about an axis perpendicular to a stationary uniform field of magnetic induction 0.75 Tesla.  The coil has mean dimensions of 15 cm by and is wound with 100 turns.  Determine the dynamically induced emf in the coil when the plane of the coil is

(i)	In the same plane as the field (ii)	At right angles to the field, and

- Inclined at 30 to the field.

*****

Code No:210204

Set No

3

II B.Tech. I Semester Examination November/December,2002

ELECTROMAGNETIC FIELDS

(Electrical and Electronics Engineering)

Time: 3 hours.									Max.Marks:80

Answer any FIVE questions.

All questions carry equal marks.

- - -

1.	Four concentrated charges Q1=.3 μc,Q2=.2 μc,Q3= -.3 μc,Q4 = .2 μc are located at the vertices of a plane rectangle.  The length of rectangle is 5cm and breath of the rectangle is 2 cm.  Find the magnitude and direction of resultant force on Q1.

2.	Calculate the capacitance of a parallel plate capacitor with following details.

Plate area = 100 sq.cm.

Dielectric Єr1 = 4,d12 = 2mm

Dielectric Єr2 = 3,d12 = 3mm

If  200 V is applied across plates what will be the voltage gradient across each dielectric.

- A squares parallel plate capacitor 200mm on side with a plate spacing of 25 mm is filled with a dielectric slap (Єr = 240 of the same dimensions if 100V  is applied to the Capacitor find.

  - the polarization P in the dielectric and

  - the energy stored by the capacitor. If the voltage source is now disconnected and the dielectric slap then slipped out from between the plates, find

  - Polarization in the dielectric

  - Energy stored in the dielectric and

  - Energy stored in the capacitor

4.a)	Derive an expression for the force between two current carrying conductors in the same direction.

b)	Describe any two applications of Ampere’s law

5.	A uniform Solenoid 100mm in diameter and 400mm long has 100 turns of wire and a current of I=3A. Find the magnetic field B on the axis of the solenoid (a)  at its centre, (b) at one end, & (c) half way from the centre to one end.

6.	A solenoid of 500 turns has a length of 50 cm and a mean radius of 10cm. It is air cored it carries a d.c. current of 10 Amps. Find vector magnetic potential at the centre of the solenoid and also at the end of the solenoid. State approximations if any.

Contd…2.

Code No:210204			        -2-				Set No.3

7.a)	Explain clearly what is meant by intrinsic impedance of a medium and derive an expression for the intrinsic impedance of a dielectric medium?

b)	A square loop of wire 25 cm by 25 cm has a voltmeter (of infinite impedance) connected in series with one side. Determine the voltage induced by the metre when the loop is placed in an alternating field, the maximum intensity of which is 1 AMP per metre. The plane of the loop is perpendicular to the magnetic field varying at a frequency of 10 MHz.

8.a)	Prove that in dimensional case the wave equation for the electric field in free space is given by

∂2Ex		1 	 ∂2Ex

-----      =	---	-----

∂Z2		 V2  	 ∂t2

- A sinusoidal plane wave is transmitted through a medium whose break down strength is 30 kV/m and whose relative permeability is 4. Determine the mean possible RMS Power flow density and the peak value of the associated magnetizing force.

- x-

Code No:210204

Set No

4

II B.Tech. I Semester Examination November/December,2002

ELECTROMAGNETIC FIELDS

(Electrical and Electronics Engineering)

Time: 3 hours.									Max.Marks:80

Answer any FIVE questions.

All questions carry equal marks.

- - -

1.a)	State and explain and the application of Gauss’s law.

b)	Electric potential in an electric field is given by

v(x,y,z) = -3x2yz

Compute the electric field intensity as a function of x,y and z coordinates. Derive the formula used.

2.a)	A uniform sheet of charge with is s = (-1/3∏) nc/m2 is located at z=5 m and a uniform line charge with 1 = (-25/9) nc/m is located at z = -3m, yk=3m. Find the electric field E  at (0, -1,0).

b)	Show that the electric field E outside a spherical shell of uniform charge density s is the same as E due to the total charge on the shell located at the center.

3.a)	Derive polarization vector P =D -  Єo   E and hence deduce to

Applied electric field (Ea)

E =  --------------------------------------

Єr

b)	The parallel plates capacitor are 0.05mapart and  are charged to a surface density of 25* 10-6 c/m2. The dielectric constant of 0.002 m layers of Єr = 3 and that of 0.003m layer has Єr = 4. Calculate E  and D for each dielectric and also polarization of each plate.

4.	Derive Maxwell’s Second & Third equation from the basic principles in both Integral & point forms.

5.	Derive an Expression for the force experienced by an electron moving in a static magnetic field.

Contd…2

Code No:210204			        -2-				Set No.4

6.	Two mutually coupled coils are connected in series.

L1 = 0.5 H	L2 = 0.6 H    M = 0.1 H

A d c current of 2 Amp.  Is passed through their system in such a way that the current increase at a uniform rate of  1 Amp. Per Sec. What  is the voltage developed  across the end point, if (a) the coils are connected in a magnetically aiding condition (b) the coils are connected in a magnetically opposing condition. Derive formula used.

7.a)	Explain the concept of traveling electromagnetic waves. Obtain the velocity of wave propagation over a transmission line having an inductance of L henry/m and capacitance of C farad/m.

b)	A single turn rectangular loop with enclosed area of 1 square metre is situated in air with its plane normal to a magnetic field which varies at the rate of 1 Tesla/Sec. estimate the EMF induced in the loop.

8.a)	Starting form first principles derive Maxwell’s equation using ampere’s law and derive these equations for harmonically varying fields.

b)	A circular copper ring has aradius of  0.1 metre and a circular cross section of radius 10-3 metre is situated in a time varying magnetic field. The magnetic flux is normal to the plane of the ring and changes at the rate of 4 x 10-3 Tesla/Sec. The conductivity of copper is 5.8 x 107 mho/metre. Determine the current in the ring.

- x -