Code No : 210303

Set No:

1

II B.Tech- I Semester Examination November/December 2002

# ELECTRICAL ENGINEERING

(Common to Mechanical Engineering, Chemical Engineering, Production Engineering, Mechatronics, Metallurgy & Material Technology, Aeronautical Engineering)

Time: 3 hours						                               Max. Marks: 80

Answer any FIVE questions.

All Questions carry equal marks.

---

1 a)	Briefly explain the principle of operation of dc generator .

- Calculate the flux in a 6-pole dc generator with 780 armature conductors, generating 500v, when running at 1000 pm if the armature is  (i) lap wound (ii) wave wound.

- Calculate the emf generated by a 6-pole wave wound armature having 45 slots with 18 conductors.  The flux per pole is 0.025 wb.  The armature is driven at 1000 rpm.

2 a)	Explain DC motor principle and its working

b)	A 250 V shunt motor on no-load runs at 1000 rpm and takes 5A.  The total armature and shunt field resistances are 0.2 and 250 respectively.  Calculate the speed when loaded and taking current of 50A if armature reaction weakers the field by 3%.

3 a)	Explain the construction, and working principle of single phase transformer.

- The efficiency of a 500 KVA, single phase transformer is 98%, when delivering full-load at 0.8 p.f., leading and 99% at Half-full-load at unity power factor calculate

i)	Iron losses		ii)	Full load copper losses

4 a)	Give the principle of Operation (or working principle) of 3-phase Induction motor

b)	A 4-pole, 3-phase Induction motor runs at a Speed of 1.440 rpm.  On 500V ; 50 Hz mains.  The Mechanical Power developed by the rotor is 20.3 H.P.  The Mechanical losses are 2.23 HP Calculate.   a) The slip       b) Rotor Copper losses c) The efficiency.

5 a)	Why is spring control to be preferred to gravity control in an electrical measuring instrument?

b)	The coil a moving coil meter has resistance of 5 and given full scale deflection when a current of 15mA passes through it.  What modification must be made to the instrument to convert it into? (i) An ammeter reading to 15A (ii) a voltmeter reading to 15V?

Conted….2

Code No: 210303 				2			        Set No. 1

6 a)	Show that average power in a purely inductive circuit is zero.

- A 60Hz voltage at 230V effective value is impressed on an inductance of 0.265H.

i) write the time equation for the voltage and the resulting current,  Let the zero axis of the voltage wave at t=0. (ii) Show the voltage and current on a phasor diagrams and  (iii) find the maximum energy stored in the inductance.

7 a)	What is meant by Active power, reactive power and apparent power.

- A coil of R=4, XL = 3 connected in series with a condenser of R=8, XC=8, this combination is connected to 4 resistor.  Across this series circuit a voltage 1000 is applied. Calculate (i) current (ii) voltage drops across coil, condenser and resistor and (iii) power absorbed by the circuit.  Take voltage vector along the reference axis.

8 a)	State and explain Kirchoff’s Laws.

- Determine the current required in a magnetizing winding having 300 terms  wound on a magnetic care shown in figure.

Figure  1

Mean dia of the ring = 20 cms.  Relation Permeability of the material is 500.  Magnetic flux to be established in air gap is 0.5 wb.

###

Code No : 210303

Set No:

2

II B.Tech- I Semester Examination November/December 2002

# ELECTRICAL ENGINEERING

(Common to Mechanical Engineering, Chemical Engineering, Production Engineering, Mechatronics, Metallurgy & Material Technology, Aeronautical Engineering)

Time: 3 hours						                               Max. Marks: 80

Answer any FIVE questions.

All Questions carry equal marks.

---

1 a)	Derive an expression for the emf induced in a d.c generator.

b)	Calculate the value of emf generated in an 8-pole lap wound generator if it  is rotated at 250 rpm.  The flux per pole is 0.05 wb and the number of armature conductors is 960.

- A 6-pole wave connected armature has 250 conductors and runs at 1200 revolutions per minute.  The electromotive force generated is 600v.  Find the useful flux per pole.

2 a)	Explain the significance of back emf in a dc motor and explain how it controls the value of armature current in a dc motor.

b)	A 250v shunt motor takes a total current of 20A.  The shunt field and armature resistances are 200 and 0.3 respectively.  Determine (i) value of back emf (ii) gross mechanical power in the armature.

c)	A 230V motor has as armature circuit resistance of 0.6.  If the full load armature current is 30A and no-load armature current is 4A, find the change in back emf from no load to full load.

3 a)	Draw, and explain the phase or Diagram of a practical single phase transformer supplying logging load.

b)	A 4 KVA, 200/400V, 50 HZ, single phase transformer gave the following test results.

No-load Test:		200V,		0.7A,	       60W. (L.V. Side)

S.C. Test 		  9V,		  6A,	        21.6W (H.V Side)

Find the efficiency, and voltage regulation of the Transformer on full load at 0.9 P.F. Lagging.

4 a)	Draw, and Explain the Torque-slip characteristics of 3-phase Induction motor, showing clearly the starting Torque, maximum Torque, and normal operating region.

b)	A 3-phase induction motor, having start-connected rotor, has an induced emf of 80V, when the slip-rings are stand-still on open circuit.  The rotor has a resistance, and reactance per phase of 1 ohm	, and 4 ohms respectively.  Calculate the current per phase, and P.F when (a) slip-rings are short circuited, and  (b) slip rings are connected to a star-connected rheostat of 3 ohms per phase.

Contd…2

Code No: 210303				-2-				Set No.2

5 a)	Draw a diagram to show the essential parts of a modern moving coil instrument.  Label each part and state its function.

b)	A moving coil millimeter has a coil of resistance 15 and full scale deflection is given by a current of 5mA.  This instrument is to be adapted to operate (i) as a voltmeter with a full scale deflection of 100A  (ii) as a ammeter with a full scale deflection of 2A.  Sketch the circuit in each case.

6 a)	Show that average power in a pure capacitor is zero.

- A 50Hz voltage of 230V effective value is impressed on a capacitance of 26.5F.

i) Write the time equation for the voltage and the resulting current.  Let the zero axis of the voltage wave be at t=0. ii) Show the voltage and current on a time diagram  iii) show the voltage and current on a phasor diagram  iv) find the maximum energy stored in the capacitance.  Find the relative heating effects of two currents waveforms of equal peak value, the one sinusoidal and the other rectangular wave form.

7 a)	A 230V, 50Hz Single phase supply is feeding following loads which are connected across it

- a motor load of 4KW, 0.8 lagging p.f.

- a rectifier of 3KW at 0.6 leading p.f.

- a lighter load of 10KVA unity p.f.

- a pure capacitive load of 8KVA

Determine total KW, total KVAR.

b)	A coil of resistance 15 and inductance of 0.05H is connected in parallel with a non-inductor resistance of 20.  Find (i) the current in each branch (ii) the total current (iii) the phase angle of whole arrangement for a applied voltage of 200V at 50Hz.

8 a)	Derive the star-delta transformation for a resistance network.

- State and explain Faraday’s law of electromagnetic induction. Distinguish between self and mutual induced emfs.

###

Code No : 210303

Set No:

3

II B.Tech- I Semester Examination November/December 2002

# ELECTRICAL ENGINEERING

(Common to Mechanical Engineering, Chemical Engineering, Production Engineering, Mechatronics, Metallurgy & Material Technology, Aeronautical Engineering)

Time: 3 hours						                               Max. Marks: 80

Answer any FIVE questions.

All Questions carry equal marks.

---

1 a)	With neat sketches, explain the construction and functions of the various parts of a d.c. machine.

b)	Calculate the emf.  Generated by a 6 pole lap wound armature with 65 slots and 12 conductors per slot, when driven at 1000 rpm.  The flux/pole is 0.02 wb.

2 a)	Give the voltage equation and power equation of a dc motor.

- A 20 KW, 250V dc shunt generator has armature and field resistances of 0.1 and 125 respectively.  Calculate the total armature power developed when running  (i) as a generator delivering 20 KW output (ii) as a motor taking 20KW input.

3 a)	Derive the condition for maximum efficiency and state the different losses in case of a Transformer.

b)	3,300/ 240V, Single-Phase transformer is supplied at 249V on no-load on L.V side. It takes 2A, and input power is 60 W. if the resistance on L.V winding is 0.8 ohm, find (i) magnetizing current  (ii) active current (iii) power factor (iv) copper loss in primary (L.V side (v) core loss in transformer.

4 a)	“An induction motor cannot run at synchronous speed”. Give reason

b)	A 4-pole, 50Hz, 3-phase induction motor runs at 1440 rpm with a load torque of 200 N-M.  If the stator loses are 2,200 W, and the friction torque is 5 N-M. Calculate its efficiency, and the rotor copper loss.

5 a)	Describe with the diagram the construction of a repulsion type moving iron instrument with particular reference to the means used for (i) deflection (ii) control (iii) damping.

b)	A moving iron voltmeter in which full scale deflection is given by 100V, has a coil of 10000 turns and resistance 2000. Calculate the number of  turns required on the coil of the instrument is converted for use as an ammeter reading 20A full scale deflection.

Conted….2

Code No: 210303 				-2-	`		Set No.3

6 a)	A voltage of V =Vm Sint is impressed across R-L-C series circuit.  Find the expression for current and power.

b)	A resistance of 20, an inductance of 0.2H and a capacitance of 100F are connected in series across 220V, 50Hz mains.  Determine the following (i) impedance (ii) current (iii) voltage across R,L and C (iv) power in watts and VA and (v) p.f. and angle of lag.

7 a)	Why the inter connection is needed in a 3. system? Explain about phase sequence.

b)	Three similar coils, each having a resistance of 20 and an inductance of 0.05H all connected in (i) star (ii) mesh to a 3-phase, 50Hz supply with 400V between lines. Calculate the total power absorbed and the line current in each case.  Draw the vector diagram of current and voltages in each case.

8 a)	Define self-inductance mutual inductance and coefficient of coupling. Derive the relation ship between them.

b)	Two identical coils when connected in series gave a total inductance of 40 mH.  When one of the Coils is reversed the total inductance is 10mH.  Determine the self and mutual inductance and co-efficient of coupling.

###

Code No: 210303

Set No:

4

II B.Tech- I Semester Examination November/December 2002

# ELECTRICAL ENGINEERING

(Common to Mechanical Engineering, Chemical Engineering, Production Engineering, Mechatronics, Metallurgy & Material Technology, Aeronautical Engineering)

Time: 3 hours						                               Max. Marks: 80

Answer any FIVE questions.

All Questions carry equal marks.

---

1 a)	What are the advantages and disadvantages of using large number of poles in a d.c. machine?

b)	An 8-pole lap wound d.c. generator has 120 slots having 4 conductors per slot.  If each conductor can carry 250H and if flux/pole is 0.05 wb, calculate the speed of the generator for giving 240V on open circuit.  If the voltage drops to 220v on full load, find the rated output of the machine.

2 a)	Derive an expression for the torque developed N-M and in kg-m in a dc motor.

b)	Determine the value of torque in kg-m developed by the armature of a 6-pole wave-wound motor having 492 conductors, 30 mwb per pole when the total armature current is 40A.

3 a)	Explain in detail the O.C., and S.C., tests of single phase transformer and their use to find regulation of a Transformer.

b)	Open circuit and short circuit tests on a 5 KVA, 200V/40V, 50 HZ 1-phase Transformer gave the following Tests.

O.C. Test:	200 V		1 A		100 W		(L.V. Side)

S.C. Test:	15   V		10A		 85 W		(H.V. Side)

- Draw the equivalent circuit referred to Primary

- Calculate the approximate regulation of the Transformer of 0.8 P.F. lag and leading.

4 a)	Explain the construction of squirrel-cage, and phase-wound induction motors.

Mention their applications.

b)	A 3-phase induction motor has 0.06 ohm rotor resistance, and 0.3ohm standstill resistance per phase.  Find the additional resistance required in the rotor circuit to make the starting torque equal to the maximum torque of the motor.

5 a)	Explain the principle of operation of one type of moving iron instrument showing how it is suitable for use on DC and AC systems.

b)	The total resistance of moving iron voltmeter is 1000 and the coil has an inductance of 0.765 H.  the instrument is calibrated with a full scale deflection 50V DC.  Calculate the percentage error when the instrument is used on (i) 25 Hz supply (ii) 250 Hz supply, the applied p.d being 50V in each case.

Contd…..2

Code No: 210303 				-2-				Set No.4

6 a)	Voltage of V=Vm Sint is impressed across R-C series circuit.  Give the expression for current and power.

b)	A tungsten filament bulb rated at 500W, 100V is to be connected to series with a capacitance across 200V, 50Hz supply.  Calculate (i) the value of capacitor such that the voltage and power consumed by the bulb are according to the rating of the bulb (ii) the power factor of the current drawn from the supply and (iii) draw the phasor diagram of circuit.

7 a)	Deduce the relation between line voltage, line current and phase voltages, phase currents in a delta connected network.

b)	A three phase 400V, 50Hz a.c. supply is feeding a three phase delta connected load with each phase having a resistance of 25 an inductance of 0.15H and a capacitance of 120F in series.  Determine the line current, volt-amp, active power and reactive volt-amp.

8 a)	Derive the expression for force on a current carrying conductor placed in a magnetic field.

b)	Define self and mutual inductance. What is coefficient coupling. Derive the relation between them.

###