Code No. 211001

## Set No

1

II/IV B.Tech. I Semester Examination November/December 2002

ELECTRICAL TECHNOLOGY

(Common to Electronics & Instrumentation Engineering, Bio-Medical Engineering, Electronics and Control Engineering)

Time: 3 Hours							Max. Marks: 80

# Answer any Five questions

All questions carry equal marks

---

1.  a)	State the reasons for a self exited DC shunt generator to fail to build up voltage, and suggest the necessary modifications.

- Explain the importance of critical field resistance and how it can be determined

- A 4-pole generator having wave-wound armature winding has 51 slots each slot contains 20 conductors.  What will be the voltage generated in the machine when driven at 1,500 rpm assuming the flux per pole to be 7.0 m Wb?

2.  a)	From the first principles, derive an equation for torque developed in a D.C. Motor.

- A 20kw, 250V d.c. shunt generator has armature and field resistances of 0.04 and 200 respectively.  Determine the total armature power developed when working i) as generator delivering 20kw output and   ii) as a motor taking 20kw input.

3.  a)	Explain with phasor diagram, how the flux in the transformer core remains fairly constant from no-load to full load assuming lagging power factor.

- The no-load current of a transformer is 15A at a power factor of 0.2 when connected to a 460 v, 50c/s supply.  If primary winding has 550 turns.  Calculate

  - Magnetizing component of no-load current

  - The iron loss

  - The maximum value of flux in the core.

4.  a)	Develop from fundamentals, the equivalent circuit representation of a single phase 2 winding Transformer.

- The maximum efficiency of a 500 KVA, 3300/500v, 50 HZ, single phase transformer is 97% and occurs at 75% of full load at U.P.F.  If the impedance is 10%.  Calculate the regulation at F.L. at 0.8 P.F. lag.

5.  a)	Explain the principle of rotating magnetic field and hence prove it is of constant magnitude and rotates at synchronous speed.

b)	A 3-phase, 50 Hz induction motor has a full-load speed of 1440 r.p.m.   For this motor, calculate the following (i)  number of poles (ii)  full-load slip and rotor frequency  (iii)   speed of stator field with respect to (a)   stator structure and      (b) rotor structure and (iv)   speed of rotor field with respect to (a)   rotor structure  (b) stator structure and (c) stator field.

Contd…..2.

Code No. 211001			::2::					Set No: 1

6.  a)	Why a 3-phase induction motor draws higher no load current compared to transformer

- The starting current of a delta-connected 3-phase induction motor at rated voltage is 5 times the full-load current and the slip at full load is 5%.  The no-load current is negligible.  (i) If an auto-transformer starter is used to limit the starting current from mains to 2 times the full-load current, estimate the starting torque then obtained as a percentage, of he full-load torque.  (ii) Neglecting stator impedance, determine the slip at which maximum torque occurs in the motor.

7.  a)	Define voltage regulation of an alternator.  Explain synchronous impedance method of determining regulation of an alternator.

- Calculate the voltage induced per phase in a 3phase 50 Hz, alternator having a flux per pole of 0.1515 wb.  The no. of conductors in series are 360.  Assume full pitch coil with a distribution factor of 0.96.

8.  a)	Explain the operation of a single phase induction motor on the basis of double revolving field theory.

- Draw a typical torque-speed curve of a single phase induction motor on the basis of double revolving field theory.

@@@@@

Code No. 211001

## Set No

2

II/IV B.Tech. I Semester Examination November/December 2002

ELECTRICAL TECHNOLOGY

(Common to Electronics & Instrumentation Engineering, Bio-Medical Engineering, Electronics and Control Engineering)

Time: 3 Hours							Max. Marks: 80

# Answer any Five questions

All questions carry equal marks

---

1.  a)	Why brushes and commutator are necessary for operating of a D.C. Machine.

b)	How D.C. Generators are classified?

c)	The armature of a 6-pole d.c. generator has a wave winding containing 664 conductors.  Calculate the generated e.m.f. when flux per pole is 0.06 Weber and the speed is 250rpm.  At what speed must the armature be driven to generate and e.m.f. of 250V if the flux per pole is reduced to 0.058 Weber?

2.  a)	With the help of neat sketches, explain torque-speed characteristics of he following D.C. Motors (i) D.C. Series (ii) D.C. Shunt

b)	Find the torque in N-m exerted by a 4-pole series motor whose armature has 1200 conductors connected up in a 2-circuit winding.  The motor current is 10A and the flux per pole is 0.02 Wb.

3.  a)	State and prove the condition for maximum efficiency of a transformer.

b)	In a 25 KVA, 2000v/200v Transformer, the iron and copper Losses are 350 and 400 W respectively.  Calculate the Efficiency on unity power factor (a) full load (b) determine the maximum efficiency and the load in KVA at which it is maximum.

4.  a)	Discuss the vector diagram analysis of a transformer on load with Resistance & Inductance’s in winding for (i) Resistive (ii) Inductive and (iii) Capacitive loading.

b)	A 660/220v, single phase Transformer takes a load current of 2A at a P.F. of 0.2 lag.  The transformer supplies a load of 30a at a P.F. of 0.8 lag.  Calculate the current drawn from the mains neglecting the winding resistance and reactance.  Also find the primary power factor.

5.  a)	With a neat sketch, explain the construction of a 3-phase induction motor.

b)	3-phase, 4-pole, 1440 rpm, 50Hz induction motor has star-connected rotor winding, having a resistance of 0.2  per phase and a standstill leakage reactance of 1  per phase.  When the stator is energized at rated voltage and frequency, the rotor-induced e.m.f. at standstill  is 120V per phase.

(i)  Calculate the rotor current, rotor power factor and torque both at starting

and at full load and compare these results.

(ii) If an external resistance of 1 ohm per phase is inserted in rotor circuit,

calculate rotor current, rotor power factor and torque at the time of starting

Contd…..2.

Code No. 211001			::2::					Set No: 2

6.  a)	Compare the relative advantages and disadvantages of cage rotor and wound rotor induction motor of the same power rating.

b)	A 3-phase squirrel cage induction motor has a rotor resistance of 0.3 per phase and a standstill rotor reactance of 2.6 per phase.  The ratio of maximum torque to full-load torque is found to be 3.  Determine the ratio of actual starting-torque to full-load torque to full-load torque for star-delta starting.  State the assumptions made in your calculations.

7.  a)	Draw a neat sketch showing the various parts of a synchronous machine and explain each part briefly.

b)	A 3 ph. 50 Hz. 20 poles Salient pole alternator with star connected stator winding has 180 slots on the stator.  Each slot consists of 8 conductors.  The flux per pole is 25 mwb and is sinusoidally distributed.  The coils are full pitch.  Calculate (i) the speed (ii) the generated e.m.f. per phase and (iii) the line e.m.f.

8.  a)	Discuss the differences  between capacitor start, capacitor start.  Capacitor run and permanent split capacitor motors.

b)	A small motor has an output torque of 0.25 N.m and a speed of 100 rad/sec.  If the input current is 0.6 A at 230 V and 0.6 lagging p.f. find (i) output power in watts (ii) efficiency.

@@@@@

Code No. 211001

## Set No

3

II/IV B.Tech. I Semester Examination November/December 2002

ELECTRICAL TECHNOLOGY

(Common to Electronics & Instrumentation Engineering, Bio-Medical Engineering, Electronics and Control Engineering)

Time: 3 Hours							Max. Marks: 80

# Answer any Five questions

All questions carry equal marks

---

1.  a)	Explain the constructional features of a D.C. Machine with a help of a neat sketch.

b)	Name the main parts of a D.C. Machine and state the materials of which each part is made.

- Explain different methods of excitation of D.C. Generators  with suitable diagrams.

2.  a)	What are all the various losses in a D.C. Machine?

b)	Derive an expression for efficiency of a D.C. Machine.

c)	A 6-pole, 250V series motor is wave-connected.  There are 240 slots and each slot has 4 conductors.  The flux per pole is 1.75 X 10-2   Wb when the motor is taking 80A.  The filed resistance is 0.05, the armature resistance is 0 .1 and the iron and frictional loss is 0.1kw.  Calculate i) Speed,  ii) bhp and  iii) shaft torque.

3.  a)	Discuss in detail the effects of resistance and leakage reactance of the winding on the performance of a Transformer.

b)	A 50 KVA, 2000/200v, 50 HZ single phase Transformer has impedance drop of 8 % and resistance drop of 4%.  Calculate the regulation of the transformer at F.L when the load P.F. is 0.8 lag.  Also determine the P.F. at which the regulation will be maximum.

4.  a)	Draw the vector diagram for a Transformer when it is loaded with                  (i) Inductive load   (ii) Capacitive load.

b)	A 30 KVA, 2400/120v, 50HZ, transformer has a high voltage winding resistance of 0.1 ohms and leakage reactance of 0.22 ohms.  The low voltage winding resistance is 0.035ohms and the leakage reactance of 0.012 ohms.  Find the equivalent winding resistance, reactance and impedance referred to the  (i) High voltage side  (ii) Low voltage side.

Contd……2.

Code No. 211001			::2::					Set No: 3

5.  a)	With usual notations deduce the expression for running torque of a 3-phase induction motor.

b)	A 3 phase, 50Hz, 6-pole induction motor has a shaft output of 10 kW at 930 r.p.m. Friction and windage loss amount to 1% of output.  Total stator losses are 600 W.  (i) Determine the rotor input and stator input.  (ii) If maximum torque is developed at 800 r.p.m. compute the starting torque with rated-voltage starting.

6.  a)	Explain why an induction motor at no load operates at a very low power factor.

b)	A 50 kVA, 440 V, 3-phase, 50 Hz induction motor is provided with a 3-phase step-down autotransformer starter which steps down the voltage to 60% of the input.  Given that the starting current of the motor on rated voltage is 6 times the rated full-load current, estimate the current drawn by the autotransformer from the mains at starting.  What is the starting kVA drawn by he autotransformer.

7.  a)	The open circuit and short circuit tests data on a 3 phase, 1MVA, 3.6KV, star connected synchronous generator is given below:

If(A) :			60	70	80	90	100	110

Voc(line) (V):		2560	3000	3360	3600	3800	3960

Isc(A) :			180

Find the regulation at full load, 0.8 p.f. lagging.

b)	Explain why a synchronous motor doesn’t have self-starting torque.  Explain one method of starting a synchronous motor.

8.  a)	Describe the construction and working principle of shaded pole induction motor.

b)	Give typical applications of capacitor star, split, phase, permenant capacitor, shaded pole induction motors.

@@@@@

Code No. 211001

## Set No

4

II/IV B.Tech. I Semester Examination November/December 2002

ELECTRICAL TECHNOLOGY

(Common to Electronics & Instrumentation Engineering, Bio-Medical Engineering, Electronics and Control Engineering)

Time: 3 Hours							Max. Marks: 80

# Answer any Five questions

All questions carry equal marks

---

1.  a)	What is the difference between the short-shunt and long-shunt Compound Generators?

b)	What is the function of an armature in a D.C. Generator?

c)	Enumerate the conditions necessary for self excitation of a D.C. Generator.

d)	A 20 kw, 200V shunt generator has an armature resistance of 0.05 and a shunt field resistance of 200. Calculate the power developed in the armature when it delivers rated output.  Give sketch.

2.  a)	what are the applications of (i) D.C. Shunt motor (ii) D.C. Series Motor and (iii) D.C. Cumulative Compound Motor.

b)	Draw typical Load-Speed characteristics of a D.C. Series and Shunt Motors and give difference in them.

- A 220V d.c. shunt motor with constant field drives a load whose torque is proportional to the speed.  When running at 1000rpm it takes 30A. Find the speed at which it will run if a 10 resistance is connected in series with its armature. The resistance of armature may be neglected.

3.  a)	Discuss the procedure for conducting OC and SC tests on a single phase Transformer. How can the parameter be found from these tests?

b)	The open circuit test and short circuit test on a 5KVA, 250/125v, 50HZ single phase transformer gave the following results.

Oc test (H.T. side)	250v;	0.7A;	90w

Sc test (L.T. side) 	12v;	30A;	90W

Calculate the full load efficiency at UPF and the regulation.

4.  a)	Define the voltage regulation of a transformer and derive the approximate expression for the voltage regulation.

b)	The primary and secondary windings of a 30 KVA, 11000/240v transformer have the resistance’s of 10 ohms and 0.016 ohms respectively.  The total reactance of the Transformer referred to the primary is 23 ohms.  Calculate the percentage regulation of the transformer when supplying a full load current at 0.8 P.F. lag.

Contd……2.

Code No. 211001			::2::					Set No: 4

5.  a)	What is the necessity of a starter for a 3-phase induction motor

b)	A three-phase 440 V distribution circuit is designed to supply not more than 1200 amperes.  Assuming that a three-phase squirrel cage induction motor has a full-load efficiency of 0.85 and a full-load power factor of 0.8 and that the starting current at rated voltage is 5 times the rated full-load current, what is the maximum permissible kW rating of the motor, (i) if it is to be started at full voltage? (ii) if it is to be started using an auto-transformer stepping down the voltage to 80%?  (iii) if it is designed for use with a start-delta starter?

6.  a)	In a polyphase induction motor show that the rotor must rotate in the direction of rotator’s magnetic field.

b)	A 3-phase squirrel cage induction motor has a rotor starting current of 6 times its full-load value.  The motor has a full-load slip of 5%.  Determine (i) the starting torque in terms of full-load torque; (ii) the slip at which maximum torque occurs; and (iii) maximum torque in terms of full-load torque.

7.  a)	Develop the general phasor diagram for a cylindrical rotor alternator.

b)	A 100KVA, 6.6KV, 3 phase star connected synchronous generator has a synchronous reactance of 25 ohm per phase.  It supplies full load current at 0.75 p.f. lagging and a rated terminal voltage.  Compute the terminal voltage for the same excitation when the generator supplies full load current at 0.8 p.f. leading.

8.  a)	Draw a diagram showing the construction of a stepper motor and discuss its operation.

b)	Discuss the various applications of steppe motor

@@@@@