Code No:210351

OR

II B.Tech. I Semester Examination November/December,2002

FLUID MECHANICS

(Mechanical Engineering,Production Engineering,Mechatronics)

Time: 3 hours.									Max.Marks:70

Answer any FIVE questions.

All questions carry equal marks.

- - -

1.a)	A 1mx1m thin plate, kept at a distance of 0.1 mm from a fixed plate is towed at the rate of 500 mm/s with a force of 2N.  The gap between the plates is filled with oil.  Determine the absolute viscosity of oil.  Hence determine the kinematic viscosity of oil, if the oil  weighs 8 KN/m3.

b)	Derive an expression for the intensity  of pressure at a point.

2.a)	An open cylindrical tank  1m in diameter and 2 m high is filled with water to a depth of 1.5m.  It is rotated about  its central vertical axis at a speed of 45 r.p.m.  Calculate (1) The depth of water at the centre of the tank (2) The depth of water at the bottom corner of the tank.

b)	A vertical pipe line gradually changes from 600 mm in diameter at A to 900 mm at B.  The velocity at A is 1.5 m/s and the pressure is 350 kPa.  If the head lost between A and B is 25 times the velocity head at B, determine the intensity of pressure at B.  Point B is 5m below A.  Water flows from A to B

3.a)	Differentiate between;

(i)	Steady and unsteady flow

(ii)	Uniform and non-uniform flow

(iii)	Rotational and irrotational flow.

b)	The components of velocity in a fluid flow are given as u=x and v=-y.  Check whether the flow is possible.  Obtain expressions for velocity potential and stream function.

4.a)	Define (i) Laminar boundary layer (ii) turbulent boundary layer (iii) laminar sub layer (iv) boundary layer thickness.

b)	A thin plate is moving  in still  atmospheric air at a velocity of 4 m/s .  The length of the plate is 0.5m and width 0.4 m.  Calculate the (i) Thickness of the boundary layer at the end of the plate (ii) drag force on one side of the plate.  Take density of air 1.25 kg/m3 and kinematic viscosity 0.15 stokes.

Contd…..2

Code No:210351				-2-					OR

5.a)	Derive the continuity equation for one dimensional compressible flow in differential form.

b)	Find the mach number when an aeroplane is flying at 1100 Km/hour through still air having a pressure of 7 N/cm2 and temperature -50C.  Wind velocity may be taken as zero.  Take R=287.14 J/kg.K.  Calculate the pressure, temperature and density of air at stagnation point on the nose of the plane.  Take K=1.4

6.a)	Explain the phenomenon of capillarity.  Obtain an expression for capillary rise of a liquid.

b)	A circular plate 3.0 diameter is immersed in water in such a way that its greatest and least depth below the free surface are 4m and 1.5m respectively.  Determine the total pressure on one face of the plate and position of centre of pressure.

7.a)	Derive Bernaulli’s equation for the flow of in compression fluid from the first principles.  State the assumption made for such a derivation

b)	A 30 cm diameter pipe carries water under a head of 15m with a velocity of 4m/s.  If the axis of the pipe turns through 450, find the magnitude and direction of the resultant force on the bend.

8.a)	What do you mean by separation of boundary layer?  What is the effect of pressure gradient on boundary layer separation?

b)	The barometric pressure at sea level is 760 mm of mercury while that on a mountain top is 735 mm, if density of air is assumed constant at 1.2 kg/m3, what is the elevation of the mountain top.

*****