Code No: 210551

# OR

II B.Tech I- Semester Examination November/December 2002.

## ELECTRONICS DEVICES AND CIRCUITS

(Computer Science & Engineering)

Time: 3 hours								          Max. Marks: 70

Answer any Five questions

All questions carry equal marks

----

1 a)	Explain the generation and recombination of charges with regard to semiconductors.

b)	Draw the V-I characteristics of a PN junction diode and label the important parameters on it.  Explain the temperature dependence of these V-I characteristics.

- What is the transition capacitance of a PN junction diode?  How does it vary with the applied volluge .  What is the order of its magnitude?

2 a)	Explain how reverse biased diodes are used as varactor diodes.  Sketch its equivalent circuit and explain.

- Explain hall effect and how it can be applied to measure the charge density ‘e’ and mobility ‘’ of change carrier

- Derive an expression for the electro-magnetic defection sensitivity of a C.R.T.

3 a)	Explain the working of a PNP transistor and derive expressive for its ‘emitter injection efficiency” and “base-transport factor” in terms of the physical parameters of the transistor.

b)	A Ge transistor with the 0 = 0.98 gives a reverse saturation current Ico = 10 A in CB configuration.  When the transistor is used in CE configuration with a base current of 0.22 m A, calculate the collector current.  Derive the formula used.

4 a)	Sketch the static drain and transfer characteristics of a JFET and justify their nature.  What is pendi-off voltage and mark it on the characteristics.

- Explain how do you obtain JFET parameters from the above characteristics.

- AJFET has Vp = -4.5v, IDSS=16 mA and IDS=4 mA.  Determine Vas required and value of “Im” at this value of vas.

5 a)	Explain the basic construction of an SCR.  Explain its principle of operation using two transistor model.  Sketch its V-I characteristics.  Define ‘Hold current ‘ and latch current’ and mark them on the characteristics.  Explain its turn-off mechanism.

b)	Describe the fabrication of (i) Enhancement move MOSFET aml  (ii) MOS capacitor with neat cross-sectional diagrams in monolithic Integrated circuits.

Contd…2.

### Code No: 210551				-2					OR

6 a)	An amplifies uses a cascade of fan BJT stages suggest suitable choice of configuration for each stage, if it is to be a high gain amplifier with high input impedance and low output impedance.  Justify an answer.

b)	In a self biased circuit comprising R2 = 90 kr, R1 = 10 kr, Vcc= 22.5v, RC = 5.6 kr and RE = 1.0 kr and a BJT with 0 = 55 and VBE = 0.6v.  Find the operating point and ‘S’ of the circuit.

c)	Prove that for any amplifier configuration using a BJT the input impedance Ri is given by

hi

Ri =       __________

1-hr AV

7 a)	Explain how a Zener diode can be used as voltage regulator.

b)	Design  a power supply for a local that requires 30 walts of power.  The load voltage is 300V and the ripple voltage in the load should be less that 0.2v.  Use an L-section fitter and critical value of inductance.  Specify the required element values and input r.m.s voltage using a single phase bridge rectifier.  Assume supply frequency of 504t.

8	Write short notes on the following:

- Back ward diodes

- Biasusing circuits for MOS FETS.

- Emitter follower amplifier

- Thermal run away in BJT amplifiers

###