Code No:210151

OR

II B.Tech. I Semester Examination November/December,2002

MECHANICS OF SOLIDS-I

(Civil Engineering)

Time: 3 hours.									Max.Marks:70

Answer any FIVE questions.

All questions carry equal marks.

- - -

1.a)	Express Poisson’s ratio interms of shear and Bulk modulii.

b)	A mild steel bar of 200mm long and 50mmx50mm cross section is subjected to an axial compressive load of 200 KN.   Taking  1/m =0.3 and E = 210 KN/mm2, calculate the changes in length, width and volume of the bar.  Also calculate the values of shear and bulk modulii of the material of the bar

2.	A beam of length 10m is simply supported and carries two point loads of 5 KN each at distances of 3m and 6m from left support and also a udl of 1 KN/m between the point loads.  Draw S.F and B.M diagrams for the beam.

3.a)	Find the dimensions of the strongest rectangular beam that can be cut of a log of wood 1.8m diameter.

b)	A timber beam of rectangular section of length 8m is simply supported.  The beam carries a udl of 12KN.m run over the entire length and a point load of 8 KN at 3m from the left support.  If the depth is 2 times the width and the stress in the timber is not to exceed 8N/mm2, find the suitable dimensions of the section.

4.a)	Show that the maximum shear stress in a rectangular section is 1.5 times the average stress.

b)	Sketch the shear stress distribution across an I-section consisting of two flanges 200mmx10mm and a web 10mmx180mm.  Take F=20 KN

5.	A beam as shown in fig below is simply supported at A and F.  AB=1m, BC = 2m, CD = 4m, DE=1m, EF=2m.  Concentrated load of 40KN at B, udl of 5 KN/m over CD and a clockwise moment 120 KN.m at E are applied.  Determine the slope at F and deflection at B and mid point of beam

Contd….2

Code No:210151			-2-				OR

6.a)	Derive the expressions for longitudinal and hoop stress in a thin cylinder subjected to fluid pressure internally.

b)	A cylindrical sheel, 3m long, 1000mm in dia, thickness of metal 12mm is subjected to an internal pressure of 1.5 N/mm2.  Calculate the max intensity of shear stress induced and also the changes in the dimensions of the shell.  Given E = 2x1.5 N/mm2 and 1/m = 0.3.

7.	For the truss shown in fig below, find the forces in the members.

8.	Analyse the cantilever truss shown in fig below graphically.  Indicate the support reactions also.

*****