Code No: 210402

# Set No.

1

II B.Tech I- Semester Examination November/December 2002.

## PROBABILITY AND RANDOM VARIABLES

(common to Electronics and Communication Engineering and Electronics and Telematics)

Time: 3 hours								          Max. Marks: 80

Answer any Five questions

All questions carry equal marks

----

1.a)	Find the variance for Poisson Distribution.

- Find the moment generating function of the random variable whose moments are mr = (r+1) ! 2r.

2.a)	Give the classical and axiomatic definitions of Probability.

- If a three digit decimal number is chosen at random, find the probability that exactly K digits are greater than equal to 5, for 0≤K≤3.

- Three boxes of identical appearance contain two coins each.  In one box both are gold; in the second both are silver and in the third box one is silver and the other is the gold coin.  Suppose that a box is selected at random and further that a coin in that box is selected at random.  If this coin proves to be gold, what is the probability that the other coin is also gold?

3.a)	Define Probability density function.  List its properties.

b)	Find the value of the constant K so that    f(x) =      Kx2(1-x3), 0<x<1

0 otherwise

Is a proper density function of a continuous random variable?

- Define the joint distribution function.  Explain how marginal density functions are Computed given their joint distribution functions.

4.a)	An ergoic random process is known to have an auto correlation function of the form Rxx( ) = 1-| |, | | ≤1

= 0, | |>1

Show that the spectral density is given by

Sin w/2    2

Sxx(w) =               -----------

w/2

- Let the Random process be given as Z(t) = X(t) cos (w0t+) where X(t) is stationary Random process with E[X(t)] = 0 and E[x2(t)] = 6x2

  - If  = 0 find E[Z(t1)] and E(Z2). R Z(t) stationary

  - If   is a random variable independent of X(t) and uniformly distributed over the Interval (-,) Show that E[Z(t)] = 0 and E [Z2(t)] = 6x2/2

### Contd….2

Code No: 210402			            -2-				   Set No.1

5.a)	Derive Frii’s formula.

- What are the important requirements of the front-end stage of a communication receiver in the point of view of noise?

6.a)	Give reasons for using FET devices in low noise communication circuits.

- What precautions are taken in the construction of antenna for SNR maximization?

7.a)	Define the terms: Information, Self Information and Information rate with respect to a source.

- A source generates eight symbols: A, B, C, D, E, F, G and H with probabilities 0.5, 0.2,0.1,0.05,0.05,0.05,0.03 and 0.02.  Find the entropy and information rate if the symbols are generated at a rate of 1000/Sec.

8.a)	A source emits six messages with probabilities ½,1/4, 1/8 , 1/16, 1/32 and 1/32.  Determine average information per message.  What would be the information if the above messages were equally probable?

- Distinguish between the terms “Self Information and Mutual Information”.

- - -

Code No: 210402

# Set No.

2

II B.Tech I- Semester Examination November/December 2002.

## PROBABILITY AND RANDOM VARIABLES

(common to Electronics and Communication Engineering and Electronics and Telematics)

Time: 3 hours								          Max. Marks: 80

Answer any Five questions

All questions carry equal marks

- - -

1.a)	If  is a normal random variable.  Find the mean and variance of this random variable.

- Find the characteristic function for a random variable X defined by the probability density function.

0, x<0

f(x) =      1, 0x1

0, x>0.

2.a)	State and derive the theorem on Total probability.

- A shipment of components consists of three identical boxes.  One box contains 2000 components of which 25% are defective, the second box has 5000 components of which 20% are defective and the third box contains 2000 components of which 600 are defective.  A box selected at random and a component is removed at random from the box.  What is the probability that this component is defective?  What is the probability that it came from the second box?

3.	The number of newspapers that a certain delivery boy is able to sell in a day is found to be a numerical valued random phenomenon, with a probability function specified by the p.m.f.p(.) given by:    p(x) =Ax      x=1, 2, 3, …………50

=A(100-x) x=51, 52, …………100

= 0 otherwise

  - Find the value of A that makes p(.) a p.m.f. and sketch its graph.

  - What is the Probability that the number of newspapers sold tomorrow is (a) more than 50 (b) less than 50 (c) equal to 50 (d) between 25 and 75 exclusive (e) an odd number.

  - Let the events indicated in (ii) be denoted respectively by A, B, C, D and E.  Find P(A/B), P(A/C), P(A/D) and P(C/D).  Are A and B independent?  Are A and D independent?  Are C & D independent events?

#### Contd….2

Code No: 210402				       -2-			                   Set No.2

4.a)	Calculate the PSD of a stationary random process for which the Auto correlation is Rxx( ) = 6v, e-||  Auto correlation function of an aperiodic power signal is

Rxx( ) = exp (- 2/262).  Find the PSD and normalized average power content of the signal.

- Which of the following are suitable autocorrelation functions:  (a) A cos wo

(b) A sin wo   (c) A (/0) where (x) is a unit area rectangular function.  (d) A(/0) where (x) is the unit triangular function.

5.a)	What are the characteristics of shot noise?

- Show that the effective noise temperature of ‘n’ networks in cascade is given by,

Te = Te1  Te2/g1 + Te3/g1g2 + …………+ Ten/g1g2…gn-1)

6.	Which of the following noise parameters is the true representation of noise in electrical circuits?  (i) Noise figure  (ii) Noise temperature  (iii) Noise bandwidth.  Support your answer with the help of suitable examples.

7.a)	State and prove Shannon’s theorem with respect to channel capacity.

- Find the entropy corresponding to the random variable whose density function shown below,

P(x)

1

-------

b - a

0             a           b            x

8.a)	Write a short on the role and use of Information theory to a communication engineer.

- A card is drawn at random from ordinary deck of 52 playing cards.  Find the information in bits that you receive when you are told that the card is a heart, a face card and a heart face card.

- - -

Code No: 210402

# Set No.

3

II B.Tech I- Semester Examination November/December 2002.

## PROBABILITY AND RANDOM VARIABLES

(common to Electronics and Communication Engineering and Electronics and Telematics)

Time: 3 hours								          Max. Marks: 80

Answer any Five questions

All questions carry equal marks

- - -

1.a)	Find the density function of the random variable X if the characteristic function is given by (t) = 1-|t| , |t| 1

= 0,  |t| >1.

- If X and Y are two independent random variables, such that E(X) = λ 1,  variance (X) = 1 2, E[y] = λ 2, variance (y) = 22.  Prove that variance (XY) = 1222 + λ12 22 + λ22 12.

2.a)	Define Probability density function and obtain the relationship between Probability and probability density.

- Consider the probability density f(x) = a e –b|x| where x is a random variable whose allowable values range from x = - to .  Find (i) the CDF F (x) (ii) the relationship between a and b. and (iii) the probability that the outcome x lies between 1 and 2.

- The joint probability density of the random variables X and Y is given by F(x, y) = ¼ e -|x| -|y|; -<x<  , -<y<  (i) Are X and Y statistically independent random variables?  (ii) Calculate the probability that X1 and Y0.

3.a)	Explain the Normal or Gaussian distribution.

- An analog signal received at the detector (measured in microvolts) may be modeled as a Gaussian random variable N (200, 256) at a fixed point in time.  What is the probability that the signal will exceed 240 Vs?  What is the probability that the signal is larger than 240 V, given that it is larger than 210 Vs?

4.a)	Find the input auto correlation function, output auto correlation and o/p spectral density of RC low pass fitter, when the fitter is subjected to a white noise of spectral density No/2.

- A Random process n(t) has a power spectral density G(f) = /2 for -f.  The Random process is passed through a lowpast fitter which has transfer function H(f) = 2 for -fMf fM and H(f) = 0 otherwise find the PSD of the waveform at the o/p of the fitter.

### Contd….2

Code No: 210402				    -2-				       Set No.3

5.a)	Derive the overall noise figure of n-stages of receivers.

- In TV receivers, the antenna is often mounted on a tall mask and a long lossy cable is used to connect the antenna and receiver.  To overcome the effect of noisy cable, a preamplifier is mounted on the antenna.  The parameters of  the different stages are

Preamplifier gain = 20 dB

Preamplifier Noise figure = 6 dB

Lossy cable noisy figure = 3 dB

Cable Loss = -20 dB

Receiver front end gain = 60 dB

Receiver Noise figure = 16 dB

Determine the overall noise figure of the system.

6.a)	What are the sources of flicker noise and how can it be reduced?

- An amplifier with ga = 40dB and BN = 20KHz is found to have Nao = 100K To when Ti = To.  Find Te and noise figure.

7.a)	State the fundamental theorem of “Information Theory” and explain its significance.

- Derive an expression for channel capacity of a continuous channel in the presence of White Gaussian noise.

8.a)	Discuss the necessity for “Source coding”.

- A source has an alphabet {a1, a2, a3, a4, a5, and a6} with corresponding probabilities {0.1, 0.2, 0.3, 0.05, 0.15 and 0.2}.  Find the entropy of its source.  Compare the entropy with the entropy of a uniformly distributed source with same alphabet.

- - -

Code No: 210402

# Set No.

4

II B.Tech I- Semester Examination November/December 2002.

## PROBABILITY AND RANDOM VARIABLES

(common to Electronics and Communication Engineering and Electronics and Telematics)

Time: 3 hours								          Max. Marks: 80

Answer any Five questions

All questions carry equal marks

- - -

1.a)	Show that the distribution function for which the characteristic function e-|t| has the density function

1

fx(x) =      -------------- ,       -<x<

 (1+x2)

- Two random variables X and Y has the following joint probability density function: f(x, y) = 2-x-y; 0x1, 0y1  = 0 otherwise.  Find (i) Marginal probability density functions of X and Y  (ii) Var (X) and Var (Y).

2.a)	Define Probability mass function.  List its properties.

- The probability density function of X is given by the following table:

X 	0	1	2	3	4	5	6

P(X)  	K	3K	5K	7K	9K	11K	13K

Find (i) P (X<4)  (ii) P(X5)  (iii) P(3<X6)

What will be the minimum value of K so that P (X2) >0.3?

3.a)	Derive an expression for, the error function of the standard normal Random variable.

- Lifetime of IC chips manufactured by a semiconductor manufacturer is approximately normally distributed with mean = 5 x 106 hours and standard deviation of 5 x 105 hours.  A mainframe manufacturer requires that at least 95% of a batch should have a lifetime greater than 4 x 106 hours.  Will the deal be made?

4.a)	Find  the Auto correlation function by time averaging and show that (Rxx ()) = (Rxx()) = Rxx(r).

- X(t) is a zero mean stationary Random process with an Auto Correlation function Rxx().  By integrating X(t) ur form a Random variable Y as

1        T

Y=     ----       ∫  X(t) dt.

2T     -T        2T

Show that E(Y) = x = 0 and σy2 = 1/T ∫  (1-/2T) Rxx() d.

### Contd….2

Code No: 210402			          -2-			              Set No.4

5.	A proposed ground station for receiving signals from a satellite has a receiver with a cooled parametric amplifier with a 0.3 dB noise figure, a bandwidth of 1 MHz and a center frequency of 3GHz.  The receiving antenna has a power gain of 50 dB and the satellite antenna has a power gain of 6dB.  Path loss in decibels is a=40+20log10(f)+20log10(d).  Where f is the centre frequency in MHz and d is the distance in miles.  If the transmitter power is 15 watt and a minimum 15 dB SNR is to be maintained at the receiver output, estimate the distance over which communication can be maintained.  Assume that the antenna temperature is 60K.

6.	Write short notes on (i) Flicker noise  (ii) Partition noise  (iii) Johnson’s noise.

7.a)	Describe the channel capacity of a discrete channel.

- Explain Shannon – Fano algorithm to develop a code to increase average information per bit.

8.a)	Show that, H(x, y) = H(x) + H(y/x)

= H(y) + H(x/y).

- State Shannon’s fundamental theorem for information transmission on noise channel.

- Define Shannon’s measure of information.

- - -