Code No.210552

OR

II  B.Tech. I-Semester Examination Nov/Dec 2002
PRINCIPLES OF ELECTRICAL ENGINEERING AND NETWORKS

(Computer Science and Engineering)

Time:3 hours								Max.Marks:70

Answer any FIVE questions

All questions carry equal marks

---

1.a)	State and explain Kirchhoff’s laws with examples.

b)	Find the value of R and the current flowing in it in the network shown below if the current flowing in branch OA is zero.

## O

2.a)	Deduce from first principles the E.M.F equation of a D.C. generator.

b)	A 4 pole D.C generator has an armature with boslots each carrying 24 lap connected conductors. If the machine runs at 1000rpm and generates 432V, what is the useful flux per pole? If the armature is wave connected, other conditions remaining same, what would be the e.m.f. generated.

3.a)	Draw the load characteristics of D.C shunt, series and compound motors and explain them.

b)	A 250V, D.C.shunt motor takes 4A when running unloaded. Its armature and field resistances are 0.3 and 250ohms respectively. Calculate its efficiency when the D.C.shunt motor takes a full load current of 60A.

4.a)	Explain the principle of operation of a transformer.

b)	A 4600/230V single phase transformer takes a no load current of 0.2A at 0.5P.F. lag on 4600V side.  If this transformer is loaded with 16A at 0.8 P.F. lag on 230V side,	(i) Estimate the input current and P.F.

ii)  Draw the phasor diagram

5.a)	Explain the typical slip-torque characteristic of a three phase induction motor and clearly mark.

i)  Working region and 	(ii) Acclerating region of the motor on this characteristic.

b)	A 3 phase, 50Hz, 12 pole induction motor develops 4.5H.P. at 450 rpm. What is the rotor out  put? Also calculate the stator input if the stator losses are 200 watts.

Contd….2

# Code No.210552				-2-					OR

6.a)	Derive the e.m.f. equation for a 3-phas alternator.

b)	The armature of a 8 pole, 3 phase, 50Hz alternator has 18 slots and has 10 conductors per slot. There are 0.04wb entering the armature from one pole. Assume the fluse to be distributed sinusoidally along the air gap. Calculate the e.m.f. per phase.

7.a)	For the circuit shown in figure below find the current blowing in resistance RL using Thevenin’s theorem.

b)	Using superposition theorem, find the current through a link that is to be connected between terminals a-b. Assume the link resistance to be zero.

8.	Write a short notes on any Three of the following:

- Substitution theorem

- Torque equation of D.C.rotor

- Types of induction motors and their construction

- Loop and nodal analysis of networks.

***	***	***