Code No.210104 Set No. 1

II-B.Tech. I-Semester Examination Nov/Dec 2002

SURVEYING

(Civil Engineering)

Time:3 hours								Max.Marks:80

Answer any FIVE questions

All questions carry equal Marks

---

1.a)	Explain the principle involved in chain survey. Explain the working of the following

- Optical square		(ii) Prism square

b)	What are the methods used for checking the triangle of a chain survey?

2.a)	Data pertaining to a cross staff survey are given below. Plot the area surveyed and calculated its area. All distances are in metres.

direction

90B 125C D

0 100 125 240 270 370

F120, G50 E100

A

- Define the following

i) Magnetic dip and magnetic declination	(ii) Isogonic and Agonic lines

3.a)	What is local attraction? How is it detected and eliminated?

- The fore bearings of the lines of a traverse are given below:

Line			Fore bearings

AB			      36o20’

BC			    100o36’

CD			    138o25’

DE			    198o40’

EA			    317o10’

Find interior angles.

4.a)	Briefly explain the various methods of orienting the plane table.

- Describe the trial and errors method of solving the three point problem.

(Contd…2)

Code No.210104			-2-			Set No.1

5.a)	How do you detect any movement of the plane table at a station between sights?

- Explain Trapezoidal rule and derive the expression.

6.	Volumes of cut and fill along the length of a proposed road are as follows:

| Chainage | Volume (cum) |  |
| --- | --- | --- |
|  | Cut | Fill |
| 0 |  |  |
| 30 | 8.21 |  |
| 60 | 21.52 |  |
| 90 | 47.57 |  |
| 120 | 17.56 |  |
| 145 | 3.40 |  |
| 150 |  | 0.57 |
| 180 |  | 3.11 |
| 210 |  | 9.11 |
| 240 |  | 17.0 |
| 270 |  | 22.10 |
| 300 |  | 19.54 |
| 330 |  | 11.33 |
| 360 |  | 3.40 |

Draw a mass-haul diagram and excluding the surplus excavated material along this length determine the overhaul if the free haul distance is 90m (use graphical method).

7.	With neat sketches explain the principle and working of the tangent clinometer.

8.	Explain in brief:

- Base line and check line

- Intersection method in plane table survey

- Whole circle bearing.

***	***	***

Code No.210104 Set No. 2

II-B.Tech. I-Semester Examination Nov/Dec 2002

SURVEYING

(Civil Engineering)

Time:3 hours								Max.Marks:80

Answer any FIVE questions

All questions carry equal Marks

---

1.a)	Explain the following terms:

i)  Check line and tie line   (ii)  Random line	   (iii)  Well conditioned triangle.

b)	B and C are two points on the opposite banks of a river along a chain line ABC which crosses the river at right angles to the bank. From a point P which is 150 ft. from B along the bank, the bearing of A is 215o30’ and the bearing of C is 305o30’. If the length AB is 200ft., find the width of the river in meters.

2.a)	Describe the methods of determining the width of a river

i) With the chain only	(ii) With the chain and optical square.

b)	What are the sources of errors in compass survey and what precautions will you take to eliminate them?

3.a)	Draw and explain the differences between prismatic compass and surveyor’s compass.

b)	In a closed traverse ABCDE, the bearing of the side AB was 19o30’. The interior angles of the traverse arc,  ABC = 112o45’,   BCD = 108o20’,  CDE = 159o10

DEA = 57o15’ and    EAB = 102o30’. Find the bearings of  all the sides of the traverse.

4.a)	Discuss the advantages and disadvantages of plane table surveying over other methods.

- How do you check the defective orientation of the plane table?

5.	Describe four methods for finding areas by offsets to base line.

6.a)	Explain and derive formulae for finding areas by one level section and two level section.

b)	A cutting is to be made through the ground where the cross slope varies considerably. At A, the depth of the cut is 12ft. at the centre line, and cross slope is 8 to 1. At B the corresponding figures are 10ft and 12 to 1 and at C 14ft and 10 to 1. AB and BC are each 100 feet. The formation width is 30 feet and the side slopes 1½ to 1. Calculate by the prismoidal method the volume of the cutting in cubic yards between A and C.

7.	Explain with figures:	(a) Hand level		(b) Pantagraph

8.	Explain briefly  (a) Amsler’s Polar Planimeter   (b) Optical square and cross staff.

***	***	***

Code No.210104 Set No. 3

II-B.Tech. I-Semester Examination Nov/Dec 2002

SURVEYING

(Civil Engineering)

Time:3 hours								Max.Marks:80

Answer any FIVE questions

All questions carry equal Marks

---

1.a)	Explain how will you continue chaining past the following obstacles.

i) a river	(ii) a hill	(iii) a tall building

b)	Explain the working of the following:

i) Optical square	(ii) Prism square

2.a)	A chain line ABC crosses a river, B and C being on the near and distant banks respectively. The respective bearings of C and A taken at D, a point 60m measured at right angles to AB from B arc 280o and 190o, AB being 32m. Find the width of the river.

b)	Differentiate between closed traverse and open traverse. When is an open traverse performed.

3.a)	In a compass survey one of the stations is close to a large iron pipe. What precautions would you take for getting correct bearing readings?

b)	The following data were measured in a traverse ABCDE. Determine the length of DE and the bearing of EA.

| Line | Length | Bearing |
| --- | --- | --- |
| AB | 884 | N 75o20’E |
| BC | 988 | N 14o30’ W |
| CD | 965 | N 85o 45’ W |
| DE | ? | N 85o 45’ W |
| EA | 1060 | ? |

4.a)	What are the errors that commonly occur in plane table surveying?

b)	What is the practical utility of three point and two point problems?

5.a)	Explain the measurement of area by mechanical method.

b)	How do you measure the area using offset to base line method?

(Contd…2)

## Code No.210104			-2-				Set No.3

6.a)	Explain finding of volumes using  prismoidal rule.

- Calculate the volume of earthwork for road cutting 120m long from the following   data

Formation width = 12m

Mean depth of cutting along the centre line = 1.5m

Transverse slopes = 10 to 1

7.a)	Explain the working procedure of pantagraph using neat sketch.

b)	Briefly explain ghat tracer.

8.	Write short notes on:

- Declination

- Lehmann’s rulses

- Simpson’s rule

***	***	***

Code No.210104 Set No. 4

II-B.Tech. I-Semester Examination Nov/Dec 2002

SURVEYING

(Civil Engineering)

Time:3 hours								Max.Marks:80

Answer any FIVE questions

All questions carry equal Marks

---

1.a)	Explain setting out a right angle with a tape at a point in the chain line.

b)	What are the factors to be considered in deciding the stations of a chain survey?

2.a)	A survey line ABC runs at right angles to a river intersecting its banks at B and C. To determine the width BC of the river a line BE 72m long was set out approximately parallel to the river. A point D was established on CE produced and through the middle point F of BD, the line EF was extended to G. So that FG is equal to EF. The line DG produced intersects the survey line at H. The distances GH and HB were measured and found to be 48m and 96m respectively. Determine the width BC of the river.

b)	Draw and explain the differences between prismatic compass and surveyor’s compass.

3.a)	i)  Differentiate between closed traverse and open traverse.

ii) Define magnetic dip and magnetic declination.

- The following bearings were taken for a closed traverse ABCD in an area under the influence of local attraction. Find the corrected bearings.

| Line | FB | BB |
| --- | --- | --- |
| AB | 69o49’ | 250o45’ |
| BC | 16o12’ | 196o12’ |
| CD | 284o20’ | 105o25’ |
| DA | 200o15’ | 18o14’ |

FB – Fore Bearing

BB – Back Bearing

4.a)	How do you detect any movement of the plane table at a station between sights?

b)	Explain	(i) Radiation	(ii) Intersection	(iii) Traversing methods.

5.	What are the methods used for the determination of areas?

(Contd…2)

### Code No.210104			-2-				Set No.4

6.a)	Explain the computation of volumes in hilly areas using side hill two-level section method.

b)	Determine the volume of earth work for a road embankment 125m long from the following data

Width of formation = 12m

Side slope of banking = 2 to 1

Traverse slope of the ground = 5 to 1

Mean height of embankment = 2.5m

7.	Explain briefly the working procedure of

a) Box soxtant		(b) Foot-Rule clinometer

8.	Write short notes on:

- Magnetic bearing		(b) Prism square	(c) Offsets	(d) Planimeter

***	***	***