Code No.210201 Set No. 1

II-B.Tech. I-Semester Examination Nov/Dec 2002

PRIME MOVERS AND PUMPS

(Electrical and Electronics Engineering)

Time:3 hours								Max.Marks:80

Answer any FIVE questions

All questions carry equal Marks

---

1.a)	Prove that the pressure inside the draft tube at the inlet is less than atmospheric pressure.

- A Pelton wheel 2.45m in diameter, operates under the following conditions:

Net head = 370m

Coefficient of velocity = 0.98

Speed ratio = 0.47

Vr₁ = 0.90 Vr

Angle of deflection = 160o

Diameter of jet = 0.88m

Determine :

i) Input power to the shaft, and 	(ii) The rpm of the wheel.

2.a)	What does priming of a centrifugal pump mean?

b)	It is required to deliver 0.05m3/s of water to a height of 25m through a 150mm diameter and 120m long pipe by means of a centrifugal pump. If the overall efficiency of the pump is 80% and coefficient of friction is 0.01 for the pipe, find the power required to drive the pump?

3.a)	What are the types of reciprocating pumps? Explain in detail.

b)	A single acting reciprocating pump runs at 60rpm. The diameter of the plunger is 0.15m and crank radius is 0.25m. The suction pipe is 0.1m in diameter and 5m long. Calculate the maximum permissible value of suction lift if the separation takes place at 2.6m of water absolute.

4.a)	What is air standard cycle? Explain the cycle used in SI engine by representing it on p-rdiagram.

- An engine working on the Otto cycle is supplied with air at 0.1Mpa, 35oC. The compression ratio is 8. The heat supplied is 2100 kJ/kg. Calculate the maximum pressure and temperature of the cycle. (For air Cp = 1.005, Cv = 0.718kJ/kg-K).

5.a)	Explain briefly with neat sketches, the working principle of a parallel flow jet condensers.

- Describe with a neat sketch a low level jet condenser of the counter flow type.

(Contd…2)

# Code No.210201		-2-					Set No.1

6.a)	Sketch and explain the working of a Locomotive Boiler.

- What is the difference between a Simple Vertical Boiler and Cochran type boiler.

7.a)	Compare the advantages and disadvantages of gas turbines and steam turbines.

- List the assumptions made in the analysis of ideal gas turbine cycle.

8.	The steam supply to a velocity-compounded impulse state with two rows of moving blades is at 600 x 104 N/m2 and 400oC. Find

i)   The stage pressure		(ii) The diagram efficiency    and

iii) The power output from the following data:

Speed 3000 RPM, mean diameter of blading 1.2m, steam flow 5 kg/s, blade speed/steam speed ratio 0.2. Nozzle efficiency 90%, nozzle angle 18%, exit angles 26o, 25o  and 40o for the first moving, fixed and second moving blades respectively. Velocity coefficient 0.86 for all blades. A power of 18.75 kW is used in disc friction and windage. What is the condition of steam leaving the stage?

***	***	***

Set No. 2

Code No.210201

II-B.Tech. I-Semester Examination Nov/Dec 2002

### PRIME MOVERS AND PUMPS

(Electrical and Electronics Engineering)

Time:3 hours								Max.Marks:80

Answer any FIVE questions

All questions carry equal Marks

---

1.a)	Distinguish between impulse and reaction turbines.

b)	A Pelton wheel has to develop 13230 KW under a net head of 800 m while running at speed of 600rpm. If the coefficient velocity of jet Cv = 0.97, speed ratio Ku = 0.46 and the jet diameter is 1/16 of wheel diameter, calculate the number of jets required for the Pelton wheel.

Calculate also:      (i) The diameter of jets	(ii) The diameter of pitch circle

iii) The quantity of water supplied to the wheel.

Assume overall efficiency as 85%.

2.a)	What is manometric efficiency of centrifugal pump?

b)	A centrifugal pump is to discharge 0.1m3/s of water at a speed of 1440 rpm against a head of 25m. The impeller diameter and width at outlet are 250mm and 50mm respectively. The manometric efficiency is 80%. Find the angle of vane at the outer periphery of the impeller?

3.a)	Show the components of a reciprocating pump with a neat sketch.

b)	A single acting reciprocating pump has a piston diameter of 0.15m and a crank radius of 0.3m. The delivery pipe is 0.10m in diameter and 30m long. The water is lifted 20m above the center of the pump. Find the maximum speed at which the pump can be run so that no cavitation takes place during delivery stroke. The separation takes place at 2.6m of water absolute. The atmosphere pressure is 10.3m of water.

4.a)	Derive the efficiency of a reversible Carnot engine operating between the temperature limits of T1 and T2 (T1>T2).

b)	Which is more effective way to increase the efficiency of a Carnot engine: to increase T1 keeping T2 constant or to decrease T2, keeping T1 constant.

5.a)	Explain the working principle of a barometric jet condenser with the help of a neat sketch.

b)	Explain the working principle of a ejector condenser with the help of a neat sketch.

6.a)	What is difference between a Locomotive Boiler and Cochran type of boiler-explain.

- Mention the parts in Locomotive Boiler and explain its functions.

(Contd…2)

## Code No.210201		-2-					Set No.2

7.a)	Discuss the relative advantages and disadvantages of reciprocating I.C engines and gas engines.

b)	With the aid of a schematic diagram, explain the working of a simple closed cycle gas turbine.

8.	A single row impulse turbine stage receives 5kg of dry saturated steam per second at 100 x 104 N/m2. The steam is expanded in the nozzle to a pressure of 70 x 104 N/m2, with an efficiency of 0.94, and discharges at an angle of 20o to the plane of rotation of the blades.

Find a suitable exit angle for the blades in order that there shall be no axial thrust on the blades, allowing a blade velocity coefficient of 0.85 and a blade speed of 175m/s. If the internal losses due to disc friction and windage amount to 0.95 kcal/kg of steam, find the efficiency and power of the stage.

***	***	***

Code No.210201 Set No. 3

II-B.Tech. I-Semester Examination Nov/Dec 2002

PRIME MOVERS AND PUMPS

(Electrical and Electronics Engineering)

Time:3 hours								Max.Marks:80

Answer any FIVE questions

All questions carry equal Marks

---

1.a)	In a Pelton wheel the buckets deflect the jet by 170o and the relative velocity is reduced by 12% due to bucket friction. For a speed ratio of 0.47, calculate from first principles the hydraulic efficiency of the wheel.

The bucket circle diameter of the wheel is 90 cm and there is one jet for which Cv = 0.98. The overall efficiency is 0.9 times the hydraulic efficiency. The wheel develops 1700 KW under a head of 550m. Calculate

i) The speed of the wheel in rpm, and 	(ii) The diameter of the nozzle.

- Explain the method of selection of turbine.

2.	A centrifugal pump discharges 0.2m3/s of water against a head of 13m. The speed of the impeller is 650rpm. The outer and inner diameter of the impeller are 500mm and 250mm respectively and the vanes are bent back at 35o to the tangent at exit. If the area of flow remains 0.065m2 at inlet and outlet determine:

a) Manometric efficiency	(b) Vane angle at inlet and

c) Loss of head at inlet to impeller when the discharge is reduced by 50% without changing the speed?

3.a)	How is reciprocating pump different from centrifugal pump?

b)	A positive displacement pump has a plunger diameter 0.3m and a troke of length 0.4m. If the speed of the pump is 80 rpm and coefficient of discharge is 0.97, determine the percentage slip and actual discharge of the pump.

4.a)	Show that the efficiency of a reversible Carnot engine is independent of the nature or amount of working substance.

b)	A Carnot engine operates between a source temperature of 800oC and a sink temperature of 30oC. What is the heat rejection per KW and net output of engine.

5.a)	Describe with a neat sketch the working of a surface condenser.

- What is the difference between jet condenser and surface condenser.

6.a)	Sketch and explain the working of Vertical Multi Tubular Boiler.

- Explain the functions of each part in Multi Tubular Boiler.

(Contd…2)

## Code No.210201		-2-					Set No.3

7.a)	Compare the working processes in reciprocating I.C engines and gas turbines.

- “Closed cycle gas turbine plants are akin to steam turbine plants”. Discuss.

8.a)	What is the disadvantage of having very small exit angles for nozzles and moving blades of an impulse turbine.

b)  	One stage of an impulse turbine consists of a row of nozzles and one of moving blades. The steam enters the nozzles at a pressure of 150 x 104 N/m2, dry saturated, with a velocity of 130 m/s. The pressure drops along the nozzle to 90 x 104 N/m2. The nozzles have discharge angle of 20o and the steam passes into the blades without shock. If the velocity coefficient for nozzle is 0.9, determine for maximum efficiency conditions,

i)   The blade angles for equiangular blades	(ii) The blading efficiency and

iii) The stage efficiency.

***	***	***

Code No.210201 Set No. 4

II-B.Tech. I-Semester Examination Nov/Dec 2002

PRIME MOVERS AND PUMPS

(Electrical and Electronics Engineering)

Time:3 hours								Max.Marks:80

Answer any FIVE questions

All questions carry equal Marks

---

1.a)	A jet of water 5cm diameter impinges on the buckets of a Pelton wheel deflecting it through an angle of 170o, the blade friction coefficient being 0.95. The head at the nozzle is 30m and the coefficient of velocity is 0.97. Determine the force exerted by the jet on a series of such buckets revolving at a velocity of 0.45 times that of the jet. Find the power developed by the buckets. Assuming that the mechanical friction loss amount to 4% of water power supplied, find the shaft power and the overall efficiency of the turbine.

- What are iso efficiency curves of a turbine.

2.a)	How do you find the minimum stating speed of a centrifugal pump?

b)	The diameter of an impeller of a centrifugal pump at inlet and outlet are 25cm and 50cm respectively. Find the minimum starting speed of the pump if it works against a head of 25m?

3.a)	What is the working principle of a reciprocating pump?

b)	A single acting reciprocating pump has a stroke of 30cms, a speed of 75rpm and a piston of 45 cms diameter. 0.57m3/sec of is lifted through a height of 18m through a delivery pipe of 15cm diameter. Find the coefficient of discharge, slip and horse power required to drive the pump.

4.a)	Explain the processes involved in Reversible Carnot cycle.

b)	An inventor claims to have developed an engine that takes in 105MJ at a temperature of 400K rejects 42MJ at a temperature of 200K and delivers 54MJ of mechanical work. Would you advice investing money to put this engine in the market?

5.a)	What are the relative merits and demerits of surface condenser over jet condenser.

- Explain the working principle of a down flow surface condenser.

6.a)	What is the difference between a Locomotive Boiler and Scotch Marine Boiler.

- Sketch and explain the working of a Scotch Marine Boiler.

7.a)	Enumerate the advantages and disadvantages of gas turbines.

b)	With the aid of a schematic diagram explain the working of a open cycle gas turbine.

(Contd…2)

# Code No.210201		-2-					Set No.4

8.	In a velocity-compounded impulse turbine the steam leves the nozzles at 500m/s and after passing through the fixed and moving blade rings it is discharged from the second moving blade ring in an axial direction. The axial velocity of flow in the first moving blade ring is twice that in the second moving blade ring. The discharge angle of the second blade ring is 30o (measured from the plane in which this blade ring rotates) and it is estimated that the relative velocity of steam falls by 10 percent in passage through each blade ring, fixed and moving. The blade angles are such that the flow is shock-less and that there is no end thrust on either moving ring.

***	***	***