Code No: 210304

# Set.No.

1

II B.Tech. I Semester Examination November/December, 2002

THERMO DYNAMICS

(Common to Mechanical Engineering and Aeronautical Engineering)

Time: 3 hours.								        Max. Marks: 80

Answer any FIVE questions.

All questions carry equal marks.

- - -

1.a)	Show that the internal energy is a property of the system.

b)	Air at a Pressure of 50 bar and a volume of 0.2m3 is expanded at constant pressure until the volume is doubled. It is then expanded according to PV1.3 = constant until the volume is 0.8m3. Calculate the work done in each process.

2.	A cylinder contains 0.115 m3 of gas at 1 bar and 900C. The gas is compressed to a volume 0.0288 m3, the final pressure being 5.67 bar. Calculate  (a) The mass of the gas  (b) The value of index is compression.  (c) The increase in internal energy of the gas and  (d) The heat transfer during the compression.

If, after the above compression, the gas is to be cooled at constant pressure to its original temperature of  900C, find the further work of compression required.  Assume  = 1.4 and R = 0.3kj / kg K.

3.a)	Prove that Kelvin-plank statement and Clausius statement of Second law of thermodynamics are equivalent.

b)	Two reversible heat engines A and B are arranged in series with A rejecting heat directly to B through an intermediate reserviour. Engine A receives 200 kJ of heat from a reservoir at 421C, and engine B is in thermal communication with a sink at 4.4C. if the work out put of A is twice that of B find  (a) the intermediate temperature between A and B, (b) the efficience of each engine and

(c) The heat rejected to the cold sink.

4.a)	Draw neatly the sequences of operation of Carnot engine on p-v and T-s diagrams.

Show that the entropy change during the cycle is zero

- A Carnot engine operates between 1000K and 300K. The change in entropy of the

Sources is 0.6kJ K. Find the heat added and the network out put.

- CO2 gas is contained in 1.0m3 tank initially at 1.2 bar and 300K. The temperature is increased by 400K by (i) stirring with a paddle wheel and (ii) heat supply from a reservoir. Find the initial and final availabilities of the gas. Which method of heating is better from the thermodynamic point of view?

( Contd…2 )

## Code. No: 210304              		              .2.				     Set.No.1

5.a)	List out the various non-flow processes when the First law of thermodynamics is applied to closed system.

b)	Derive the relationship between two principle specific heats and characteristic gas    constant for a perfect gas.

c)	Find the molecular weight and gas constant for the gas whose specific heats are  as follows:

Cp = 1.967KJ/Kg K		Cv = 1.507 KJ / Kg K

6.a)	Define (i) Mole fraction

(ii) Mass fraction

b)	The volumetric analysis of a dry flue gas in a boiler trial is given in percentage as 13% CO2 , 1.5% CO, 3.5%O2 and 82% N2.

Determine the percentage gravimetric analysis.

Also find the specific gas constant of the mixture.

7.a)	Define:

- Relative Humidity

ii)   Specific Humidity

b)	An air water-vapour mixture at  250C and 1 bar has a relative humidity of 50% determine

- The partial pressure of steam;

ii)   The dew point;

iii)  The density of each constituent;

- The specific humidity

8.a)	Explain the working of simple Rankine cycle and what are different ways to improve the efficiency of a cycle?

b)	Explain the functioning of vapour compression refrigeration system along with diagrams?

Code No: 210304

# Set No.

2

II B.Tech. I Semester Examination November/December, 2002

THERMO DYNAMICS

(Common to Mechanical Engineering and Aeronautical Engineering)

Time: 3 hours.								          Max. Marks: 80

Answer any FIVE questions.

All questions carry equal marks.

- - -

1.a)	Define zeroth law of thermodynamics. Explain how it is basis for the temperature measurement.

- 1.5 Kg of liquid having constant specific heat of 2.5kj / kg0C is stirred in a well insulated chamber causing the temperature rise by 150C find the change in energy and work done for the process.

2.	2m3 of hydrogen at a pressure of 1 bar and 200C is compressed is entropically to 4 bar. The same gas is expanded isothermally to the original volume. Finally, the gas pressure is restored to the original volume by a constant volume heat rejection process. Determine  (a)  pressure, volume and temperature at each and of operation   (b)   the heat added during the isothermal process   (c)  the heat rejected during constant volume process and  (d)  change in internal energy during each process.  Assume R = 4.206 kj / kg K  and  CP = 14.25 kj / kg K.

3.a)	State and prove Clausius inequality and hence deduce that the property entropy exist.

b)	A cylinder contains 0.5 m3 of a gas at 0.1 MPa and  900C, The gas is compressed to a volume of  0.125 m3. The final pressure is 800 kPa. Determine the heat transferred and the change in entropy of the gas during the process.

Assume  R = 0.287 kj / kg K and  Cv = 0.713 kj / kg K.

4.a)	Obtain expression to find entropy change for a system of ideal gas in terms of pressure, temperature and volume during any process.

b)	1 kg  of air is compressed according to the law pv 1.25 = C from a pressure of

105 kPa and temperature of 150C to a pressure of 1680 kPa. Calculate the temperature at the end of compression, the heat received or rejected by the air and change in entropy. Assume for air Cp = 1.0005 kj / kg K and  R = 0.287 kj / kg K.

(Contd…2)

## Code No. 210304              		          ..2..				         Set.No.2

5.a)	Prove that in constant pressure non-flow process, the heat transfer is equal of enthalpy change.

b)	A closed system of constant volume experiences a temperature rise of  200C when a certain process occurs. The heat transferred in the process is 18KJ. The specific heat at constant volume for the pure substance comprising the system is

1.2 kj / kg0 C, and the system contains 2Kg of this substance. Determine the change in internal energy and work done.

6.a)	Explain Dalton’s  Law of partial pressures

b)	Three kg mole of  CO2 at  1.5 bar, 750C are mixed with 6kg – mole of Oxygen at the same pressure and temperature in a thermally insulated cylinder. Determine the increases in entropy for the process.

7.a)	Define:

- Saturated Air.

- Dew point.

b)	Air has a dry bulb temperature of 270C and a wet bulb temperature of  200C. If the barometer reads 1bar,

Calculate :

- The humidity ratio

- The relative humidity

- The dew point

- The enthalpy of mixture per kg of dry air.

8.a)	Explain about the re-heating and regeneration of a steam cycle with help of neat diagrams?

b)	Derive the efficiency of a ideal Diesel cycle with  p-v and  T-s diagrams.

Code No:210304

# Set No.

3

II B.Tech. I Semester Examination November/December, 2002

THERMO DYNAMICS

(Common to Mechanical Engineering and Aeronautical Engineering)

Time: 3 hours.								         Max. Marks: 80

Answer any FIVE questions.

All questions carry equal marks.

- - -

1.a)	Differentiate between intensive and extensive properties. Give examples in each case.

b)	1 kg of gas at 250C and occupying a volume of 2m3 is heated at constant pressure to 2000C. Find heat added, work done and change in internal energy.

2.	A cylinder closed at both contains a free piston, on one side of which is nitrogen and other side air. The initial pressure and volume of each being 1.03 bar and

0.5 m3 respectively. Both the piston and cylinder are perfectly insulated. In the cylinder on the air side of the piston there is an electric heater which is used to heat the air. Heat is added to the air in this manner until the volume occupied by the nitrogen is 0.3 m3. The initial temperature of each gas is 500C. Determine

(a)  the final temperature of air and   (b)  the heat supplied to air. Assume Cp for air as 1.005 and R for air as 0.287 kj / kg K  and   = 1.4  for nitrogen. Also draw the  PV diagram.

3.a)	Prove the statement  “ Of all engines which operate between given two thermal reservoirs the reversible engine possesses the maximum thermal efficiency ”

b)	A heat engine is used to drive a heat pump. The heat transfer from the heat engine and from the heat pump are used to heat the water circulating through the radiators of a building. The efficiency of the heat engine is 27% and C.O.P. of the heat pump is 4. Evaluate the ratio of heat transfer to the circulating water to the heat transfer to the heat engine.

4.a)	Enunciate the two classical statements of second law of thermodynamics

b)	1 kg of water is heated at a constant pressure of 0.7MPa. The boiling point is 164.970C and the latent heat of evaporation is 2066.3 kj / kg. If the initial temperature of water is 00C, find the increases in entropy of the water if the final state is dry saturated steam. Assume for liquid water Cp = 4.2 kj / kg K.

(Contd…2)

## Code.No: 210304              		                   .2.				      Set.No.3

5.a)	Derive the expressions for Heat transfer and work done during a reversible hyperbolic process.

b)	1 kg of ideal gas is heated from 18.30C to 93.40C. Assuming R = 287 j / Kg K and

 = 1.18 for the gas, find out:

- Specific heat

- Change in internal energy and

- Change in enthalpy.

6.	A mixture of ideal gases consists of 3 kg of nitrogen and  5 kg of carbon dioxide at a pressure of 300Kpa and a temperature of 200C.

find:

- The mole fraction of each consistuent

- The equivalent molecular weight of the mixture

- The equivalent gas constant of the mixture

- The volume of  the mixture.

7.a)	Define:

- Wet bulb temperature

- Dry bulb temperature

b)	Air at 250C  and 1 bar has a relative humidity of 50%. Using psychometric chart, determine.

- The partial pressure of steam

- The dew point

- The humidity ratio

- The wet bulb temperature.

8.a)	Draw  p-v diagram and T-s diagram of an Otto cycle and derive the cycle efficiency and mean effective pressure?

b)	Calculate the thermal efficiency of an Otto cycle working with maximum temperature of 14000C and minimum temperature of  420C. If the compression ratio is 10 and pressure of air the beginning of the compression is 1.5 bar, then estimate mean effective pressure.

Code No: 210304

# Set No.

4

II B.Tech. I Semester Examination November/December, 2002

THERMO DYNAMICS

(Common to Mechanical Engineering and Aeronautical Engineering)

Time: 3 hours.								         Max. Marks: 80

Answer any FIVE questions.

All questions carry equal marks.

- - -

1.a) 	Define first law of thermodynamics and explain the perpetual motion machine of first kind.

b)	Determine the change in internal energy of gas when it is compressed at constant pressure from 0.4m3 and 105kpa to final state of 0.2m3, during the process 42.5KJ of heat is transferred.

2.	A system containing 0.2 m3 of air at a pressure of 4 bar and 1600C expands isentropically to a pressure of 1.06bar and after this the gas is heated at constant pressure till the enthalpy increases by 65 kj. Calculate the work done. Now imagine that these process are replaced by a single reversible polytrophic process producing the same amount work between the initial and final state, find the index of expansion in this case. Take  Cp of air as 1.005 kj / kg K.

3.a)	What is absolute temperature scale? Develop this scale from Carnot theorem

b)	A reversible engine during a cycle of operation interacts with three thermal reservoirs maintained at 200K, 300K and 400K. It receives 15MJ of heat from the reservoir at 400K and produces a net positive work of 840kJ. Find the amount and direction of heat interaction with other reservoirs.

4.a)	Prove the clausius inequality and hence prove that property entropy exist.

b)	The volume of one kg air increases from 0.5m3 to 1.3m3 while its pressure decreases from 1MPa to 0.25 MPa during a process. Then 100kJ of heat is added to it in a constant pressure process. Calculate the enteopy change for the whole events. Assume for air  Cp = 1.005 kJ / kg.K and R = 0.87kJ / kg.K

5.a)	What is an adiabatic process? Derived and expression for work done during the adiabatic process of ideal gas.

b)	1 kg of gas at 2400C, expands adiabatically so that its volume is doubled and the temperature falls to 1150C. The work done during the expansion is 89.86 KJ. Calculate the two specific heats.

(Contd…2)

## Code.No: 210304              		                   .2.				      Set.No.4

6.	A mixture of gases contains 1 kg of oxygen and 1.5 kg of Nitrogen. The pressure and temperature of mixture are 3.5 bar and 270C. Determine for the mixture.

- The mass and mole fraction of each constituent gas.

- The average molecular weight

- The specific gas constant

- The density.

7.a)	Define:

- Vapour pressure

- Degree of saturation

b)	A room of dimensions  5m x 3m x 3m contains an air water vapour mixture at 1bar, 300C and 70% relative humidity. Calculate.

- Mass of air

- Mass of water Vapour

- Also find the degree of saturation.

The universal gas constant is 8.3143 KJ / kg – Mole K and molecular mass of air and water vapour is 29 and 18 respectively.

8.a)	Show that the efficiency of an Otto cycle depends only on compression ratio? And also derive the equation for mean effective pressure?

b)	Calculate the air standard efficiency of an Otto cycle with compression ratio and compression begins at 350C and 0.1 Mpa. The max temperature of the cycle is 11000C.