Code No: 211401

# Set No.

1

II B.Tech I- Semester Examination November/December 2002.

## THERMODYNAMICS AND FLUID MECHANICS

(common to Production Engineering and Mechatronics)

Time: 3 hours								          Max. Marks: 80

Answer any Five questions

All questions carry equal marks

----

1.a)	Explain briefly Zeroth law of thermodynamics.

- 1 kg of a fluid is compressed neversibly according to a law pr = 0.25 where p is in bars and υ is in m3/kg.  The final volume is ¼ of the initial volume.  Calculate the work done on the fluid and sketch the process on a p-v diagram.

2.a)	Explain the First Law of Thermodynamics as referred to closed systems undergoing a cyclic change.

- Steam enters a turbine at 20 m/sec and specific enthalpy of 3000 kJ/kg and leaves the turbine at 40 m/sec and specific enthalpy of 2500 kJ/kg.  Heat lost to the surroundings is 25 kJ/kg of steam as the steam passes through the turbine.  If the steam flow rte is 360,000 kg/hour, determine the output from the turbine in MW.

3.a)	Give the following statements of second law of thermodynamics.  (i) Clausins statement.  (ii) Kelvin – Planck statement.

- Air at 150C and 1.05 bar occupies 0.02 m3.  The air is heated at constant volume until the pressure is 4.2 bar, and then cooled at constant pressure back to the original temperature.  Calculate the net heat flow to a from the air and the net entropy change.  Sketch the process on a T-S diagram.

4.a)	Define the term ‘availability’.

- 1000 kJ of heat leaves hot gases at 14000C from a fire box of a boiler and goes to steam at 2500C.  Atmospheric temperature is 200C.  Divide the energy into available and unavailable portions  (i) as it leaves hot gases  (ii) as it enters the steam.

5.	Derive the Maxwell relation and explain their importance in thermodynamics.

6.a)	Derive an expression for the efficiency of a Dual Combustion cycle.

- The compression ratio of a dual cycle is 10.  The temperature and pressure at the beginning of the cycle are 1 bar and 270C.  The maximum pressure of the cycle is limited to 70 bar and heat supplied is limited to 675 kJ/kg of air.  Find the thermal efficiency of the cycle.

Contd….2

Code No: 211401				   -2-				       Set No.1

7.a)	Define the following: (i) Steady flow  (ii) Mpm-uniform flow  (iii) Lamniar flow  (iv) Two-dimensional flow.

- A 40 cm diameter pipe, conveying water, branches into two pipes of diameters 30 cm and 20 cm respectively.  If the average velocity in the 40 cm diameter pipe is 3 m/sec.  Find the discharge in this pipe.  Also determine the velocity in 20 cm pipe if the average velocity in 30 cm diameter pipe is 2 m/sec.

8.a)	What is a Venturimeter?  Derive an expression for the discharge through a Venturimeter.

- A pipe line carrying oil of specific gravity 0.8, changes in diameter from 300 mm at a position A to 500 mm diameter to a position B which is at 5 m at a higher water level.  If the pressures at A and B are 19.62 N/cm2 and 14.91 N/cm2 respectively and the discharge is 150 litres/sec, determine the loss of head and direction of flow.

- - -

Code No: 211401

# Set No.

2

II B.Tech I- Semester Examination November/December 2002.

## THERMODYNAMICS AND FLUID MECHANICS

(common to Production Engineering and Mechatronics)

Time: 3 hours								          Max. Marks: 80

Answer any Five questions

All questions carry equal marks

----

1.a)	Explain the following terms:  (i) State (ii) Process and (iii) Cycle.

- A turbine is supplied with steam at a gauge pressure of 1.4 Mpa.  After expansion in the turbine the steam flows into a condenser which is maintained at a vacuum of 710 mm Hg.  The barometric pressure  is 772 mm Hg.  Express the inlet and exhaust pressures in pascals.  Take the density of mercury as 13,600 kg/m3.

2.a)	Why in constant pressure non-flow process, the enthalpy change is equal to heat transfer?

- A stream of gases at 7.5 bar, 8000C and 150 m/sec is passed through a turbine of a Jet engine.  The steam comes out of the turbine at 2.0 bar, 6000C and 300 m/sec.  The process may be assumed adiabatic.  The enthalpy of gas at the entry and exit of the turbine are 960 kJ/kg and 700 kJ/kg of gas respectively.  Determine the capacity of the turbine if the gas flow is 4 kg/sec.

3.a)	What do you mean by ‘Thermodynamic temperature’?

- 0.03 m3 of nitrogen contained in a cylinder behind a piston is initially at 1.05 bar and 150C.  The gas is compressed isothermally and reversibly until the pressure is 4.2 bar.  Calculate the change of entropy, the heat flow and the work done and sketch the process on a p-v and T-S diagram.  Assume nitrogen to act as a perfect gas.  Molecular weight of nitrogen=28.

4.a)	Derive an expression for availability in steady flow systems.

- In a heat exchanger of parallel flow type, water enters at 600C and leaves at 800C while oil of specific gravity 0.8 enters at 2500C and leaves at 1000C.  The specific heat of oil is 2.5 kJ/kg-K and surrounding temperature is 300K.  Determine the loss in availability on the basis of one kg of oil flow per second.

5.a)	Derive Gibb’s relations.

- Why Gibb’s function does not change during change of phases of first order?

Contd…2

Code No: 211401				-2-				       Set No.2

6.a)	Derive an expression for air standard efficiency of an engine working on Otto cycle.

- An oil engine works on ideal diesel cycle.  The pressure and temperature at the beginning of the cycle are 1 bar and 3800K.  The maximum pressure of the cycle is limited to 50 bar.  The heat is supplied to 6% of the stroke.  Find the following:  (i) the percentage clearance  (ii) Air standard efficiency of the cycle  (iii) Mean effective pressure.

7.	Derive from first principles, the condition for irrotational flow.  Prove that for potential flow, both the stream function and velocity potential function satisfy the Laplace equation.

8.a)	What is a pitot tube?  How will you determine the velocity at any point with the help of pitot tube?

- The water is flowing through a pipe having diameters 20 cm and 15 cms at sections 1 and 2 respectively.  The rate of flow through the pipe is 40 litres/sec.  The section 1 is 6 m above datum line and section 2 is 3 m above the datum.  If the pressure at section 1 is 29.43 N/cm2, find the intensity of pressure at section 2.

- - -

Code No: 211401

# Set No.

3

II B.Tech I- Semester Examination November/December 2002.

## THERMODYNAMICS AND FLUID MECHANICS

(common to Production Engineering and Mechatronics)

Time: 3 hours								          Max. Marks: 80

Answer any Five questions

All questions carry equal marks

----

1.a)	What is a quasi-static process?  How does a homogeneous system differ from a heterogeneous system.

- A gas having a volume of 0.05 m3 and pressure of 6.9 bar expands reversibly in a cylinder behind a piston according to the law pv1.2 = constant until the volume is 0.08 m3.  Calculate the work done by the gas.

2.a)	Define ‘internal energy’ and prove that it is a property of a system.

- 12 kg of a fluid per minute goes through a reversible steady flow process.  The properties of fluid at the inlet are p1 = 1.4 bar, 1=25 kg/m3, C1=120 m/sec, and   u1 = 920 kJ/kg and at the exit are p2 = 5.6 bar, 2= 5 kg/m3, C2 = 180 m/sec, and  u2 = 720 kJ/kg . During the passage, the fluid rejects 60 kJ/sec and rises through 60 meters.  Determine (i) the change in enthalpy and (ii) work done during the process.

3.a)	Prove that entropy is a property of the system.

- 1 kg of air is allowed to expand reversibly in a cylinder behind a piston in such a way that the temperature remains constant at 2600C while the volume is doubled.  The piston is then moved in, and heat is rejected by the air reversibly at constant pressure until the volume is the same as it was initially.  Calculate the net heat flow and over all change of entropy.  Sketch the process on a T-S diagram.

4.a)	Explain the concept of available and unavailable energy.

- A system receives 10,000 kJ of heat at 500 K from a source at 1000 K.  The temperature of the surrounding is 300 K.  Assuming that the temperature of the system and source remains constant during heat transfer, find: (i) the entropy production due to heat transfer (ii) decrease in available energy.

5.	For the following differential equations,

du = Tds – pdr   and du = Tds + vdp ; Prove that for perfect gas equation

u                             u

---    = 0 and             ---     = 0.	
            	p                             p

T

Contd….2

Code No: 211401				-2-				Set No.3

6.a)	Derive an expression for the efficiency of a Diesel cycle.

- An air standard Diesel cycle has a compression ratio of 18, and heat transferred to the working fluid per cycle is 1800 kJ/kg.  At the beginning of compression stroke, the pressure is 1 bar, and temperature is 300 K.  Calculate (i) Thermal efficiency  (ii) Mean effective pressure.

7.a)	Define Velocity potential function and stream function.

- The diameters of a pipe at the sections 1 and 2 are 15 cm and 20 cm respectively.  Find the discharge through the pipe if the velocity of water at section 1 is 4 m/sec.  Determine also the velocity at section 2.

8.a)	State the momentum equation.  How will you apply momentum equation for determining the force exerted by a flowing liquid on a pipe band?

- A pipe of diameter 30 cm carries water at a velocity of 20 m/sec,  The pressure at point A and B are given as 34.335 N/cm2 and 29.43 N/cm2 respectively.  While the datum head at A and B are 25 m and 28 m, find the loss of head between A and B.

- - -

Code No: 211401

# Set No.

4

II B.Tech I- Semester Examination November/December 2002.

## THERMODYNAMICS AND FLUID MECHANICS

(common to Production Engineering and Mechatronics)

Time: 3 hours								          Max. Marks: 80

Answer any Five questions

All questions carry equal marks

----

1.a)	Define a thermodynamic system.  Differentiate between open system, closed system and an isolated system.

- A cylindrical vessel of 60 cm diameter and 80 cm height contains 3.2 kg of a gas.  The pressure measured with manometer indicates 60 cm of Hg above atmosphere when barometer reads 760 mm of Hg.  Find (i) the absolute pressure of the gas in the vessel in bars and (ii) specific volume and density of the gas.

2.a)	For isothermal flow and non-flow steady processes, prove that

2	              2

∫ pdv = - ∫ v.dp.

1            1

Also state the assumptions made.

- A turbine operating under steady flow conditions receives steam at the following state: Pressure = 13.8 bar, specific volume = 0.143 m3kg, Internal energy = 2590 kJ/kg, velocity = 30 m/sec.  The state of steam leaving the turbine is: Pressure = 0.35 bar, specific volume = 4.37 m3/kg, Internal energy = 2360 kJ/kg, velocity = 90 m/sec.  Heat is lost to the surroundings at a rate of 0.25 kJ/sec.  If the rate of steam flow is 0.38 kg/sec, what is the power developed by the turbine?

3.a)	What do you mean by ‘clausins inequality’?

- A fluid undergoes a reversible adiabatic compression from 0.5 Mpa, 0.2 m3 to 0.05 m3 according to the low pr1.3 = constant.  Determine the change in enthalpy, internal energy, entropy and heat transfer, and work transfer during the process.

4.a)	Derive an expression for availability in non-flow systems.

- 500 kJ of heat from an infinite source at 1000 K is supplied to 2 kg of gas initially at 2 bar and 350 K in a closed tank.  Find the loss in available energy due to above heat transfer.  Take cr (gas) = 0.8 kJ/kg and surrounding temperature as 300 K.

5.	Explain Maxwell and Gibb’s relations and derive the formulas.

Contd….2

Code No: 211401				-2-				Set No.4

6.a)	Derive an expression for the efficiency of Carnot cycle.

- An engine working an OHO cycle has a volume of 0.5 m3, pressure 1 bar and temperature 270C at the commencement of compression stroke.  At the end of compression stroke the pressure is 10 bar.  Heat added during the constant volume process is 200 kJ.  Determine (i) Percentage clearance  (ii) Air standard efficiency  (iii) Mean effective pressure (iv) Ideal power developed by engine if the engine runs at 400 rpm so that there are 200 complete cycles per minute.

7.	State Bernoulli’s theorem for steady flow of an incompressible fluid.  Derive an expression for Bernoulli’s equation from first principle and state the assumptions made for such a derivation.

8.a)	Define the following terms: (i) Velocity potential (ii) stream function.

- The following cases represent the two velocity components, determine the third component of velocity such that they satisfy the continuity equation.

  - u = 4x2 , v= 4xyz

  - u = 4x2 + 3xy , w = z3 – 4xy – 2yz.

- - -