Code No.210153 OR

II-B.Tech. I-Semester Examination Nov/Dec 2002

# THERMO DYNAMICS

(Civil Engineering)

Time:3 hours								Max.Marks:70

Answer any FIVE questions

All questions carry equal Marks

---

1.a)	Explain the following terms:

i) Control volume	(ii) System	(iii) Property

- Limitations of first law of thermodynamics.

- What is isothermal process? Derive an expression for the work done and heat transfer during isothermal expansion process of an ideal gas.

2.a)	Define entropy and prove entropy is a point function or a property of a system.

- Derive an expression for change in entropy of isothermal process of an ideal gas.

3.a)	Explain the following terms related to properties of steam:

i)   Wet steam	(ii) Enthalpy of wet steam	(iii) Entropy of wet steam

iv) Specific volume of wet steam.

- Prove that with the help of  T-S-diagram that for the same compression ratio and same heat added, Otto cycle is more efficiency than diesel cycle.

4.a)	Explain the working of a 4-stroke diesel engine with the help of sketches.

- Draw value timing diagram for 4-stroke diesel engine. Explain silent points.

5.a)	Explain the principle of working of  Parson-reaction turbine.

b)	Draw velocity diagrams for impulse turbine and write equations for Tangential force, and power developed by the turbine.

6.a)	State merits and demerits of closed cycle gas turbine over open cycle gas turbine.

b)	List the advantages and disadvantages of gas turbine over steam turbine over steam turbine and diesel or petrol engines.

7.a)	Distinguish between reciprocating air compressor and rotary compressor.

- With the help of neat diagram explain how rotary compressor produces   compressed air.

(Contd…2)

Code No.210153			-2-					OR

8.a)	Explain the following:

i)   Ton of refrigeration	(ii) Cop	(iii) Application of refrigeration and

iv) Methods of producing cold.

b)	It is required to store food using air refrigeration system. It needs 40 Tons of refrigeration. The temperature of air entering the compressor is 7oC and after compression it is cooled to 27oC     (i) Calculate Cop of the cycle

ii) Power required by the compressor .

Assume that mass flow rate of air flow is 80 kg/min.

The index of compression and expansion = 1.3

Cp = 1.005 kJ/kg.k for air and  r = 1.4.

***	***	***