Code No.210102 Set No. 1

II  B.Tech. I-Semester Examination Nov/Dec 2002
STRENGTH OF MATERIALS

(Civil Engineering)

Time:3 hours								Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.	A solid steel bar of diameter 50mm and length 320mm is placed inside an aluminium cylinder of inside diameter 60mm and outside diameter 100mm. The length of the aluminium cylinder is 0.5mm more than the steel bar. An axial load of 1200KN is applied on the assembly through rigid cover plates on both sides. Estimate the stresses developed in the steel bar and the aluminium cylinder. Take Es = 2 x 105N/mm2 and EAℓ = 0.7 x 105 N/mm2.

2.	Draw the shear force and bending moment diagrams for the beam shown in figure giving all the salient values.

3.a)	Explain what do you understand by pure bending. List out all the assumptions in the theory of simple bending.

b)	Determine the safe load carrying capacity uniformly distributed load over a simply supported span of 8m. The cross section of the beam is rectangular of size 300mm x 500mm with a circular hole of diameter 200mm with its center at 200mm from the soffit. The allowable stresses in tension and compression are 150 N/mm2 and 100 N/mm2 respectively.

4.a)	Derive the relation C = FAY  for the shear stress at any fibre in the cross section of

Ib

a beam carrying a shear force of ‘F' at the section.

b)	Draw the variation in shear stress across the depth of a T-section with a flange of 500mm x 20mm and web of 480mm x 20mm (with overall depth of 500mm). It caries a shear force of 150KN.

(contd…2)

Code No.210102			-2-			Set No.1

5.	Find the forces in the members BC, BE and FE of the truss shown in figure.

6.	Find the slope and the deflection at important locations of a simple beam of length ‘ℓ’ carrying a udℓ of intensity w/unit length over its entire span. Use moment-area method only. Take EI as constant.

7.	Design a double riveted double cover butt joint for connecting two plates of 15mm thickness. Take S = 100 N/mm2, fb = 200 N/mm2 and ft = 150 N/mm2.

8.	A seamless spherical sheel, 0.9m in diameter and 10mm thick, is being filed with a fluid under pressure until the volume increases by 150 cm3, calculate the pressure exerted by the fluid on the shell, taking modulus of elasticity for the material of the sheel as 2 x 105 N/mm2 and Poisson’s ratio as 0.3.

***	***	***

Code No.210102 Set No. 2

II  B.Tech. I-Semester Examination Nov/Dec 2002
STRENGTH OF MATERIALS

(Civil Engineering)

Time:3 hours								Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.	Find the axial deformation of the rod shown in figure below. Take E = 2.1 x 105 N/mm2.

2.	Sketch the shear force and bending moment diagrams for the beam shown in figure giving all salient values.

3.a)	A beam of square cross section a x a is placed with one diagonal vertical. Find out what portion of the cross section is to be cut for minimum bending stress in the section for a given moment ‘M’.

b)	A floor has to carry a load of 8KN/m2. If the span of each joist is 4.5m and each joist is of 225mm x 75mm, calculate the maximum possible spacing of the joists if the bending stress is not to exceed 75 N/mm2.

(contd…2)

# Code No.210102			-2-			Set No.2

4.a)	A beam of square cross section is used to support a shear force of ‘F’ at a section with one of its diagonals horizontal. Find the max. shear stress in the cross section of the beam. Also sketch the shear stress distribution across the depth of the section.

b)	The cross section of a beam is an equilateral triangle of side 150mm. It carries a shear force of 200KN. If it is placed with one side horizontal, plot the distribution of shear stress across the depth.

5.	Find the forces in every member of the truss shown in figure.

6.	 A simple beam of length 7m is supported at A and B. It is carrying two concentrated loads 5KN and 15KN at a distance of 1m and 2m respectively from the left end of the beam. Using Macaulay’s method, find

- The slope at the support points

- The position and magnitude of maximum deflection

Take E = 90 Gpa   and   I = 18 x 10-6m4

7.	Two plates of 20mm thickness are to be connected by a single riveted lap joint with rivets of 18mm nominal diameter. Find the necessary pitch of the rivets. Take S = 100 N/mm2, fb = 300 N/mm2 and fb = 120 N/mm2

8.	A cylindrical shell of 180mm diameter and 1m length is filled with a fluid at atmospheric pressure. The wall thickness is 8mm. If an additional 20CC (20,000mm3) of the fluid is pumped into the cylinder, find the pressure exerted by the fluid on the wall of the cylinder. Find also the hoop stress induced. Assume E as 2.1 x 105 N/mm2 and Poisson’s ratio as 0.25.

***	***	***

Code No.210102 Set No. 3

II  B.Tech. I-Semester Examination Nov/Dec 2002
STRENGTH OF MATERIALS

(Civil Engineering)

Time:3 hours								Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.	A short steel tube of external diameter 75mm and internal diameter 50mm is surrounded by a brass tube of same length and having external diameter 100mm and internal 75mm. The tubes are rigidly fixed and an axial load of 20KN is placed on the tubes. Find the load carried by each and also shortening of each tube, if it is 25cm long.

Given Esteel = 2 x 105 N/mm2 and Eblass = 1 x 105 N/mm2.

2.	Draw the shear force and bending moment diagrams for the beam shown in figure below:

3.a)	A 2m long bar of uniform section 50mm2 extends 2mm under a limiting axial stress of 200 N/mm2. What is the resilience of the bar.

b)	What is the instantaneous elongation of a steel rod 50mm in diameter, 4m long, subjected to an axial suddenly applied load of 60KN.

4.a)	A cantilever of span 2m carries a udl of  1 KN/m over its entire length. If its hollow rectangular section has outer dimensions of 10cm x 20cm and inner dimensions of 6cm x 16cm, sketch the shear stress distribution at a location 150cm from the free end.

b)	The cross section of a beam is an equilateral triangle of side 150mm. It carries a shear force of 200KN. If it is placed with one side horizontal, plot the distribution of shear stress across the depth.

(contd…2)

# Code No.210102			-2-			Set No.3

5.	Find the forces in members CD, FE, AC and FB of the truss shown in figure.

6.	Calculate the slope and deflection at the points A,B and E for the beam shown in figure. Use Moment-Area method and take EI = 105 KN-m2.

7.	Design a double cover butt joint to connect two plates of 20mm thickness and 300mm width. Take S = 100 N/mm2, fb = 300 N/mm2 and ft = 100 N/mm2.

8.	A thin cylindrical shell of inside diameter 1.5m is made of 10mm thick steel plate. It is of 4m length and is closed at its both ends. The shell is subjected to an internal fluid pressure of 2N/mm2. Find the change in length, the final length, the change in diameter, the final diameter, the change in volume and the final volume. Also calculate the maximum shear stress induced in the cylinder. Take the modulus of elasticity of steel as 2.1 x 105 N/mm2 and the Poisson’s ratio of 0.30.

***	***	***

Code No.210102 Set No. 4

II  B.Tech. I-Semester Examination Nov/Dec 2002
STRENGTH OF MATERIALS

(Civil Engineering)

Time:3 hours								Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

- A steel tube, 3.2cm internal diameter, 0.25cm thick and 4m long, is covered and lined throughout with copper tubes 0.2cm thick. The three tubes are firmly united at the ends. This compound tube is subjected to tension and the stress produced in steel is 60 N/mm2.

Determine :  (a)  The stress in the copper tubes,

b)  The load carried by the composite if Esteel = 2 x 105N/mm2 and

Ecopper = 1 x 105N/mm2.

2.a)	Derive the relationship between intensity of loading, shear force and bending moment for a cantilever beam.

b)	Draw the shear force and bending moment diagrams for the beam shown in figure and indicate all the salient values.

3.a)	Derive the flexure formula, mentioning the assumptions made in the formulation of the theory.

b)	Show that a circular, square and rectangular cross sections have flexural efficiency in the increasing order, if they have same cross sectional area.

4.	A beam having a cross section of an isoceles triangle has a base of 150mm and height of 450mm and subjected to a max.shear force of 30KN at a section. Find the shear stress at neutral axis. Also find the magnitude of max shear stress and where it acts.

(contd…2)

## Code No.210102			-2-			Set No 4

5.	A warren truss shown in figure below consists of three equilateral triangles each of side 2m. Determine the force in each member of the truss.

6.	Find the expressions for slope and deflection for a simple beam carrying uniformly distributed load of intensity w/unit length over its entire span.

7.	Two plates 100mm x 6mm size are joined by a triple riveted lap joint such that each row consists of 3 rivets of 15mm nominal diameter and 30mm pitch. If the allowable stresses in the rivets in shearing and bearing are 100 N/mm2 and 300 N/mm2 respectively and the allowable tensile stress of plate is 120 N/mm2, find the rivet value, strength of the joint and its efficiency.

8.	A cylindrical shell is 3.2m long, 1.6cm internal diameter and 20mm metal thickness. Find the intensity of maximum shear stress induced if the internal pressure is 2.5 N/mm2. E = 2.1 x 105 N/mm2, 1 = 0.33. Find also the change in	                       						 m

volume.

***	***	***