Code No:210203

Set No.

1

# II B.Tech. I Semester Examination November/December 2002

SWITCHING THEORY AND LOGIC DESIGN

(Common to Electrical & Electronics Engineering, Electronics & Communication Engineering, Computer Science & Engineering, Computer Science & Information Technology, Electronics & Control Engineering, Electronics & Instrumentation Engineering, Electronics & Telematics, Electronics & Computer Engineering, Computer Science & Systems Engineering.)

Time: 3 Hours								          Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1.  a)	Obtain the simplified expressions in sum of products for the following Boolean functions using Karnaugh-Map.

- F (A, B, C, D)	=  (7, 13, 14, 15)

- F (w, x, y, z)	=  (2, 3, 12, 13, 14, 15)

b)	Minimize the following Boolean expressions to the required no. of literals

- BC + AC1 + AB + BCD to four literals

- ABC + A1B1C + A1BC + ABC1 + A1B1C1 to five literals

c)	Make the indicated conversions for the following numbers

(i)	(432)5	= (		)7

(ii)	1431)8	= (		)10

2.	Consider the following four codes.

Code A		Code B		Code C		Code D

0001			000			01011			000000

0010			001			01100			001111

0100			011			10010			110011

1000			010			10101									110

111

101

100

a)	Which of the following properties is satisfied by each of the above codes?

  - Detects single errors

  - Detects double errors

  - Detects triple errors

  - Corrects single errors

  - Corrects double errors

  - Corrects single and detects double errors.

b)	How many words can be added to code A with out changing its error-detection and correction capabilities? Give a possible set of such words. Is this set unique?

Contd…2

## Code No: 210203				:: 2 ::				Set No. 1

3.	Derive the implicant chart for the given expression

f (v, w, x, y, z) = m(0,4,12,16,19,24,27,28,29,31)

Is the prime-implicantt chart cyclic?  Apply the suitable method to find the minimal expression.

4.  a)	Minimise the given multiple output function x, y, z are inputs and F,G are outputs.

X		Y		Z		F		G

0		0		0		0		1

0		0		1		0		1

0		1		0		0		0

0		1		1		1		1

1		0		0		0		0

1		0		1		0		0

1		1		0		1		0

1		1		1		1		0

b)	Give a two-level NAND-NAND circuit for the given expression F=(w.x.y) + (y.z)

c)	Obtain the two-level ORR-AND circuit for the given expression

F=(x+y’).z) + (x’.y.z’)

5.	A sequential circuit has three D flip-flops, A, B, C and one input x.  It is described by the following flip-flops input functions

DA=(BC1+B1+C) X + (BC+B1C1)X1  ,   	DB = A  ,	DC = B

a)	Derive the state stable for the circuit.

b)	Draw two state diagrams one for x=0, and the other for x=1.

6.	Design a sequential circuit with two D flip flops A and B and one input x.  When x=0, the state of the circuit remains the same.  When x=1, the circuit goes through the state transitions from 00 to 01 to 11 to 10 back to 00 and repeats.

7.  a)	Explain in detail the mealey state diagram and ASM chart for it with an example.

b)	Show that 8 exit paths in an ASM block emanating from the decision boxes the check the eight possible binary values of three control variables x, y, z.

## 8.  a)	Distinguish between melay and moore machines.

b)	Find the equivalence partition for the given machine and a standard form of the corresponding reduced machine.

| PS | NS, Z |  |
| --- | --- | --- |
|  | x = 0 | x = 1 |
| A B C D E | B10 E10 D11 C11 B10 | E10 D10 A10 E10 D11 |

@@@@@

Code No:210203

Set No.

2

# II B.Tech. I Semester Examination November/December 2002

SWITCHING THEORY AND LOGIC DESIGN

(Common to Electrical & Electronics Engineering, Electronics & Communication Engineering, Computer Science & Engineering, Computer Science & Information Technology, Electronics & Control Engineering, Electronics & Instrumentation Engineering, Electronics & Telematics, Electronics & Computer Engineering, Computer Science & Systems Engineering.)

Time: 3 Hours								          Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1.  a)	Use a Karnauughs map to minimize.

_  _  _	 _  _  _  _           _         _                           _    _      _     _

F = A B C D + A B C D + AB C D + A BCD + ABD + B CD + A B C D

b)	(i)	Convert (A98B)12 to (			)3

- Determine gray code for (97)10

- Perform the substraction on the following decimal numbers by using 9’s and 10’s complements.

(1)	6521-433		(2)	3674-598

2.  a)	Derive the following properties of Boolean algebra directly form the Huntington postulates.

- For each a in B, these exists a unique a1 in B

- For every a in B, (a1)1 = a

- For every Boolean algebra, 0’=1 and 1’=0

- In any Boolean algebra

(a+b)1 = a1.b1 and (a.b)1 = a1+b1

b)	(i)	Find 10’s complement of (935)11

(ii)	Convert (347)8 to (	)16

c)	(i)	Expand the given expression into canonical SOP form, then use only the

logical adjacency theorem to simplify.

_  _        _  _                     _

F (A,B,C) = A C + A B C + ABC + A B C

- Simplify the function using Karnaugh’s map method.

F (A,B,C,D) =  (1,4,6,8,11,13,14)

3.  a)	Show that more than one 2-to-4 decoder plus a gate switching matrix can be connected to form a 4-to-16 decoder.

- Show with an example how ROM can be used to implement a combinational function.

## Contd……2

## Code No: 210203				:: 2 ::				Set No. 2

4.  a)	Implement the following function using a multiplexer of proper size.

F (w,x,y,z) =  m(0,1,2,3,4,9,13,14,15)

- Explain a 16-line to 16-line mux/demux system using ICs like 74150(16-to-1 mux with active-low enable input) and 74154 (1-to-16 demux).

- Write a note on ‘high-speed adders’.

5.  	Design the sequential circuit specified by he state diagram using T flip flops.

0

00

00

00

00

1

1

1

1		    0

0

0

6.	Design a sequential circuit with two JK flip flops A and B and two inputs E and x.  If E=0, the circuit remains the same state regardless of the value of x.  When E=1 and x=1, the circuit goes through the state transition from 00 to 01 to 10 to 11 back to 00 and repeats.  When E=1 and x-0, the circuit goes through the state transitions from 00 to 11 to 10 to 01 back to 00 and repeats.

7.	Draw the state diagram of serial adder and then convert it to ASM chart.

Design the control circuit o the serial adder sing

- PLA method & register

- D flip flop & a decoder

## 8.	Find the equivalence partition and a corresponding reduced machine in standard form.

| PS | NS, Z |  |
| --- | --- | --- |
|  | x = 0 | x = 1 |
| A B C D E F G H | B,1 F,1 D,0 C,0 D,1 C,1 C,1 C,0 | H,1 D,1 E,1 F,1 C,1 C,1 D,1 A,1 |

## Code No:210203

Set No.

3

# II B.Tech. I Semester Examination November/December 2002

SWITCHING THEORY AND LOGIC DESIGN

(Common to Electrical & Electronics Engineering, Electronics & Communication Engineering, Computer Science & Engineering, Computer Science & Information Technology, Electronics & Control Engineering, Electronics & Instrumentation Engineering, Electronics & Telematics, Electronics & Computer Engineering, Computer Science & Systems Engineering.)

Time: 3 Hours								          Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1.  a)	Demonstrate by means of  truth-tables the validity of the following theorems of Boolean algebra.

- Associative Laws

- DeMorgan’s Theorems for three variables

- The distributive law of + over.

b)	Convert the number (0.875) 10 into its.

(i) Binary		(ii) Trinary		(iii) Quinary

- Octal equivalents using the multiplication method.

c)	(i)	Convert 3-bit gray code to 3-bit binary code

- Using Karnaughs-map method express the function in standard SOP form

f  = AB + A C + C + AD + A B C + ABC

2.  a)	(i)	Show that (B+D)(A+D)(B+C)(A+C) = AB+CD

- Obtained the truth table for the POS expression

F=(A+B+C) (  A + B +  C )

- Convert  421.6095 to binary, octal and hexadecimal

__                       __

b)	(i)	Simplify the logic function f  = AB + A C + C + AD + A B C + ABC

- Simplify Boolean function F = A’C + A’B + AB’C + BC

3  a)	Show how following functions can be realized by an AND-OR array.

F1 (a,b,c) = ab+c’

F2 (a,b,c) = ab+b’c

b)	Implement the following function using ROM

f(a,b,c,d) =  m(0,1,6,8,9)

4.	Use a 32 x 6 ROM to convert a 6-bit binary number to its corresponding 2-digit BCD representation.

(a5 a4 a3 a2 a1 ao)2 = ((x3, x2, x1, x0) BCD (y3, y2,y1,y0) BCD)10

Show the ROM contents in a truthtable format

## Contd……2

## Code No: 210203				:: 2 ::				      Set No. 3

5.	Design the sequential circuit specified by he state diagram using T flip flops.

0

00

00

00

00

1

1

1

1		    0

0

0

6.	Design a shift register which has one  input, one output, one shift pulse input and a control input.  If the control input is a 1, shift left once, else shift right once for each shift pulse

7.	Draw the ASM chart for binary divider and realize it with JK flip flop & gates.  Draw the timing diagram for the same.

8.	Find the equivalence partition for the given machine.  Show a standard form of the corresponding reduced machine.

| PS | NS, Z |  |
| --- | --- | --- |
|  | x = 0 | x = 1 |
| A B C D E F G | F10 G10 B10 C10 D10 E11 E11 | B11 A11 C11 B11 A11 F11 G11 |

@@@@@

## Code No:210203

Set No.

4

# II B.Tech. I Semester Examination November/December 2002

SWITCHING THEORY AND LOGIC DESIGN

(Common to Electrical & Electronics Engineering, Electronics & Communication Engineering, Computer Science & Engineering, Computer Science & Information Technology, Electronics & Control Engineering, Electronics & Instrumentation Engineering, Electronics & Telematics, Electronics & Computer Engineering, Computer Science & Systems Engineering.)

Time: 3 Hours								          Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1.  a)	(i)   Distinguish between the weighted and non-weighted codes.

(ii)  Using 2’s complement representation method find the result of 01100-00011

b)	(i)   Find the complement of the Boolean function (BC’+A’D)(AB’+CD’) and

reduce it to a minimum number of literals.

(ii)  Convert the function f(x,y,z)= (0,3,6,7) to the other canonical form.

c)	(i) Simplify the Boolean function F in sum of products using the don’t care conditions d: (Karnaugh map method)

F = y’ +x’z’

2.  a)	(i)	Simplify the expression

T (A,B,C,D) = A1B + ABD + AB1CD1+BC

- List the Properties of BCD Code

- Perform 1101.1101 + 1011.10 = (	)2

- Explain about Error Codes.

b)	(i)	Explain sum of products and product of sums

(ii)	Find the Complement of the function Y=(A+BC) (B+CA) in its POS form

c)	Minimise the function using Karnaugh’s map method

F (A,B,C,D) =m (1,2,3,8,9,10,11,14) + d(7,15)

3.  a)	F1 (x,y) = m(0,3)

F2 (x,y) = m(1,2,3)

Implement the combinational circuit by means of the 2-to-4 line decoder with enable input and external nand gates.

b)	Draw the logic diagram of a 2-line to 4-line decoder-demux using NOR gates only.

4.	Design a BCD-to-decimal decoder which gives an output of all 0’s when any

invalid input combination occurs.

## Contd……2

## Code No: 210203				:: 2 ::				Set No.4

5.	Design a circuit to check whether a five bit sequence satisfies an even parity.

6.	Construct the state diagram and state table for a logic circuit that gives an output whenever the sum of the 1’s digits in a repetitive five bit serial input sequence is 2

7.	Draw the state graph for full adder and convert it to ASM chart and realize the above using the JK flip flop & gates.

8.	Find the equivalence partition and a corresponding reduced machine in standard form

| PS | NS, Z |  |
| --- | --- | --- |
|  | x = 0 | x = 1 |
| A B C D E F G H | D10 F11 D10 C10 C11 D11 D11 B11 | H11 C11 F11 E11 D11 D11 C11 A11 |

@@@@@